|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | (AUTO)[ShortDocNo] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | (AUTO)[SgText] |  |  |
|  |  |  | English only |  |  |
| Question(s): |  | (AUTO)[Q nos separated by commas (e.g 3/13, 5/16)] |  | (AUTO)[Venue], [dd-dd mmm yyyy] |  |
| RAPPORTEUR GROUP MEETING − (AUTO)[DocTypeText] |  |  |  |  |  |
| Source: |  | (AUTO)[Source - Do not touch;select during upload] |  |  |  |
| Title: |  | TR. Carrier Switching |  |  |  |
| Contact: |  | Mr. Tejpal Singh Organization : Ministry of Communications, Department of Telecom Country :India |  |  | Tel: :+919818885790 E-mail: tejpal.singh70@gov.in |
| Contact: |  | Mr. Vijay Kumar Roy Organization : Ministry of Communications, Department of Telecom Country :India |  |  | Tel: +917011000101 E-mail: vk.roy@gov.in |
| Contact: |  | Mrs. Preetika Singh Organization : Ministry of Communications, Department of Telecom Country :India |  |  | Tel +918375988195 E-mail: preetika.singh87@gov.in |
| Abstract: |  | This contribution proposes update of different sections of new work item i.e. Technical Report [TR. Carrier Switching] as agreed in the last SG2 Meeting (Geneva, 16-20 May 2022) based on contribution SG2-C048. |  |  |  |

Introduction

This contribution has the main objective of providing inputs on different sections of technical report [TR. Carrier Switching]. For that purpose, we have analysed different stakeholders associated with the use of SIMs and e-SIMs in M2M/IoT, relationship between stakeholders, identities required including the options to implement carrier switching

|  |  | International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | Technical Report |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (ddMMMYYYY) |
|  | ESTR-B.CS Technical report on the carrier switching of SIM and e-SIMs for enterprises (x is the closest Series related to this document: F, G, H, I, T, V, etc, ACRONYM is 4 letters reflecting the title or subject matter) |  |  |  |

Summary

Subscriber Identity Module (SIM) and embedded SIM (e-SIM)  are increasingly being used to support mobile connectivity for M2M/IoT devices at enterprise level such as by car manufacturers, energy suppliers, managed security service providers etc. in order to enhance or add more features and improve customer experience.  Unlike consumer controlled SIM management in traditional mobile ICT devices, the SIM/eSIMs are embedded in the M2M/IoT devices with least physical access for replacement. Further, eSIM are part of internal circuitry itself and cannot be changed during the device lifecycle. In order to avoid carrier lock-in scenario in M2M/IoT device ecosystem, there is need to evolve the framework for carrier switching as per choice of user enterprise similar to what is available to mobile service users. Carrier switching is also critical for ensuring required QoS especially in cases of upcoming latency sensitive use cases as offered by 5G services.

The discussion around enterprises, such as car manufacturers or energy suppliers, utilising SIM and e-SIMs for M2M/IoT communication usually focuses upon the use of the identifiers from an assignment aspect.  The on-going life cycle management, such as enterprises changing the communication provider are yet to evolve fully with increasing adoption of M2M/IoT devices. GSMA White Paper on ‘Business Process for Remote SIM Provisioning in M2M’, in Official Document CLP.05, provide specimen points for M2M OEMs and Operators providing eUICC-based services based in M2M scenarios. This document cover the typical business processes required in the delivery of such services, such that all parties can attain a high degree of confidence in the business processes.

This technical report provide guidance to Mobile Network Operators(MNO) and National Regulators to evolve operational and regulatory framework to implement carrier switching that may require further consideration as national NNAI identifiers are deployed in support of M2M/IoT

Keywords

M2M/IoT, enterprise, consumers, carrier switching [if any provided]

Change Log

This document contains Version 1 of the ITU-T Technical Report ESTR-B.CS on "Error! No text of specified style in document." approved at the ITU-T Study Group xx meeting held in Geneva, dd-dd MMMM YYYY.

[If there are earlier versions: This version supersedes Version VV, which was approved at the ITU-T Study Group xx meeting held in Geneva, d MMMM YYYY]

| Editor: | Name Company Country | Tel: Email: |
| --- | --- | --- |

CONTENTS

Page

1. Scope 6 2. References 6 3. Terms and definitions 6 Terms defined elsewhere 6 Terms defined here 6 4. Abbreviations 6 5. Introduction 7 Appendix I 15

List of Tables

Page

Table 1: Example of a table Error! Bookmark not defined.

List of Figures

Page

Figure 1: Example of a figure Error! Bookmark not defined.

Draft Technical Report ITU-T ESTR-B.CS

ITU-T Technical Report Error! No text of specified style in document.

Summary

This technical report provides guidance to Mobile Network Operators (MNO) and National Regulators to evolve operational and regulatory framework to implement carrier switching in enterprise consumer scenario for M2M/IoT deployment. It may also require further consideration by Member States as national NNAI identifiers are deployed in support of M2M/IoT in enterprise user ecosystem for global mobility.

# Scope

# This technical report analyses role of different stakeholders associated with the use of SIMs and e-SIMs in M2M/IoT, relationship between different stakeholders, identities required for implementation of carrier switching to implement it in the enterprise user ecosystem.

# References

# Terms and definitions

## Terms defined elsewhere

This Technical Report uses the following terms defined elsewhere:

## Terms defined here

This Technical Report defines the following terms:

3.1.1	Carrier Switching: The ability of an enterprise to move their SIMs/e-SIMs between MNOs and service providers without the  intervention of the end consumer

3.1.2	Enterprise: the entity providing a service to a consumer that makes use of SIMs/e-SIM

3.1.3 Profile: A Profile comprises of network operator data related to Service Subscription, including the Operator credentials or third party SIM/eSIM based applications. It contains the IMSI, ICCID, Security algorithms belonging to MNO.

# Abbreviations

| eSIM eUICC IC ICCID IMSI MNO NNAI OTA PIN PUK SIM | Embedded Subscriber Identity Module Embedded universal integrated circuit card Integrated Circuit Integrated Circuit Card ID International mobile subscriber identity Mobile Network Operator Naming, Numbering, Addressing and Identification Over The Air Personal Identification Number Personal Unblocking Key Subscriber Identity Module |
| --- | --- |

# Introduction

SIM is an integrated circuit (IC), also called Universal Integrated Circuit Card(UICC), intended to securely store the international mobile subscriber identity (IMSI) number and its related keys. The information stored on SIM is used to identify and authenticate subscribers on mobile ICT devices.  SIM contains a unique serial number (ICCID), international mobile subscriber identity (IMSI) number, security authentication and ciphering information, temporary information related to the local network, a list of the services the user has access to, and two passwords: a personal identification number (PIN) for ordinary use, and a personal unblocking key (PUK) for PIN unlocking. SIM is transferable between different mobile devices by removing the card itself. SIM has evolved over a period of time and it is available in different Form Factors(FF) to support different type of mobile devices. The form factors of removable SIM are from 1FF to 4FF (nano-SIM) depending on the size of the SIM. The size of SIM has been reducing for each new generation with 4FF being the smallest SIM in physical size. However, with increasing levels of miniaturization of mobile  & ICT devices and rapid adoption of IoT/M2M devices, the option of embedded SIM (e-SIM) has been introduced which  is replacing physical SIM cards in some domains, including cellular telephony and IoT/M2M devices. eSIM uses a software-based SIM provisioning where the eUICC is embedded into the device and remain irremovable during its lifecycle. The FF currently in use for eSIM is MFF2. The overview of different FF of SIM is given below.

| Form Factor | Shape | Size |
| --- | --- | --- |
| 2FF- Mini SIM |  | Height : 25mm Width: 15mm Thickness: 0.76mm |
| 3FF-Micro SIM |  | Height : 15mm Width: 12mm Thickness: 0.76mm |
| 4FF-Nano SIM |  | Height : 12.3mm Width: 8.8mm Thickness: 0.67mm |
| MFF2- M2M Form Factor |  | Height : 5mm Width: 6mm Thickness: 0.67mm |

Figure-1: SIM and eSIM size and shapes

An eSIM is programmable SIM that is embedded directly into a device. The surface mount format provides the same electrical interface as the full size, 2FF and 3FF SIM cards, but is soldered to a circuit board as part of the manufacturing process. In M2M applications where there is no requirement to change the SIM card, this avoids the requirement for a connector, improving reliability and security. An eSIM can be provisioned remotely; end-users can get MNO added or removed without the need to physically swap a SIM from the device. Also an integrated SIM (iSIM) is being introduced which is a form of SIM directly integrated into the model chip itself. As a consequence they are smaller, cheaper and more reliable than eSIMs. They can improve security and ease the logistics and production of small devices.

This technical report cover following aspects related to use of SIMs and eSIM in the deployment of M2M./IoT capabilities in an enterprise environment.

- SIM and eSIM provisioning process

- The entities involved

- The identifiers involved

- The relationship between the entities

- The identifiers involved

- The relationship of the entities to the identifiers

- Security Considerations

- SIM and eSIM provisioning process

This section provides overview of steps involved in transforming of the raw electronic UICC into a usable SIM/eSIM  for end user.

- SIM: In case of removable SIM, the transformation lifecycle comprise of taking the printed card body, development of IC by milling and embedding the IC into the printed card and subsequently loading of operating systems, IMSI, OTA Keys, application and other keys into the chipset. The high level production lifecycle of SIM are as follows:

Stage 1: Development, Manufacturing, Testing and Packaging of ICs

Stage 2: SIM Product finishing process in which the external finishing is done and SIM hardware is ready for further configuration also called SIM personalisation.

Stage 3: Development and loading of SIM Operating System: The SIM operating system is responsible to manage all execution/process in SIM. The key functions of the SIM Operating System include:

• Management of interfaces between the card and the outside world, primarily in terms of the interchange protocol.

• Management of the files and data held in memory.

• Access control to information and functions (for example, select file, read, write, and update data).

• Management of card security and the cryptographic algorithm procedures.

• Maintaining reliability, particularly in terms of data consistency, sequence interrupts, and recovering from an error.

Stage 4: This is most important stage of SIM production wherein all important data like IMSI/ICCID and security keys such as authentication keys, OTA keys, Application keys, PINs/PUKs and all network related profiles, as provided by network operator, are securely injected into the chip.

- eSIM : The integrity of traditional SIM cards is ensured through the secure SIM personalisation process. As eUICC is already embedded in to the user device,  it require secure facilities and channels for eSIM personalisation to any location where the device can be reached over the internet. eSIM protocols provide security and integrity for data transfer relating to eSIM personalisation. The GSMA has created solutions suited to different types of channels i.e. ‘direct-to-consumer’ for normal consumers and M2M solution for ‘business to business to consumer’ to serve specifically enterprise ecosystem in IoT/M2M use cases. The eSIM provisioning include following phases.

SM-SR

eUICC

- In the initial phase eUICC is designed and operating system, software with bootstrap profile and parameters for creating secure channel for profile management are loaded.

- Subsequently eUICC manufacturer delivers the eUICC to M2M/IoT device Manufacturer which has the responsibility of integrating the eUICC onto the IoT/M2M device.

- The device manufacturer or M2M Service Provider, loads the MNO related parameters, Security keys, Application keys, PIN or other Network related profile parameters. Additionally, profiles and other applications as required for different use cases also loaded into the eUICC.

- During the deployment in M2M applications, the eUICC profiles are enabled remotely using secure platforms. The profiles can also be disabled, deleted or new MNO profile loaded through remote provisioning.

- After enabling of profile the M2M Application Service Provider (ASP) enables the end user to utilise the service supported by different applications.

- The different profiles used in eSIM  personalisation are as follows:

a.	Provisioning Profile :

It is a profile containing network authentication parameters. It enables access to communication network to provide transport capability for eUICC management and profile management between eUICC and remote provisioning Nodes/Servers.

b.	Bootstrap profile:

It is a profile containing network authentication parameters which enable access to communication Network from Remote File Management and OTA platforms.

c.	Operational Profile:

It is a profile comprising of network authentication parameters along with MNO(s) and 3rd party applications. The Profile contains the connectivity parameters, MNO-Security Domain comprising of OTA key sets, at least one Network Access identifier, applications, certificates, algorithm parameters as required. The eUICC can hold multiple operational profiles for respective MNO based on user subscription.

d.	Fallback Profile:

This profile is enabled when the existing or enabled Profile loses network connectivity; the eUICC will then swap the failing profile, to a profile that has the fall-back parameter set as active.

e.	Emergency profile

An Operational Profile with a Profile Attribute allocated to emergency, indicates that the Profile is an Emergency Profile. An Emergency Profile has to comply with regulatory requirements and have the capability to make Emergency Calls and receive calls from an Emergency centre.

- Remote eSIM Provisioning System Architecture: The following is the high level flow for remote provisioning of eSIMs in enterprise M2M ecosystem. Three system elements play key roles in remote eSIM provision.

GSMA Certificate Issuer (CI)

ES2

SM-DP

ES1

eUICC Manufacturer

ES3

ES4

MNO-BSS

SM-SR

SM-SR-x

ES7

ES5

ES8

hghgkk

ES6

ghggh

eUICC in end consumer device

Figure-2 : High Level Architecture of eSIM Provisioning System

  - Subscription Manager-Data Preparation(SM-DP):

This node is responsible for preparation, storing and protection of MNO Profiles. All the MNO specific data such as configuration parameters, IMSI values, ICCID, algorithm, application related keys along with additional files are stored in this server in encrypted form. It also downloads or push and install profiles on eUICC.

  - Subscription Manager-Secure Routing (SM-SR):

This node carries the relevant data pertaining to eUICC and hence manages different functions such as enabling, disabling and deleting different Profiles into the eUICC. It holds the data base of eUICC under its control along with keysets.

During the change of location or change of operator the eUICC parameters can be transferred into another server securely. All the installation, downloading and activation of profiles into eUICC, by SM-DP, can only be done with secure access through SM-SR. It also secures the communications link between the eUICC and SM-DP for the delivery of operator Profiles.

  - eUICC

The eUICC contains one or more subscription Profiles. Each Profile enables the eUICC to function in the same way as a removable SIM.

GSMA Certificate Issuer (CI) issues digital certificates which enable entities to securely communicate with each other. M2M solution authentication uses Pre Shared Key (PSK) with the SM-SR and only allows a single SM-SR to communicate with the eUICC at any given time. However, in case of ‘Business to Consumer’ solution, the PKI based authentication is used. Based upon enterprise level business agreements, eUICC manufacturer can also load Bootstrap Profile or other relevant profiles that enables the device to access a mobile network for provisioning.

- Entities involved and their role in SIM and eSIM Provisioning in Enterprise use case

The entities involved in M2M/IoT eSIM provisioning ecosystem include the eUICC Manufacturer (EUM), IoT Device Manufacturer, Enterprise Customer, MNO and M2M/IoT Service Provider.

Figure 3 shows the eSIM and SIM lifecycle model that forms the basis of the entities and their role in eSIM provisioning.

Consumer

Enterprise

Service 
Provider

Enterprise

Ce

Esp

SPe

Traditional SIM Lifecycle

Download Profile

End of MNO Subscription

Carrier Switching

Usage

- eSIM Lifecycle

Figure 3: SIM and eSIM Lifecycle

  - Consumer/End User:

The Consumer or End user is the person or entity having M2M device and utilizing the IOT services.

  - IoT/M2M Service Provider or Service Provider :

The IoT/M2M Service Provider provides the Network Connectivity and manages SM-DP and SM-SR. It defines the Profile features, Characteristics, applications along with other profile Management related activities such as Profile Creation, Download and deletion etc. This role can also be performed by MNOs in case they setup their in-house SM-DP and SM-SR systems as per GSMA requirement.

  - eUICC Manufacturer:

eUICC manufacturer is responsible for manufacturing the IC Chip, loading the default applications along with default parameters in the eUICC Chip. It also incorporates in device a default profile for initial provisioning.

  - IoT/M2M Device Manufacturer or Enterprise:

Enterprise manufacturers the IoT/M2M device having communication module. This may also include the M2M chipset manufacture, module manufacturer and end device manufacturer.

  - M2M Application Provider:

This entity realises the service logic of M2M application and provide M2M application services to end users by utilising the capabilities/resources of Network Provider, Device Manufacturer and M2M Service Provider.

- Relationship between key entities

The End Consumer is the entity who signs up for a service.  That service may be for an energy supplier, where M2M/IoT is being used to allow the energy supplier to provide connectivity to take remote readings of the meter at the consumer’s premises.  Alternative entities may be associated with the vehicle industry, and may provide connectivity for the vehicle manufacturer for engine management, or for access by the occupants of the vehicle to emergency services, or to infotainment services.

In the basic model of connectivity, the entities are the consumer, the enterprise and the service provider.  The basic model is logical, and shows two instances of the enterprise.  The two instances maybe considered to be two elements of the enterprise, e.g., equipment in the consumer’s location such as a smart meter, and the management software that manages the reading from the consumer equipment, for example billing software.

The elements are connected and the interfaces between them are identified.  The interface between the Consumer and the enterprise is the Ce interface, the interface between the enterprise and the service provider is the Esp interface and the interface between the service provider and the enterprise is the SPe interface.  The basic information that flows across these interfaces is determined by the type of service being offered and how that service is established.

- Operational Scenario for Carrier Switching

There are two different uses cases/models for the deployment of Embedded SIM.

In the first use cases the consumer’s initiates request to the M2M service provider for accessing the Application. This is applicable to consumer device cases such as Mobile Phones, Smart Watch etc. This use does not fall in the category of enterprise use case.

The second use case is where the enterprise has taken initial subscription from service provider ‘X’ and intends to change its relationship and want to migrate to service provider ‘Y’. In this use case, the M2M service Provider initiates the carrier switching request. This generally applies to scenarios where device is remotely located and management is done remotely on eSIM.

In generic terms this is carrier switching as shown in Figure 4.

Consumer

Enterprise

Service

Provider

Enterprise

Ce

Esp

SPe

Service

Provider

X

X

Figure 4: Carrier Switching

In choosing a different Service Provider, it is only the enterprise that is affected.  The consumer has no knowledge of the change nor is the consumer service impacted.

Both of the above models involve the usage of secure element or eUICC and used Pre-Shared Key and Public Key Infrastructure based cryptography.

- Identifier(s) Involved in Carrier Switching

As a minimum, the identifier segment should identify the Enterprise, End Consumer and corresponding IoT/M2M device which may or may not be a telephone number.  If there is  requirement to communicate through a public communications service with the IoT/M2M device then a PSTN/PLMN identifier may be required.  Where no communication is required through PSTN/PLMN and a private identification scheme is used then any relevant identifiers may be used.  When a public identification scheme is used in the relationship between the consumer and the enterprise, it will have to have a relationship with the IMSI.

The relationship between the identifier that is used by the enterprise, the consumer identifier and the IMSI is transferred across the Esp and the SPe interface.

The consumer will have equipment related to the service to be received from the enterprise, often provided by the enterprise, and it will contain either a SIM card or an e-SIM.  Equipment provided by an enterprise without a SIM or an e-SIM is not considered further in this report.

- Security Considerations

Security is the most serious concern in the world of IoT/M2M. Though IoT market has matured sufficiently to address this issue, however, new technology may expose new security vulnerabilities. Since an eSIM is configured remotely, all user credentials are essentially transferred over the air. As a result, side channel attacks are possible against the eSIM remote provisioning architecture. If the hacker is successful in breaking into the connection between the eSIM and the platform, they will have access to actual carrier profiles, which may be exploited in a variety of fraudulent scenarios. Therefore, it is extremely important to implement security controls and process as per GSMA requirements to avoid any risks to end consumers.

- Geographic Regulations

Though GSMA governs and ensures the standardisation of Remote eSIM Provisioning solutions, it is to note that the requirements for hosting a Subscription Management platform and Data-Centers may differ in different Member States based on their local regulations..

Case Studies

Bibliography

[GSMA White Paper CLP.05] - Business Process for Remote SIM Provisioning in M2M

Consumer

Enterprise

Service

Provider

Enterprise

Ce

Esp

SPe

Service

Provider

X

X

Appendix I

Implementation Case Study in India

- Assuming a foreign vehicle with initial foreign MNO eSIM subscription is supplied in India by vehicle manufacturer. The vehicle needs local subscription of Indian MNO as foreign MNO’s eSIM is in international roaming. Though the vehicle manufacturers i.e. Enterprise may continue with international roaming for the vehicle without any change for a specified period of time as per extent regulations, however, due to wide gaps in mobile service tariffs in India and outside India, the Enterprise decides to switch to the services subscription of Indian MNO.  Therefore, the Carrier Switching scenario becomes applicable.  Technically, following two options are possible to implement Carrier Switching.

- SM-DP-2 to SM-SR-1(ES3) Switching:

In this option of Carrier switching, the SM-DP-2 of Indian MNO will interface with the SM-SR-1 of Foreign MNO. After activation of interfaces as per GSMA standard, Indian MNO will push its subscription profile from its SM-DP to the eSIM of vehicle via SM-SR-1 of Foreign MNO. However, due to regulation on certain type of data movement outside geographical boundaries of India, this option is not implemented.

- SM-SR to SM-SR Switching(ES7 integration) :

In SM-SR to SM-SR switching, the foreign MNO’s SM-SR (1) will be integrated with Indian MNO’s SM-SR(2). After successful integration, foreign MNO’s SM-SR will transfer the EIS (eUICC Information Set) to Indian TSP’s SM-SR. Now eSIM ownership/registration lies with Indian MNO and It will be removed from Foreign MNO’s SM-SR. This is as per GSMA guidelines that an eSIM cannot be in two SM-SR’s at the same time. Now Indian MNO will push its subscription from its SM-DP to roaming eSIM via its own SM-SR.

Before carrier switching-

Remote eSIM provisioning by foreign MNO

After Carrier Switching-

Remote e-SIM provisioning by local MNO

SM-DP-2

SM-SR-2

Profile-2

HLR-2

SM-DP-1

SM-SR-1

eSIM

Profile-1

HLR-1

SM-SR Switch

Figure-5: SM-SR to SM-SR Carrier Switching

- Implementation Scenarios :

Following implementation scenarios exist for carrier switching. However, actual implementations are subject to regulatory and legal framework in respective Member State.

- Use Case 1:  Single MNO Subscription

Option 1: SM-DP to SM-SR (ES3 Integration)

- To be  MNO’s SM-DP will be integrated with existing MNO’s SM-SR

- Post that to be MNO will push its subscription from its SM-DP to roaming eSIM via existing MNO’s SM-SR.

Option 2: SM-SR to SM-SR Swap (ES7 Integration)

- Present MNO’s SM-SR will be integrated with to be MNO’s SM-SR

- Post that present MNO’s SM-SR will transfer the EIS (eUICC Information Set) to to be MNO’s SM-SR.

- Now eSIM ownership/registration lies with to be MNO and it will be removed from present MNO’s SM-SR as per GSMA guidelines.

- Now new MNO will push its subscription from its SM-DP to roaming eSIM via its own SM-SR.

- Use Case 2: Main and Fallback  MNO Subscription

The vehicle manufacturer may also need another MNO’s subscription, as fallback network, to ensure seamless network connectivity while travelling in areas where main MNO’s network is not available. The following process of provisioning of main and fallback profile may be used.

Option 1: SM-DP to SM-SR (ES3 Integration)

- Fallback MNO’s SM-DP will be integrated with SM-SR of main MNO.

- Post that fallback MNO will push its subscription from its SM-DP to eSIM via main MNO’s SM-SR as eSIM is registered with main MNO’s SM-SR.

Option 2: Profile Donation

- If fallback network MNO do not have M2M eSIM Platform (SM-DP, SM-SR), in that case it can donate the profile/data to main MNO.

- Then main will configure fallback network MNO’s profile/data in its SM-DP. Post that main MNO’s SM-DP will push fallback MNO’s subscription to the eSIM via its own SM-SR.

____________________