|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG2- |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 2 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 1/2 |  | Virtual, 13-22 March 2023 |  |
| CONTRIBUTION |  |  |  |  |  |
| Source: |  | India |  |  |  |
| Title: |  | E.156 amendments |  |  |  |
| Contact: |  | Tejpal Singh India |  |  | Tel: +919818885790 E-mail: tejpal.singh70@gov.in |

| Abstract: | This contribution proposes to revise Appendix I and Appendix II of E.156 under work item E.156. |
| --- | --- |

Proposals

- ITU-T E.156 define misuse as ‘The use of international telecommunication numbering resources specified in ITU-T Recommendations other than by the assignee or for purposes other than those for which they were assigned or use of unassigned resources.’

Therefore, ‘spoofing’ and ‘tampering’ of International Calling Party Number Delivery (ICPND) is also a misuse of numbering resources and also a non-compliance of ITU-T Recommendation E.157.  Misuse of ICPND through ‘spoofing’ and ‘tampering’ may result in many frauds and affects national security. Therefore, the Appendix II should also include possible actions for this scenario of misuse.

- Appendix I uses two terms i.e. ‘calling party’ in section 1 and ‘calling subscriber’ in remaining sections. These terms are neither defined nor any distinction explained. It is proposed to use the same term i.e. ‘calling party’ for consistency. This term is also used in Appendix IV.

Appendix I

Examples of number resource misuse

(This appendix does not form an integral part of this Recommendation.)

The following constitute examples of misuse.

1. The calling party does not have any reserved or allocated number (e.g., when delivering web or voice over Internet protocol communication services)  for access to public telephone networks, and for call handling requires that an operator or service provider assign that party an ITU-T E.164 number that is random or does not belong to that calling party or is not bound to a user, and the number is transferred over the network.

2. The calling  party has a reserved or allocated number, but during call handling, calling party number is substituted with another number without any justification or agreement in accordance with ITU-T E.157.

3. The calling  party has no reserved or allocated number; in routing an operator or a service provider  does not transfer any identifiers of a calling  party (nor transfer it partly).

4. The calling  party has a reserved or allocated number and during call handling an operator or a service provider ceases the transfer of such a number as an identifier of that  calling party

5. Calling party uses rented numbering resource from other party for making calls for the purpose of concealing the delivery of International Calling Party Number delivery in violation of ITU-T Recommendation E.157

6. The International Calling Party Number delivery is ‘spoofed’ or ‘tampered’ by calling party or any other transit operators in the call completion.

_______________________

Appendix II

Possible national actions in case of misuse of indirectly assigned resources

(This appendix does not form an integral part of this Recommendation.)

Administrations may consider taking the following actions in the case of misuse.

1) Initiate an investigation that initially will include the following:

a. requesting information from the OA regarding how the disputed calls have been transited to the  destination;

b. requesting information from the numbering resource   owner to determine whether the calls had terminated through their switch;

c. obtaining full call records from the OA operator so that these can be examined to determine whether there is evidence of traffic inflation (overlapping calls etc.);

d. if the alleged misuse involves a private branch exchange (PBX) or international call forwarding, checking with any third party carriers to determine whether the calling line identities (CLIs) are genuine or "spoofed".

2) From these investigations, which should be completed as soon as possible, the administration may determine whether this is misuse or misappropriation and also whether the misuse of misappropriation has occurred using number hijacking (i.e., the misuse of a valid number in order to generate revenue for the originator of the misuse).

3) If it involves number hijacking, the administration may recommend that payment be stopped on the basis that the service being charged for has never been delivered, e.g., a call billed to a specific country has been “short-stopped” elsewhere.

4) If the incident involves obvious misuse or misappropriation (traffic inflation, multiple overlapping calls etc.) and involves numbers from the operators who make numbers available for international premium rate number (IPRN) services, then it is also recommended that payment be stopped, and that the IPRN provider who has been assigned these numbers is advised that payment will be withheld.

5) If the incident involves spoofing or tampering of International Calling Party Number Delivery (ICPND) using national numbering resources of Member State, then it is recommended that Member States may implement technical measures to drop the calls at International Gateway while allowing calls with calling line identification of mobile numbers roaming outside the Member State.