|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | T22-SG09-C-0080 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 9 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 2/9 |  | Bangalore, 09-18 May 2023 |  |
| CONTRIBUTION |  |  |  |  |  |
| Source: |  | Ministry of Communications, India |  |  |  |
| Title: |  | Proposed baseline text of proposed draft new ITU-T Technical Report TR.FSR “Technical report on factual subscriber-base reporting and protected content delivery in Conditional Access System (CAS)”. |  |  |  |
| Contact: |  | Avinash Agarwal Ministry of Communications, India |  |  | Tel: +91-9868171222 E-mail: avinash.70@gov.in |
| Contact: |  | Rakesh S. Desai Ministry of Communications, India |  |  | Tel: +91-9426203403 E-mail: rakesh.desai@gov.in |
| Contact: |  | Harsh Sharma Ministry of Communications India |  |  | Tel: +91-9425499000 E-mail: dircb2.tec-dot@gov.in |
| Contact: |  | Rajiv Khattar India |  |  | Tel: +91-9899456795 E-mail: rajivkhattar@gmail.com |

| Abstract: | This Contribution contains the draft baseline text for the draft ITU-T Technical Report TR.FSR "Technical report on factual subscriber-base reporting and protected content delivery in conditional access system (CAS)". In the first SG9 meeting (fully virtual, 6-14 September 2022) , it was agreed to start a new work item on ITU-T Technical Report TR.FSR “Technical report on factual subscriber-base reporting and protected content delivery in conditional access system”. A.13 Justification for this work item is available in the report of Working Part1 [SG9-TD161/Gen (2022-09)]. This work was further discussed during the interim Rapporteur Group e-Meeting of Q2/9 held on 10th February 2023, report available as [SG9-TD286-Gen (2023-05)]. In this contribution, India proposes a new baseline text of this draft Technical Report. |
| --- | --- |
|  | The draft baseline text for the proposed draft ITU-T Technical Report TR.FSR is placed as ANNEX-1. |

______________________

|  |  | ANNEX-1 International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | Technical Report |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (DD MMM YYYY) |
|  | TR.FSR Technical report on factual subscriber-base reporting and protected content delivery in conditional access system (CAS) |  |  |  |

Summary

Technical Report TR-FSR on factual subscriber-base reporting and protected content delivery in conditional access system (CAS) is prepared to specify the terminology definition, service scenario, system architecture, and use cases related to the proposed new Recommendation J.FSR-REQ highlighting the characteristics of related technologies and network architecture. The goal of the technical report is to develop secured network and ecosystem to ensure authorised access to the content utilising the standard conditional access system (CAS) in Digital Addressable Systems (DAS) for television broadcasting. The objective of the technical report is also to supplement the under-development draft new Recommendation J.FSR-REQ that addresses two major concerns related to CAS, namely, underreporting of subscriber numbers and content piracy, leading to revenue loss to broadcasters, content providers, and governments. The proposed new Recommendation J.FSR-REQ elaborates on the various functional requirements of the CAS such as log requirements, reports requirements, database requirements, security requirements, service requirements, and more. Compliance with these requirements in CAS performance will address the twin concerns mentioned above.

Keywords

Conditional Access, Subscriber Management System, CAS, SMS, piracy, misreporting, testbed preparations.

Change Log

This document contains Version 1 of the ITU-T Technical Report TR.FSR on "Factual subscriber-base reporting and protected content delivery in conditional access system (CAS)" approved at the ITU-T Study Group 09 meeting held in Geneva, 06-14 Sept. 2022.

Note

This is an informative ITU-T publication. Mandatory provisions, such as those found in ITU-T Recommendations, are outside the scope of this publication. This publication should only be referenced bibliographically in ITU-T Recommendations.

ITU 2023

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

| Editor: | Avinash Agarwal Ministry of Communications India | Tel: +91-9868171222 Email: avinash.70@gov.in |
| --- | --- | --- |
|  | Rakesh S. Desai Ministry of Communications, India India | Tel: +91-9426203403 E-mail: rakesh.desai@gov.in |

CONTENTS

Page

1 Introduction 1 2 Scope 1 3 References 1 4 Terms and definitions 2 4.1 Terms defined elsewhere 2 4.2 Terms defined here 2 5 Abbreviations 2 6 Background of CAS 3 7 Service Scenario 3 8 System Architecture 4 9 Use cases 5 10 Testbed preparations………………………………………………………………………………………6 11. Functional Requirements………………………………………………………………………...6 12 Concluding Remarks 6 13 Bibliography 7

List of Tables

Page

No table of figures entries found.

List of Figures

Page

No table of figures entries found.

Technical Report ITU-T TR.FSR

ITU-T Technical report on factual subscriber-base reporting and protected content delivery in conditional access system (CAS)

# Introduction

# This report describes the essential elements of the Conditional Access system ( CAS) with the Headend’s in the Cable networks or the DTH ( herein jointly referred as distribution platforms owners DPO ). The objective of the technical report is to supplement the draft new Recommendation J.FSR-REQ that addresses two major concerns related to CAS, namely, underreporting of subscriber numbers and content piracy, leading to revenue loss to broadcasters, content providers, and governments. This draft new Recommendation J.FSR-REQ elaborates on the various functional requirements of the CAS such as log requirements, reports requirements, database requirements, security requirements, service requirements, and more. Compliance with these requirements in CAS performance will address the concerns mentioned above. The objective of this report is also to assist the Conditional access system providers and the DPO in implementation of the CAS in the distribution networks. The correct implementation is the critical element for the factual and accurate reporting of the subscriber numbers, the channels subscribed and the reporting to the content providers and efficient working of the Headend and the consumer premises equipment. This will also help in optimizing the resources at Headend and ensuring that consumer gets an uninterrupted service.

# Scope

The proposed technical report will specify the terminology definition, service scenario, system architecture, and use cases related to the draft new Recommendation J.FSR-REQ.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[1]	 [ITU-R BT.1852-1]	Recommendation ITU-R BT.1852-1 (2016), Conditional-access systems for Digital broadcasting.

[2]	[ITU-T J.95]	Recommendation ITU-T J.95 (1999), Copy protection of intellectual property for Content delivered on cable television systems.

[3]	[ITU-T J.93]	Recommendation ITU-T J.93 (1998), Requirements for conditional access in the Secondary distribution of digital television on cable television systems.

# Terms and definitions

## Terms defined elsewhere

This Technical Report uses the following terms defined elsewhere:

4.1.1	Conditional Access system (CA) [ITU-T J.95]: The complete system for ensuring that cable services are accessible only to those who are entitled to receive them, and that the ordering of such services is not subject to modification or repudiation.

4.1.2	Encryption [ITU-T J.93]: The process of scrambling signals to avoid unauthorized access.

4.1.3	Piracy [ITU-T J.93]: The act of acquiring unauthorized access to programs, usually for the purpose of reselling such access

4.1.4	Scrambling [ITU-T J.93]: The process of using an encryption function to render television and data signals unusable to

4.1.5	Content [ITU-R BT.1852-1]: This is any form of digital data that can be acquired and presented by a device.

4.1.6	Service [ITU-R BT.1852-1]: This is one or more data flows intended to be presented together.

## Terms defined here

4.2.1 Factual subscriber-base reporting: Factual subscriber-base reporting in the context of a conditional access system refers to the accurate and truthful representation of the number of subscribers of each service in a distribution platform without any manipulation or distortion of the underlying data.

4.2.2 Protected delivery of content: Protected delivery of content in the context of a conditional access system used in broadcasting and distribution platforms refers to the secure encrypted transmission of digital content, such as TV programs, movies, or music, to authorized users while preventing unauthorized access or piracy.

# Abbreviations

| API | Application Programming Interface |
| --- | --- |
| CAS | Conditional Access System |
| DB | Database |
| DPO | Distribution Platform Operator |
| DTH | Direct To Home |
| EPG | Electronic Programme Guide |
| FP | Finger Printing |
| FTA | Free-To-Air |
| HITS | Headend In The Sky |
| IPTV | Internet Protocol Television |
| LCN | Logical Channel Number |
| OTA | Over-The-Air |
| OTT | Over-The-Top |
| PIN | Postal Index Number |
| SLA | Service-Level Arrangement |
| SMS | Subscriber Management System |
| STB | Set Top Box |
| TEC | Telecommunication Engineering Centre |
| TRAI | Telecom Regulatory Authority of India |
| UA | Unique Access |
| UI | User Interface |
| VC | Viewing Card |

# Background of CAS

Most countries have implemented Digital Addressable Systems (DAS) for distribution platforms such as Cable TV, Direct to Home (DTH), Internet Protocol Television (IPTV), and Headend In The Sky (HITS). DAS enables subscribers to have a choice and option to select their channels and packages. Conditional Access System (CAS) and Subscriber Management System (SMS) are integral parts of a DAS environment. While CAS is responsible for encrypting the content and its delivery in a secure and encrypted manner only to authorized subscribers, SMS handles the subscriber management functions. The quality of service depends on the CAS and SMS systems deployed by the Distribution Platform Owner (DPO). Since the content providers and broadcasters have no control over the CAS and SMS, as these are deployed at the DPO end, they often complain about the loss of revenue due to various malpractices, such as under-reporting of subscriber base, distribution of channels to unauthorized users, and content piracy. Therefore, it is necessary to frame benchmarks for the CAS systems, prescribing technical specifications focusing on functional requirements to ensure factual reporting of subscriber-base and protected delivery of content to authorized subscribers. Creating a framework that prevents the deployment of sub-standard CAS will bring preventive control from potential threats and revenue losses arising due to the vulnerability of such systems, bring economic efficiency, improve quality of service, and improve the end-consumer experience.

# Service Scenario

Conditional access systems are an important and integral part of the pay tv broadcast and cable environment. The Distribution platforms use Conditional Access (CAS) to ensure that content is securely encrypted and is available to the authorized subscribers. Subscriber Management System (SMS) is another key link between the Content provisioning to the authorized subscribers, Customer Relations Management System of the Distribution platform and payment to the pay tv content providers based on the numbers of the subscribers authorized to watch their content. The quality of service depends on the CAS and SMS systems deployed by the Distribution Platform Owner (DPO).

In any market there are number of CAS and SMS solutions deployed and it is a need that a frame work be developed which ensures that all the CAS and SMS solutions perform a minimum basic requirement.

Figure 1: Relationship of Key Players in a Pay TV Ecosystem (Source: TRAI)

Distribution platforms depend on various subsystem (refer fig 1) and have a relationship and dependency not only for a smooth operation but also for the introduction of the newer features, meeting regulatory and business needs and also keep the system fool proof.

By implementation of a CAS and SMS solution which does not meets the minimum required criteria and functionalities can lead to

i	The CAS and SMS providers are unable maintain and upgrade the products /offerings to the subscribers to meet the growing business requirements

ii	Consumer choices can be limited as there are limitations on the products creations and provisions

iii	The invoice generation to the subscribers is not generated on time thus impacting the service provisioning to them.

# Use cases

Market systems need that CAS and SMS should enable a secure, safe systems which enable the content provider to give content without the fear of being redistributed in an unauthorized manner and the distribution platform gives a transparent and true numbers of its distribution.

All established solutions of CAS and SMS deploy advanced security to ensure that content is secure and the implementation is secure, however sometimes some exceptions are seen. This leads to the disruption of the services to the Distribution platforms as they do not get the content and subscribers are also not able to view the content. The content providers do not get the right due share of the revenue. It is sometimes seen that few DPO also take advantages of the shortcomings in the CAS and the SMS systems and take that route to shortchange the content provider.

The CAS and SMS systems should comply as per below requirements.

a) CAS system through its user interface or application or direct command line interface shall not allow the DPO to directly activate /deactivate STB /VC, channels, packages, create/modify/delete channels and/or packages, add, modify, delete subscriber, etc. The CAS must execute commands initiated from its designated SMS only.

b) Historical logs of package creation and modifications along with date and time stamp should be available in CAS & SMS.

c) Every individual channel should be available only on one unique LCN in the entire network, which includes the LCO networks as well. Channel descriptor should match the channel name. The same channel cannot be distributed in the entire network under another LCN and / or another channel descriptor or in unencrypted form.

d) CAS & SMS should able to handle in ala-carte mode all the channels distributed by the DPO.

e) The CAS & SMS shall be able to tag and blacklist VC and STB numbers that have been involved in piracy in the past to ensure that such VC or the STB cannot be re-deployed.

f) CAS system shall be capable of generating reports pertaining to the channel/ bouquet subscriptions and active/ deactivated subscribers, or any combination thereof; start and end date of the subscribers subscription per package / channel, viewing Card (VC) Number or virtual card number of the STB as the case may be. CAS shall share such report with SMS as a scheduled activity, and also upon request.

# Testbed preparations

To test the CAS, it is essential that the testing agency is able to simulate the right conditions of a network. It should have Signal sources, encoders/transcoders, multiplexers (which can handle the CAS systems working in the region), upconverters, down convertors, a model SMS and relevant Set Top Boxes (STBs).

The signal sources at the Headend at the test lab has to be configured to the MPEG2/MPGE4/H.264 or the H.265 encoders and same to be configured in the Multiplexing equipment.

The CAS has to be configured with the respective Multiplexing equipment and the SMS system. The STBs to be put in the networks in the lab to test the features of the CAS.

# Functional Requirements

The CAS shall meet the following functional requirements: -

# Log and Reporting Requirements: -

# CAS Database Requirements

# Security Requirements

- Service Requirements

Detailed information on the above-mentioned functional requirements may please be perused in the J.FSR-REQ.

# Concluding Remarks

The conformance to the functional requirements prescribed in J.FSR-REQ prevents the deployment of sub-standard CAS that will bring preventive control from potential threats and revenue losses arising due to the vulnerability of such systems, bring economic efficiency, improve quality of service, and improve the end-consumer experience.

# Bibliography

[NIL]