|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG5 |
| --- | --- | --- | --- |
|  |  |  | STUDY GROUP 5 |
|  |  |  | Original: English |
| Question(s): |  | 3/5 | Sofia Antipolis, 13 June-23 June 2023 |
| CONTRIBUTION |  |  |  |
| Source: |  | Ministry of Communication (India) |  |
| Title: |  | Proposal of the content of the draft ITU-T Recommendation K.devices |  |

| Contact: | Neha Upadhyay TEC, DoT Ministry of Communications, India | Tel: +91 11 23323221 Fax: E-mail: neha.upadhyay86@gov.in |
| --- | --- | --- |
|  | Bhoomika Gaur TEC, DoT Ministry of Communications, India | Tel: +91 9315251773 Fax: E-mail: bhoomika.gaur@gov.in |

| Abstract: | In this Contribution there is proposed content to certain sections of the new draft Recommendation ITU-T K.devices |
| --- | --- |

The proposed content of the draft Recommendation ITU-T K.devices is presented below.

Draft Recommendation ITU-T K.devices

RF-EMF exposure assessment of devices operating close to the human body

Summary

With the proliferation of IMT 2020, Internet of Things (IoT) and mmWave wireless communication networks, world is witnessing the emergence of next generation personal communication devices, such as hand-held devices including mobile phones, laptops, tablets, wearable devices, and sensors etc., which can operate at high frequencies, including mmWave frequencies. Technologies like Machine to Machine (M2M) communication, Internet of Things have given rise to the densification of wireless communication networks and have triggered a digital revolution in our daily lives in form of smart homes, smart cities, smart factories, smart vehicles, etc. On one hand, this digital revolution has enabled ubiquitous connectivity not only for ‘everyone’ but also for ‘everything’, and on the other, it has brought wireless networks and devices closer and closer to human life, leading to quantum jump in the number, types and variety of wireless communication devices that operate in close proximity to the human body.

Recommendation ITU-T K.devices focusses on the assessment and monitoring of human exposure to Radio frequency (RF) electromagnetic fields (EMFs) from wireless communication devices operating in close proximity to the human body. This includes procedures for evaluating exposure and the compliance with safe exposure limits, with reference to existing standards. This new Recommendation will give dedicated and clear guidance for the EMF exposure compliance assessment of devices operating close to the human body

Keywords

RF EMF compliance assessment, absorbed power density, SAR, wireless devices operating close to the human body

Introduction

<Optional – This clause should appear only if it contains information different from that in Scope and Summary>

# 1	Scope

This Recommendation gives guidance on the means to assess and monitor human exposure to RF-EMF from wireless communication devices operating in close proximity to human body.

# 2	References

[ITU-T K.91]	Recommendation ITU-T K.91 (2022), Guidance for assessment, evaluation and monitoring of human exposure to radio frequency electromagnetic fields.

[IEC/IEEE 62209-1528] IEC/IEEE 62209-1528:2020, Measurement procedure for the assessment of specific absorption rate of human exposure to radio frequency fields from hand-held and body-worn wireless communication devices − Part 1528: Human models, instrumentation and procedures (Frequency range of 4 MHz to 10 GHz).

[IEC 62209-3]	Measurement procedure for the assessment of specific absorption rate of human exposure to radio frequency fields from hand-held and body-mounted wireless communication devices - Part 3: Vector measurement-based systems (Frequency range of 600 MHz to 6 GHz)

[IEC 62232]	IEC 62232:2017, Determination of RF field strength, power density and SAR in the vicinity of radiocommunication base stations for the purpose of evaluating human exposure

[ICNIRP-2020]            ICNIRP PUBLICATION:2020, ICNIRP GUIDELINES for Limiting Exposure to Electromagnetic Fields (100kHz to 300GHz).

# 3		Definitions

## 3.1	Terms defined elsewhere

<Normally, terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>.

3.1.2	<Term 2> [Reference]: <optional quoted definition>.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

<abbr>	<expansion>

<Include all abbreviations and acronyms used in this Recommendation>

# 5	General Guidance

TBD

# 6	General characteristics of typical wireless communication devices operated in close proximity of human body

TBD

# 7	Physical quantities for demonstrating compliance to safe exposure limits

Personal wireless communication devices, such as hand-held devices (mobile phones, two-way radios, tablets), laptops, desktop computers and body-mounted wireless devices are characterized by either Specific Absorption Rate (SAR), incident power density, or absorbed power density evaluation.

As per the ICNIRP 2020 guidelines for limiting exposure to Electromagnetic fields- for frequencies of operation below about 6 GHz, where the EMFs penetrate deep into tissue, it is useful to describe EMF exposure in terms of specific energy absorption rate (SAR) (W kg−1), while conversely, for frequencies of operation above 6 GHz, where EMFs are absorbed more superficially, it is useful to describe exposure in terms of absorbed power density (Sab) (W m−2).

SAR evaluation of personal wireless communication devices can be performed in accordance with [IEC/IEEE 62209-1528].

Incident power density evaluation could be performed for portable wireless devices in accordance with [IEC TR 63170].

Absorbed power density evaluation standards are currently under development within IEC.

# 8	Compliance assessment and Exclusion criteria

The assessment of EMF exposure compliance for devices used in close proximity to the human body is primarily carried out through measurement of Specific Absorption Rate( SAR) or Power Density to comply with the defined limits.

8.1 Measurement Methodologies

- SAR Measurement

SAR measurement is carried out to assess the SAR value of RF devices used in close proximity to the human body. SAR measurement uses standardized models of the human head,body, limbs that are filled with liquids that simulate the RF absorption characteristics of different human tissues. In order to determine compliance, each RF device is tested while operating at its highest power level in all the frequency bands in which it operates, and in various specific positions, to simulate the way different users’ typically use the RF device. To test RF devices for SAR compliance, the RF device is placed in various common positions next to the head and body, and a robotic probe takes a series of measurements of the electric field at specific pinpoint locations in a very precise, grid-like pattern. The worst case scenario for RF exposure is expected to be simulated during the SAR measurement. The highest SAR value observed for the various positions or modes of operation of the device is reported as the SAR value for the device.

- Power Density Measurement

This technique is used in case of mmWave frequency of operation in the RF devices. Unlike the lower frequency signals, mmWave signals do not get absorbed in the human body to a higher degree. In fact, mmWave radiation penetrates only a few mm of the skin surface and does not reach the inner regions of the body. Hence, specially in case of 5G (IMT-2020) devices, for assessing compliance to RF EMF exposure, the measurement of power density for the higher frequency bands of operation has to be employed.

8.2 Compliance Limits for SAR and Power Density

ICNIRP defines the limits for SAR and Power Density for ensuring safe exposure to EMF from devices. Countries adopt these limits, however there are many countries who prescribe their own limits for these quantities.

8.3 Standards for different Measurement Methodologies

- Standards for SAR Measurement

- IEC 62209-1528: Measurement procedure for the assessment of specific absorption rate of human exposure to radio frequency fields from hand-held and body-worn wireless communication devices - Human models, instrumentation and procedures (Frequency range of 4 MHz to 10 GHz)

IEC/IEEE 62209-1528:2020 specifies protocols and test procedures for the reproducible and repeatable measurement of the conservative exposure peak spatial average SAR (psSAR) induced inside a simplified model of the head and the body by radio-frequency (RF) transmitting devices, with a defined measurement uncertainty. These protocols and procedures apply to a significant majority of the population, including children, during the use of hand-held and body-worn wireless communication devices. These devices include single or multiple transmitters or antennas, and are operated with their radiating structure(s) at distances up to 200 mm from a human head or body.

This document is employed to evaluate SAR compliance of different types of wireless communication devices used next to the ear, in front of the face, mounted on the body, operating in conjunction with other RF-transmitting, non-transmitting devices or accessories (e.g. belt-clips), or embedded in garments. The applicable frequency range is from 4 MHz to 10 GHz. Devices operating in the applicable frequency range can be tested using the phantoms and other requirements defined in this document.

The device categories covered include, but are not limited to, mobile telephones, cordless microphones, and radio transmitters in personal, desktop and laptop computers, for multi band operations using single or multiple antennas, including push-to-talk devices. This document can also be applied for wireless power transfer devices operating above 4 MHz.

- IEC 62209-3: Measurement procedure for the assessment of specific absorption rate of human exposure to radio frequency fields from hand-held and body-mounted wireless communication devices - Part 3: Vector measurement-based systems (Frequency range of 600 MHz to 6 GHz)

IEC 62209-3: 2019 specifies measurement protocols and test procedures for the reproducible measurement of peak spatial-average specific absorption rate (psSAR) induced inside a simplified model of a human head or body by radio-frequency (RF) transmitting devices, with a specified measurement uncertainty. Requirements are provided for psSAR assessment using vector measurement-based systems. Such systems determine the psSAR by three-dimensional (3D) field reconstruction within the volume of interest in accordance with the requirements herein for the measurement system, calibration, uncertainty assessment and validation methods. The protocols and procedures apply for the psSAR assessments covering a significant majority of people including children during use of wireless communication devices operated in close proximity to the head or body.

This document is applicable to wireless communication devices intended to be used at a position near the human head or body at distances up to and including 200 mm. This document may be employed to evaluate SAR compliance of different types of wireless communication devices used next to the ear, in front of the face, mounted on the body, combined with other RF-transmitting or non-transmitting devices or accessories (e.g. belt-clip), or embedded in garments. The overall applicable frequency range is from 600 MHz to 6 GHz.

The wireless communication device categories covered include but are not limited to mobile telephones, cordless microphones, auxiliary broadcast devices and radio transmitters in personal computers, desktop and laptop devices, multi-band, multi-antenna, and push-to-talk devices.

- Standards for Power Density Measurement

- IEC/IEEE 63195-1: Assessment of power density of human exposure to radio frequency fields from wireless devices in close proximity to the head and body (frequency range of 6 GHz to 300 GHz) - Part 1: Measurement procedure

IEC/IEEE 63195-1:2022 specifies protocols and test procedures for repeatable and reproducible measurements of power density (PD) that provide conservative estimates of exposure incident to a human head or body due to radio-frequency (RF) electromagnetic field (EMF) transmitting communication devices, with a specified measurement uncertainty. These protocols and procedures apply for exposure evaluations of a significant majority of the population during the use of hand-held and body-worn RF transmitting communication devices. The methods apply for devices that can feature single or multiple transmitters or antennas, and can be operated with their radiating structure(s) at distances up to 200 mm from a human head or body.

The methods of this document can be used to determine conformity with applicable maximum PD requirements of different types of RF transmitting communication devices being used in close proximity to the head and body, including if combined with other RF transmitting or non-transmitting devices or accessories (e.g. belt-clip), or embedded in garments. The overall applicable frequency range of these protocols and procedures is from 6 GHz to 300 GHz.

The RF transmitting communication device categories covered in this document include but are not limited to mobile telephones, radio transmitters in personal computers, desktop and laptop devices, and multi-band and multi-antenna devices.

8.5 Factors for consideration in compliance assessment

Compliance assessment to safe exposure to EMF from devices is dependent on a number of factors. These factors govern which limits of SAR or power density apply, which standard of measurement to use, number of measurements that need to be carried out for a particular device etc. These factors that affect the compliance assessment of devices are as below:

- Category of device (occupational use/general use)

- Usage position of the device (near head, on the body, on the limbs, in close proximity to the body etc.)

- Frequency of operation of the device (SAR measurement for < 6 GHz range, Power density measurement for > 6 GHz range)

- Operating power of the device

- Different communication modes of the device (cellular, Wi-Fi, Bluetooth, NFC etc.)

- Distance of measurement from device.

8.6 Computational Methods for EMF Exposure Assessment

<To be developed, the following standards and their brief description may be included>

- IEC/IEEE 63195-2: Assessment of power density of human exposure to radio frequency fields from wireless devices in close proximity to the head and body (frequency range of 6 GHz to 300 GHz) - Part 2: Computational procedure

- IEC/IEEE 62704-1:2017 Determining the peak spatial-average specific absorption rate (SAR) in the human body from wireless communications devices, 30 MHz to 6 GHz - Part 1: General requirements for using the finite difference time-domain (FDTD) method for SAR calculations

- IEC/IEEE 62704-4:2020 Determining the peak spatial-average specific absorption rate (SAR) in the human body from wireless communication devices, 30 MHz to 6 GHz - Part 4: General requirements for using the finite element method for SAR calculations

8.7 Exclusion Criteria in Conformance Assessment

For certain RF devices or under a certain mode of operation of a RF device, the maximum operating power is below a certain threshold and hence the measurement of SAR in such cases is not required. These threshold power levels are prescribed in standards as well as defined by certain countries under their regulatory frameworks.

# 9 	Impact on the overall exposure level

TBD

_______________________