eSIM Summary

Q. The suggestion of designing a robust regulatory body such as NPCI which continuously vets the complete Chip Ecosystem including Embedded OS and give certification may be examined in next 30 days by TEC/NCCS. In GSM it is largely based of self-certification.

Q. There is a need to set up SIM testing lab under DoT in order to continuously validate, Re-Validate and In-Validate any SIM card component threatening national security elements. TEC/ NCCS may explore identify a private job or set up this lab by themselves in next 3 months. Proposal may be fast tracked.

- eSIM manufacturing consist of several stages, First Design House stage (Chip manufacturing), Second is Fabrication stage, Next is Wafer fabrication level, then Dicing Module and finally chip packaging.

- eSIM chip consists of 3 layers Integrated Chip, Operating System and Application Platform. Thus, at every level security certification is requirement. At present, for chip security and OS platform security certification is given by GSMA eSA or Common Criteria Certification, for Application platform Java standards maybe applied on Production profile.

- Standards used by eSIM ecosystem are ETSI TS 102 221, GSMA SGP .02, SGP .22, ETSI TS 102 671 (M2M UICC). For OTA services ETSI 102 225, ETSI 102 226, 3GPP 23 048. Profile Management of eSIM includes TCA compliance, Java card compliance Profile download including security data using GSMA RSP, profile can be deleted/activated/deactivated, profile switching can also be done using SM-SR/auto/using OTA.

- In eUICC ecosystem consist of multiple stakeholders, connected to eSIM of an Automotive for an instance, such as Use case Application Owner, Application Developer, Use case Application provider, Collator/Decollator, Service Provider RSP & OTA, OEM, Mobile Network Operators, and last but not the least Custodian. These all stakeholder are connected with various Cloud services. Various parties seeking eSIM connectivity for cloud services which could lead to security requirements. As the matter of fact, no security validator is in between the eSIM and Application provider, Vulnerability can be pushed via these applications.

- In case of Connected Vehicle there are some security requirements both for Hardware and Software. A vehicle, having an embedded SIM captures data such as vehicle health feed, vehicle location tracking and User specific information, driving behavior, network switching application and other specific application. Further, the vehicle is connected to MNOs for Active data transfer and fallbacks is given to eSIM’s application domain. Now, at every stage security is required, eSIM layered security, Security required for connecting module having native design, as well as App security is also required.

- Following actions may be needed to secure IoT automotive for its privacy and security are –

  - Regulatory policy for security assurance for eSIM platform or acceptance of Global certification scheme from CC or from GSMA eSA.

  - TEC TR Security by design implementation in IoT devices (OEM)

  - eSIM OS platform to handle the security verification for Remote application to be downloaded.