|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  |  |  | SG5-CXXX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | STUDY GROUP 5 |  |  |
|  |  |  |  |  | Original: English |  |  |
| Question(s): |  | 4/5 |  |  |  | Geneva, 03-12 June 2025 |  |
| CONTRIBUTION |  |  |  |  |  |  |  |
| Source: |  | Ministry of Communications, India |  |  |  |  |  |
| Title: |  |  | Proposal to update baseline text of draft new ITU-T Recommendation K.emc_satellite_ES “Electromagnetic compatibility requirements and test methods for Earth stations for satellite communications” [Geneva, 03-12 June 2025] |  |  |  |  |
| Contact: |  |  |  | Anshul Kumar Gupta TEC, DOT Ministry of Communications, India |  |  | Tel: +91 9785951159 E-mail: anshulkr.gupta@gov.in |
| Contact: |  |  |  | Neha Upadhyay DoT Ministry of Communications, India |  |  | Tel: +91 E-mail: neha.upadhyay86@gov.in |
| Abstract: |  |  | This document contain proposal for update in baseline text of the draft new ITU-T Recommendation K.emc_satellite_ES “Electromagnetic compatibility requirements and test methods for Earth stations for satellite communications”. |  |  |  |  |

# Proposal

This document proposes following updates in baseline text of draft new ITU-T Recommendation K.emc_satellite_ES:

- Summary added

- Update in reference

- Addition of content in clause 6 and 7

- Update in clause 8 and 9.

Draft new Recommandation K.emc_satellite_ES

Electromagnetic compatibility requirements and test methods for Earth stations for satellite communications

Summary

This Recommendation establishes the essential electromagnetic compatibility (EMC) requirements for Satellite Earth Stations and ancillary accessories used in the Fixed Satellite Services (FSS). This Recommendation specifies the emission and immunity requirements for Satellite Earth Stations and ancillary equipment. It also describes test conditions for emission and immunity testing. Performance assessment and criteria for immunity tests are also specified.

Keywords

Electromagnetic compatibility, EMC, satellite earth station

Draft new Recommandation K.emc_satellite_ES

Electromagnetic compatibility requirements and test methods for Earth stations for satellite communications

# Scope

This Recommendation defines the essential electromagnetic compatibility (EMC) requirements for Satellite Earth Stations and ancillary accessories used in the Fixed Satellite Services (FSS). This Recommendation also describes the test conditions for emission and immunity testing. Methods of measurement and limits for emissions, test methods and levels, performance assessment, and performance criteria for immunity tests are also specified. This new Recommendation will not cover any spectrum requirement of these devices because these provisions are produced by ITU-R.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T K.116]	Recommendation ITU-T K.116 (2019), Electromagnetic compatibility requirements and test methods for radio telecommunication terminal equipment.

[ITU-T K.136]	Recommendation ITU-T K.136 (2022), Electromagnetic compatibility requirements radio telecommunication equipment

[IEC CISPR 32]	IEC CISPR 32 (2015), A1 (2019), Electromagnetic compatibility of multimedia equipment – Emission requirements.

[IEC 61000-3-2]	IEC 61000-3-2 (2018), Electromagnetic compatibility (EMC) Part 3-2: Limits – Limits for harmonic current emissions (equipment input current ≤ 16 A per phase).

[IEC 61000-3-3]	IEC 61000-3-3 (2013), A1 (2017) Electromagnetic compatibility (EMC) – Part 3-3: Limits – Limitation of voltage changes, voltage fluctuations and flicker in public low-voltage supply systems, for equipment with rated current ≤ 16 A per phase and not subject to conditional connection.

[IEC 61000-3-11]	IEC 61000-3-11 (2017), Electromagnetic compatibility (EMC) – Part 3-11: Limits – Limitation of voltage changes, voltage fluctuations and flicker in public low-voltage supply systems – Equipment with rated current ≤ 75 A and subject to conditional connection.

[IEC 61000-3-12]	IEC 61000-3-12 (2012), Electromagnetic compatibility (EMC) – Part 3-12: Limits – Limits for harmonic currents produced by equipment connected to public low-voltage systems with input current > 16 A and ≤ 75 A per phase.

[IEC 61000-4-2]	IEC 61000-4-2 (2008), Electromagnetic compatibility (EMC) – Part 4-2: Testing and measurement techniques – Electrostatic discharge immunity test.

[IEC 61000-4-3]	IEC 61000-4-3 (2010), Electromagnetic compatibility (EMC) – Part 4-3: Testing and measurement techniques – Radiated, radio-frequency, electromagnetic field immunity test.

[IEC 61000-4-4]	IEC 61000-4-4 (2012), Electromagnetic compatibility (EMC) – Part 4-4: Testing and measurement techniques – Electrical fast transient/burst immunity test.

[IEC 61000-4-5]	IEC 61000-4-5 (2014), Electromagnetic compatibility (EMC) – Part 4-5: Testing and Measurement Techniques – Surge Immunity Test.

[IEC 61000-4-6]	IEC 61000-4-6 (2023), Electromagnetic compatibility (EMC) – Part 4-6: Testing and measurement techniques – Immunity to conducted disturbances, induced by radio-frequency fields.

[IEC 61000-4-11]	IEC 61000-4-11 (2004), AMD1 (2017), Electromagnetic compatibility (EMC) – Part 4-11: Testing and measurement techniques – Voltage dips, short interruptions and voltage variations immunity test.

# Definitions

## Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	ancillary equipment [b-ETSI EN 301 489-1]: Electrical or electronic equipment, that is intended to be used with a receiver or transmitter.

NOTE – It is considered as an ancillary equipment if:

–	the equipment is intended for use with a receiver or transmitter to provide additional operational and/or control features to the radio equipment, (e.g., to extend control to another position or location); and

–	the equipment cannot be used on a stand-alone basis to provide user functions independently of a receiver or transmitter; and

–	the receiver or transmitter, to which it is connected, is capable of providing some intended operation such as transmitting and/or receiving without the ancillary equipment (i.e., it is not a sub-unit of the main equipment essential to the main equipment basic functions).

## Terms defined in this Recommendation

This Recommendation defines the following term:

<TBD>

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

RF	Radio Frequency

# Conventions

None.

# Test conditions

## General

The equipment shall be tested under normal test conditions according to the satellite product standard or the information accompanying the equipment, which are within the manufacturer's declared range of humidity, temperature and supply voltage. The test conditions shall be recorded in the test report.

The Equipment Under Test (EUT) is a satellite Earth station with the selected configuration of ancillary equipment.

For Earth Stations with or without ancillary equipment, and/or various terrestrial ports, the selection of test configurations shall be determined. The mode of operation and the EUT operational frequencies shall represent the intended use and shall be recorded in the test report.Arrangements for test signals

## Arrangements for test signals

## The provisions of ITU-T K.116, section 6.2 shall apply with the following additions.

In order to measure the system emissions and electromagnetic immunity under operational conditions, the following arrangements shall be provided:

- a drive equipment to put the ES terminal in its normal operating mode, and providing the ES with a receive signal to emulate the operational conditions of reception. This equipment shall control the EUT, when it is capable of transmission, so that it switches between the transmission disabled, carrier-on and carrier-off states;

- a Quality of Transmission Measurement Apparatus (QTMA).

For the measurement of the quality of transmission, a communications link shall be established and the wanted input signal shall be applied to the Radio Frequency (RF) input of the receiver via the antenna.

### Arrangements for test signals at the input of transmitters

The provisions of ITU-T K.116, section 6.2.1 shall apply.

### Arrangements for test signals at the output of transmitters

The provisions of ITU-T K.116, section 6.2.2 shall apply.

### Arrangements for test signals at the input of receivers

The provisions of ITU-T K.116, section 6.2.3 shall apply with the following additions.

If necessary, an appropriate test fixture and/or message generator capable of generating the wanted input signal may be supplied with the satellite receiver to facilitate testing.

Alternatively, the wanted signal may be presented to the EUT via a live sky signal from outdoors.

For radiated immunity, the level of the wanted signal at the input of the receiver or the enclosure port of the EUT, shall be 20 dB (±3 dB) above the Pmin for the EUT or be representative of the EUT intended use. For all other tests the level of the wanted signal, required to establish a communication link, shall be representative of the EUT intended use.

NOTE: A simple method to establish the required communication link is establishing link, reduce power to point of link failure then increase by 20 dB.

### Arrangements for test signals at the output of receivers

The provisions of ITU-T K.116, section 6.2.4 shall apply.

## RF exclusion band

The provisions of ITU-T K.116, section 6.3 shall apply with the following additions.

There shall be no exclusion bands for the ancillary equipment.

Exclusion bands shall be in accordance with clauses 6.3.1 and 6.3.2.

### Transmitter exclusion band

The provisions of ITU-T K.136, section 7.3.1shall apply.

### Receiver exclusion band

For EUT that operate above 6 GHz, there is no exclusion band specified as test ranges stop at 6 GHz.

For EUT that operate below 6 GHz, the provisions of ITU-T K.136, section 7.3.2 shall apply.

### Narrow band responses of receivers

The provision of ITU-T K.116, section 6.4 shall apply.

## Normal test modulation

The test modulation signal shall be a signal generated by a signal generator, so that the EUT receives a correctly representative modulated signal.

The wanted signals required to establish a communications link shall be representative of the EUT's intended use.

# Performance assessment

## General

At the time of submission of the equipment for test, the information required in ITU-T K.116, section 7.1 and the following should be recorded in the test report:

- The dedicated class (A or B) for the ES in accordance with the information contained in the instructions accompanying the ES

- The ranges of the operational parameters, e.g. the power delivered to the antenna, operating frequency ranges;

- The minimum quality of transmission, and the method to be used to assess it.

# Equipment Configuration

# For radiation measurements in carrier-on state, the ES shall be put in a continuous transmit mode or to the maximum burst rate where applicable. The ES shall be operated at the maximum operating Equivalent Isotropically Radiated Power (EIRP).

# <TBD>

# Equipment classification

# The equipment are classified into the two classes:

# Class A of ES is where no loss of function is accepted during immunity testing with EMC transient phenomena as defined in clause 8.3;

# Class B ES is where any loss of function during immunity testing with EMC transient phenomena shall be in accordance with performance criteria defined in clause 8.4

<TBD>

# Performance criteria

# General

# Performance criteria for Continuous phenomena applied to the EUT

# Performance criteria for Transient phenomena applied to a class A EUT

# Performance criteria for Transient phenomena applied to a class B EUT

<TBD>

# Applicability overview tables

## Emission

## Immunity

<TBD>

<TBD>

Bibliography

[b-ITU-T K.116]	Recommendation ITU-T K.116 (2019), Electromagnetic compatibility requirements and test methods for radio telecommunication terminal equipment

[b-ETSI EN 301 489-1]	ETSI EN 301 489-1 (2019-11), ElectroMagnetic Compatibility (EMC) standard for radio equipment and services; Part 1: Common technical requirements

[b-ETSI EN 301 489-12] 	ETSI EN 301 489-12 V3.1.1 (2019-04): Electro Magnetic Compatibility (EMC) standard for radio equipment and services; Part 12: Specific conditions for Very Small Aperture Terminal, Satellite Interactive Earth Stations operated in the frequency ranges between 4 GHz and 30 GHz in the Fixed Satellite Service (FSS)

______________