|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  |  |  | SG5-CXXX |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | STUDY GROUP 5 |  |  |
|  |  |  |  |  | Original: English |  |  |
| Question(s): |  | 4/5 |  |  |  | Geneva, 03-12 June 2025 |  |
| CONTRIBUTION |  |  |  |  |  |  |  |
| Source: |  | Ministry of Communications, India |  |  |  |  |  |
| Title: |  |  | Proposal to update baseline text of new ITU-T Recommendation K.EMC_UWB "Electromagnetic Compatibility requirements and test methods for Ultra Wide Band Equipment" [Geneva, 03-12 June 2025] |  |  |  |  |
| Contact: |  |  |  | Anshul Kumar Gupta TEC, DOT Ministry of Communications, India |  |  | Tel: +91 9785951159 E-mail: anshulkr.gupta@gov.in |
| Contact: |  |  |  | Neha Upadhyay DoT Ministry of Communications, India |  |  | Tel: +91 E-mail: neha.upadhyay86@gov.in |

| Abstract: | This document contains proposal for update in text of the new Recommendation ITU-T K.emc_UWB “Electromagnetic compatibility requirements and test methods for Ultra Wide Band equipment” for discussion in the Q4 meeting (Geneva, 03-12 June 2025). |
| --- | --- |

# Introduction

This document proposes to update the following in baseline text:

- Provide text for clause 6  and 8 of document;

Draft new Recommendation ITU-T K.emc_UWB

Electromagnetic compatibility requirements and test methods 
for ultra wide band equipment

Summary

This Recommendation establishes the essential electromagnetic compatibility (EMC) requirements for ultra wide band equipment and ancillary accessories. This Recommendation specifies the emission and immunity requirements for ultra wide band and ancillary equipment. It also describes test conditions for emission and immunity testing. Performance assessment and criteria for immunity tests are also specified.

Key words:

EMC, radio, ultra-wide band, short range

Draft new Recommendation ITU-T K.emc_UWB

Electromagnetic compatibility requirements and test methods 
for ultra wide band equipment

# Scope

This Recommendation defines the essential electromagnetic compatibility (EMC) requirements for ultra wide band equipment; such as such as short-range devices used in medical applications, short-range indoor and outdoor communications, intelligent transportation, which utilize both Impulse based and RF Carrier based Technology. This Recommendation also describes the test conditions for emission and immunity testing. Methods of measurement and limits for emissions, test methods and levels, performance assessment, and performance criteria for immunity tests are also specified. If there is an ITU-R provision, it takes precedence to this Recommendation.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T K.136]	Recommendation ITU-T K.136 (2022), Electromagnetic compatibility requirements radio telecommunication equipment

[IEC CISPR 32]	IEC CISPR 32 (2015), A1 (2019), Electromagnetic compatibility of multimedia equipment – Emission requirements.

[IEC 61000-3-2]	IEC 61000-3-2 (2018), Electromagnetic compatibility (EMC) Part 3-2: Limits – Limits for harmonic current emissions (equipment input current ≤ 16 A per phase).

[IEC 61000-3-3]	IEC 61000-3-3 (2013), A1 (2017) Electromagnetic compatibility (EMC) – Part 3-3: Limits – Limitation of voltage changes, voltage fluctuations and flicker in public low-voltage supply systems, for equipment with rated current ≤ 16 A per phase and not subject to conditional connection.

[IEC 61000-3-11]	IEC 61000-3-11 (2017), Electromagnetic compatibility (EMC) – Part 3-11: Limits – Limitation of voltage changes, voltage fluctuations and flicker in public low-voltage supply systems – Equipment with rated current ≤ 75 A and subject to conditional connection.

[IEC 61000-3-12]	IEC 61000-3-12 (2012), Electromagnetic compatibility (EMC) – Part 3-12: Limits – Limits for harmonic currents produced by equipment connected to public low-voltage systems with input current > 16 A and ≤ 75 A per phase.

[IEC 61000-4-2]	IEC 61000-4-2 (2008), Electromagnetic compatibility (EMC) – Part 4-2: Testing and measurement techniques – Electrostatic discharge immunity test.

[IEC 61000-4-3]	IEC 61000-4-3 (2020), Electromagnetic compatibility (EMC) – Part 4-3: Testing and measurement techniques – Radiated, radio-frequency, electromagnetic field immunity test.

[IEC 61000-4-4]	IEC 61000-4-4 (2012), Electromagnetic compatibility (EMC) – Part 4-4: Testing and measurement techniques – Electrical fast transient/burst immunity test.

[IEC 61000-4-5]	IEC 61000-4-5 (2014), Electromagnetic compatibility (EMC) – Part 4-5: Testing and Measurement Techniques – Surge Immunity Test.

[IEC 61000-4-6]	IEC 61000-4-6 (2023), Electromagnetic compatibility (EMC) – Part 4-6: Testing and measurement techniques – Immunity to conducted disturbances, induced by radio-frequency fields.

[IEC 61000-4-11]	IEC 61000-4-11 (2004), AMD1 (2017), Electromagnetic compatibility (EMC) – Part 4-11: Testing and measurement techniques – Voltage dips, short interruptions and voltage variations immunity test.

# Definitions

## Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	ancillary equipment [b-ETSI EN 301 489-1]: Electrical or electronic equipment, that is intended to be used with a receiver or transmitter.

NOTE – It is considered as an ancillary equipment if:

–	the equipment is intended for use with a receiver or transmitter to provide additional operational and/or control features to the radio equipment, (e.g., to extend control to another position or location); and

–	the equipment cannot be used on a stand-alone basis to provide user functions independently of a receiver or transmitter; and

–	the receiver or transmitter, to which it is connected, is capable of providing some intended operation such as transmitting and/or receiving without the ancillary equipment (i.e., it is not a sub-unit of the main equipment essential to the main equipment basic functions).

3.1.2	portable equipment [b-ETSI EN 301 489-1]: Radio and/or ancillary equipment intended for portable (e.g., handheld) operation, powered by its own integral battery.

3.1.3 Ultra Wide Band (UWB) technology [b-ETSI EN 300 440]: technology for short-range radio communication, involving the intentional generation and transmission of radio-frequency energy that spreads over a very large frequency range, which may overlap several frequency bands allocated to radio communication services as defined in Recommendation ITU-R SM.1755 [i.3]

## Terms defined in this Recommendation

This Recommendation defines the following term:

<TBD>

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

RF	Radio Frequency

UWB	Ultra-Wide Band

# Conventions

None.

- Test Conditions

  - General

The equipment shall be tested under normal test conditions according to the relevant product and basic standards or the information accompanying the equipment, which are within the manufacturer's declared range of humidity, temperature and supply voltage. The test conditions shall be recorded in the test report.

The test configuration and mode of operation shall represent the intended use and shall be recorded in the test report.

  - Arrangements for test signals

    - General

The provisions of ITU-T K.136, clause 7.2 shall apply.

    - Arrangements for test signals at the RF input of transmitters

The provisions of ITU-T K.136, clause 7.2.1 shall apply.

    - Arrangements for test signals at the RF output of transmitters

The provisions of ITU-T K.136, clause 7.2.2 shall apply with the following modification.

The transmitter shall be operated at its maximum rated RF output power, modulated with normal test modulation.

    - Arrangements for test signals at the RF input of receivers

The provisions of ITU-T K.136, clause 7.2.3 shall apply with the following modification.

A communication link shall be established if appropriate at the start of the test and maintained during the test.

For all radio determination devices, the normal operation mode shall be applied.

    - Arrangements for test signals at the RF output of receivers

The provisions of ITU-T K.136, clause 7.2.4 shall apply.

    - Arrangements for testing transmitter and receiver together (as a system)

The provisions of ITU-T K.136, clause 7.2.5 shall apply with the following modification.

For the immunity tests of duplex transceivers, the EUT may be configured in the repeater mode, consistent with the conditions given above.

  - RF Exclusion bands

  - UWB devices in the scope of the present document can operate in a broad permitted range of frequencies from 30 MHz to 10.6 GHz, as defined in table 1 of the present document. The intended range of operation gives the preferred range of operating frequencies for the UWB operation based on the allowed spectrum mask with increase permitted emission levels in the intended range of operation

  - The RF exclusion band applies to UWB equipment with an operating frequency up to 6 GHz, or for equipment operating above 6 GHz, but whose RF bandwidth extends to a frequency below 6 GHz. For equipment operating at frequencies above 6 GHz and whose RF bandwidth does not extend to a frequency below 6 GHz, there is no exclusion band.

    - for EMC emission test

The RF exclusion band for UWB equipment under EMC emission testing is the operating bandwidth(s) of UWB equipment:

• The lower frequency of the exclusion band is the lower frequency of the operating bandwidth(s)

• The upper frequency of the exclusion band is the upper frequency of the operating bandwidth(s)

There shall be no frequency exclusion band applied to emission measurements of the receiver part of transceivers or the stand alone receiver under test, and/or associated ancillary equipment

    - For EMC immunity test

The exclusion band for UWB equipment under EMC immunity testing is calculated as below:

For the lower edge for the exclusion band: EXband(lower) = BandRX (lower) – BWRX and

for the upper edge of the exclusion band: EXband(upper) = BandRX(upper) + BWRX

• BWRX corresponds to the operating bandwidth of the UWB device.

• BandRX(lower) corresponds to the lower edge of the operating bandwidth.

• BandRX(upper) corresponds to the upper edge of the operating bandwidth.

Editor’s Note: The operating bandwidth is the -10 dBc bandwidth(s) of intended UWB signal(s) transmitted by the equipment

  - Narrow band responses of receivers

The provision of ITU-T K.136, clause 7.4 shall apply.

  - Normal test modulation

The transmitter under test shall be modulated according to the normal test modulation specified in the relevant product standard dealing with the particular type of UWB equipment. The receiver under test shall be provided with a wanted RF input signal modulated according to the normal test modulation specified

The test signal generator (modulation) shall be able to produce a continuous stream of data. The test signal receiver (de-modulator) shall be, where appropriate, able to produce a readout of Bit Error Ratio (BER) of a continuous data stream acceptance.

- Performance assessment

  - General

The provisions of ITU-T K.136, clause 8.1 shall apply.

  - Equipment which can provide an UWB communications link

The provisions of ITU-T K.136, clause 8.2 shall apply.

  - Equipment which does not provide an UWB communications link

The provisions of ITU-T K.136, clause 8.3 shall apply.

  - Ancillary equipment

The provisions of ITU-T K.136, clause 8.4 shall apply.

  - Equipment classification

The UWB equipment shall be classified as one or a combination of the following classes:

- Generic UWB

- UWB for location tracking applications

(iii)	For vehicular use

(iv)	Material sensing and object discrimination

(v)		GPR/WPR (Ground- and Wall- Probing Radar applications)

(vi)	(T)LPR (Tank; Level Probing Radar)

- Performance criteria

## General

The performance criteria are used to take a decision on whether a radio equipment passes or fails immunity tests.

Only the performance criteria specified in this recommendation or in ITU-T K.136 where referenced shall apply.

The equipment shall meet the minimum performance criteria as specified in clauses 8.2 and 8.3.

## Performance criteria for Continuous Phenomena

<TBD>

## Performance criteria for Transient Phenomena

<TBD>

- Applicability overview tables

## Emission

  - Immunity

<TBD>

- Methods of measurement and limits for EMC emissions

<TBD>

- Test methods and levels for immunity tests

<TBD>

Bibliography

[b-ITU-T K.116]	Recommendation ITU-T K.116 (2019), Electromagnetic compatibility requirements and test methods for radio telecommunication terminal equipment [b-ETSI EN 301 489-1]	ETSI EN 301 489-1 (2019-11), ElectroMagnetic Compatibility (EMC) standard for radio equipment and services; Part 1: Common technical requirements

[b-ETSI EN 301 489-33]	ETSI EN 301 489-33 (2019-04), ElectroMagnetic Compatibility (EMC) standard for radio equipment and services; Part 33: Specific conditions for Ultra-WideBand (UWB) devices.

[b-ETSI EN 302 065-1]	ETSI EN 302 065-1 (2016-11), Short Range Devices using Ultra Wide Band Technologies; Part 1: Requirements for Generic UWB applications.

[b-ETSI EN 300 440]	ETSI EN 300 440 (2017-03), Short Range Devices to be used in the 1GHz to 40 GHz frequency range

[b-ETSI EN 303 883]	ETSI EN 303 883 (2016-09), Short Range Devices using Ultra Wide Band (UWB); Measurement Techniques