|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  |  | SG11-TD277/GEN |
| --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 11 |
|  |  |  |  | Original: English |
| Question(s): |  | 13/11 |  | Geneva, 19-28 February 2025 |
| TD |  |  |  |  |
| Source: |  | Editor |  |  |
| Title: |  | Output - Baseline text of draft new Technical Report ITU-T QSTR.SRv6_Conf “Method for Verifying Conformance to SRv6” (Geneva, 19-28 February 2025) |  |  |
| Contact: |  | Krishna Kumar Lahoti CNLABS India | Tel: +91 7207580125 E-mail: krishna@cnlabs.in |  |

| Abstract: | This document is the output of the baseline text of draft new technical report ITU-T QSTR.SRv6_Conf which describes the method to verify conformance to SRv6 standards. It includes the discussion results from the Q13/11 Meeting held on 21 February 2025 for Document Number ITU-T SG11 (Study Period 2025) C127. |
| --- | --- |

The following table presents the discussion results for the input documents:

| Document Number | Source | Title | Meeting results |
| --- | --- | --- | --- |
| ITU-T SG11 (Study Period 2025) Contribution 127 | Ministry of Communications, India | TR.SRv6_Conf Proposal on draft Technical Report - “Method for Verifying Conformance to SRv6” | - Change the template from “Recommendation” to “Technical Report - Expand scope to include more RFCs or provide justification - Make “SR Endpoint Behavior - End.T (Specific IPv6 Table Lookup)” from “Mandatory” to “Optional”. - Add more SR Endpoint Behavior testing requirements - Clause to address that Interoperability is out of scope of this document |

Draft New Technical Report ITU-T QSTR.SRv6_Conf

Method for Verifying Conformance to SRv6

Summary

Segment Routing with IPv6 (SRv6) is a cutting-edge networking technology merging the capabilities of Segment Routing and IPv6. By embedding instructions directly into IPv6 headers, SRv6 simplifies packet forwarding, providing better scalability, flexibility, and network programmability. It facilitates efficient traffic engineering, service chaining, and network slicing and plays a pivotal role in shaping the landscape of next-generation networks.

However, the rapid adoption of SRv6 introduces critical challenges in ensuring conformance, interoperability, and reliability across diverse implementations and deployments. Without standardized testing methodologies, discrepancies in SRv6 implementations can lead to operational inefficiencies, reduced network performance, and barriers to widespread adoption. Addressing this gap is imperative to unlock the full potential of SRv6 and foster trust in its deployment.

This technical report seeks to define standardized test methodologies to ensure the conformance of SRv6 implementations on network devices to the SRv6 standards created by the IETF. The verification of conformance to SRv6 standards is aligned with key IETF RFCs (RFC 8402, RFC 8986, RFC 8754). The technical report will identify key conformance testing requirements, establish rigorous testing methodologies, and outline compliance criteria to ensure consistent and compliant implementations of SRv6 across network devices.

Through detailed verification processes, this technical report will provide network equipment manufacturers, service providers, and regulatory bodies with a structured approach to assess, validate, and certify SRv6 conformance. This effort is pivotal for fostering collaboration within the industry, accelerating the adoption of SRv6, and creating a robust ecosystem of reliable and compliant next-generation networks.

Keywords

Segment Routing, IPv6, SRv6, Conformance

Table of Contents

1.	Scope	4

2.	References	4

3.	Definitions	5

3.1.	Terms defined elsewhere	5

3.2.	Terms defined in this Recommendation	5

4.	Abbreviations and acronyms	5

5.	Conventions	6

6.	Overview	6

7.	Relevance of this Technical Report and Gap Analysis with Existing Standardization Work	7

8.	Research for Verifying Conformance to SRv6	8

8.1.	Identification of Core SRv6 Standards to Verify Conformance	8

8.2.	Identification of Testing Requirements to Verify SRv6 Conformance	9

8.3.	Test Topologies for Verifying Conformance to SRv6	10

8.4.	SRv6 Conformance Testing – Testing Methodology, Test Procedures and Expected Results	12

9.	Relationship with other related standard groups for SRv6	12

10.	Conclusion and Future Work	12

Bibliography	13

Draft New Technical Report ITU-T QSTR.SRv6_Conf

Method for Verifying Conformance to SRv6

# Scope

This technical report aims to research methods for verifying conformance to SRv6 standards.

The scope of this technical report includes the following:

- Identification of core SRv6 standards to verify a device’s conformance to SRv6 technology

- Identification of testing requirements from core SRv6 standards

- The test topologies required for verifying conformance to SRv6

- SRv6 Conformance Testing – Testing Methodology, Test Procedures and Expected Results

- Relationship with other related standard groups for SRv6

Since this technical report aims to propose a method to verify conformance only to SRv6 standards, the verification of a device’s conformance to certain routing protocols (such as OSPF, BGP and IS-IS) that may be used in conjunction with SRv6, remains out of scope of this technical report.

Additionally, while conformance to SRv6 standards can guarantee a device’s adherence to SRv6 technical specifications, it does not guarantee 100% seamless interoperability. Various factors such as variability in implementations due to interpretation, support for optional features & extensions, network conditions & security policies, edge cases & unspecified behaviours, may lead to interoperability issues. As such, this technical report neither aims to cover all interoperability issues nor suggest potential solutions to address them.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Technical Report. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Technical Report are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Technical Report does not give it, as a stand-alone document, the status of a Recommendation.

IETF RFC 8402 – Segment Routing Architecture

IETF RFC 8754 – IPv6 Segment Routing Header (SRH)

IETF RFC 8986 – SRv6 Network Programming

IETF RFC 8665 – OSPFv3 Extensions for SRv6

IETF RFC 8666 – IS-IS Extensions for SRv6

IETF RFC 8667 – BGP Prefix-SID Extensions for SRv6

IETF RFC 9252 – BGP Overlay Services Based on Segment Routing over IPv6 (SRv6)

IETF RFC 9352 – IS-IS Extensions to Support Segment Routing over the IPv6 Data Plane

IETF RFC 9433 – Segment Routing over IPv6 for the Mobile User Plane

IETF RFC 9487 – Export of Segment Routing over IPv6 Information in IP Flow Information Export (IPFIX)

IETF RFC 9602 – Segment Routing over IPv6 (SRv6) Segment Identifiers (SIDs) and the IPv6 Addressing Architecture

IETF RFC 9259 – Operation, Administration & Maintenance (OAM) in SRv6 Networks

# Definitions

## Terms defined elsewhere

This technical report uses the following terms, as defined elsewhere:

    - segment [b-IETF RFC 8402]: An instruction a node executes on the incoming packet (e.g., forward packet according to shortest path to destination, or, forward packet through a specific interface, or, deliver the packet to a given application/service instance).

    - SID [b-IETF RFC 8402]: A segment identifier.

    - SRv6 [b-IETF RFC 8402]: The instantiation of SR on the iPv6 data plane.

    - SRv6 SID [b-IETF RFC 8402]: An iPv6 address explicitly associated with the segment.

    - SRv6 SID function [IETF RFC 8986]: The function part of the SID is an opaque identification of local behavior bound to the SID. It is formally defined in Section 3.1 of this document.

    - SRv6 endpoint behavior [IETF RFC 8986]: A packet processing behavior executed at an SRv6 Segment Endpoint Node. SRv6 Endpoint behaviors related to traffic engineering and overlay use cases.

## Terms defined in this Recommendation

This Recommendation defines the following terms:

None

# Abbreviations and acronyms

This technical report uses the following abbreviations and acronyms:

IPv6	            Internet Protocol version 6

SRv6		Segment Routing Over IPv6

SID		Segment ID

SL		Segment List

SRH		Segment Routing Header

PSID		Path Segment Identifier

IETF		Internet Engineering Task Force

MTU 		Maximum Transmission Unit

RUT 		Router Under Test

TLLA 		Target Link-layer Address

TN 		Test Node

TR 		Test Router

# Conventions

In this Technical Report, potential requirements which are derived from a given use case, are classified as follows:

The keywords "it is of critical value" indicate a possible requirement which would be necessary to be fulfilled (e.g., by an implementation) and enabled to provide the benefits of the use case.

The keywords "it is expected" indicate a possible requirement which would be important but not absolutely necessary to be fulfilled (e.g., by an implementation). Thus, this possible requirement would not need to be enabled to provide complete benefits of the use case.

The keywords "it is of added value" indicate a possible requirement which would be optional to be fulfilled (e.g., by an implementation), without implying any sense of importance regarding its fulfilment. Thus, this possible requirement would not need to be enabled to provide complete benefits of the use case.

# Overview

As outlined in the IETF standards RFC 8402, RFC 8986, and RFC 8754, Segment Routing over IPv6 (SRv6) provides an advanced approach to routing by encoding instructions, known as Segment Identifiers (SIDs), directly into IPv6 packet headers. This architecture enables advanced functionalities, including:

- Simplification of network operations by eliminating complex signaling protocols.

- Efficient traffic engineering to optimize resource utilization and meet specific Quality of Service (QoS) requirements.

- Seamless implementation of service chaining and network slicing, crucial for next-generation network architectures.

- Enhanced scalability and flexibility to meet evolving networking demands.

SRv6’s capabilities make it a foundational technology for next-generation networks, but ensuring conformance across diverse implementations is critical to unlocking its full potential. Inconsistent or non-compliant implementations can lead to interoperability issues, diminished performance, and potential network failures.

The proposed method for verifying SRv6 conformance, focuses on verifying adherence to the core specifications and functional requirements outlined in the relevant RFCs. The framework is designed to:

- Validate the correct encoding and processing of Segment Identifiers (SIDs) within IPv6 headers.

- Ensure compliance with SRv6's forwarding and processing behavior across network devices.

- Establish clear testing criteria and scenarios to evaluate the performance and reliability of SRv6-enabled networks.

- Identify potential interoperability challenges between SRv6 implementations from different vendors.

This technical report aims to develop standardized testing methodologies to ensure that SRv6 implementations conform to relevant IETF standards. The focus is on validating network device behavior to ensure reliable deployment of SRv6, and to foster interoperability within a multi-vendor ecosystem.

# Relevance of this Technical Report and Gap Analysis with Existing Standardization Work

# This technical report aims to create a SRv6 conformance test framework by identifying conformance testing requirements, testing methodologies, testing procedures and expected behaviour under various test conditions. Conformance testing ensures that an implementation correctly follows the SRv6 standards. It ensures protocol compliance, improves network reliability, enhances interoperability by providing the foundation for interoperability testing for multi-vendor deployments, reduces deployment risks & costs, and builds confidence among operators and regulators.

# The following table provides a gap analysis between this proposed technical report (Method for Verifying Conformance to SRv6) and other relevant standards and testing specifications from ITU-T.

| S. No. | SDO | Title | Status | Analysis |
| --- | --- | --- | --- | --- |
| 1 | ITU-T | ITU-T Y.3216 (09/2024): Fixed, mobile and satellite convergence – Distributed core network for IMT-2020 networks and beyond | In Force | Recommendation ITU-T Y.3216 specifies the general considerations, requirements, network function enhancements, procedures and security considerations of distributed core network in fixed, mobile and satellite convergence (FMSC) network, in the context of international mobile telecommunications for 2020 (IMT-2020) and beyond. SRv6 is acknowledges as a relevant technology. Whereas, this technical report aims to create a method for verifying conformance of network devices to SRv6 standards. |
| 2 | ITU-T | ITU-T Q.4141 (12/2023): Requirements and signalling of intelligence control for the border network gateway in computing power networks | In force | This Recommendation aims to study the requirements and signalling of intelligence control for the border network gateway in a computing power network (CPN). SRv6 is acknowledges as a relevant technology. Whereas, this technical report aims to create a method for verifying conformance of network devices to SRv6 standards. |
| 3 | ITU-T | ITU-T Y.3657 (12/2023): Big data driven networking – Requirements and capabilities of network visibility | In Force | Recommendation ITU-T Y.3657 specifies requirements and capabilities of network visibility for big-data-driven networking (bDDN). It focuses on the scenario where network infrastructure layer of bDDN corresponds to Internet protocol (IP) bearer network. Whereas, this technical report aims to create a method for verifying conformance of network devices to SRv6 standards. |
| 4 | ITU-T | ITU-T Q.4070 (02/2023): Test suite for interoperability testing of virtualized broadband network gateways | In force | Recommendation ITU-T Q.4070 specifies the interoperability testing of virtualized broadband network gateway (vBNG), including an overview of the test suite and test cases for interoperability testing of vBNG. Whereas, this technical report aims to create a method for verifying conformance of network devices to SRv6 standards. |
| 5 | ITU-T SG-11 | TR.MPM-SRv6 under Q13/11 | TR | The MPM.SRv6 work item is currently in progress and is currently a TR under Q13/11, It aims to specify performance monitoring parameters and metrics for measuring performance of SRv6 networks such as Traffic Monitoring, Path Monitoring, Statistical Analysis, Log Analysis, etc. Whereas, this technical report aims to create a method for verifying conformance of network devices to SRv6 standards. |

# Research for Verifying Conformance to SRv6

## Identification of Core SRv6 Standards to Verify Conformance

The Internet Engineering Task Force (IETF) is a global standards development organization (SDO) creating internet standards (RFCs) for networking protocols and technologies. For SRv6, the IETF has created several standards such as:

- RFC 8402 – Segment Routing Architecture

- RFC 8754 – IPv6 Segment Routing Header (SRH)

- RFC 8986 – SRv6 Network Programming

- RFC 8665 – OSPFv3 Extensions for SRv6

- RFC 8666 – IS-IS Extensions for SRv6

- RFC 8667 – BGP Prefix-SID Extensions for SRv6

- RFC 9252 – BGP Overlay Services Based on Segment Routing over IPv6 (SRv6)

- RFC 9352 – IS-IS Extensions to Support Segment Routing over the IPv6 Data Plane

- RFC 9433 – Segment Routing over IPv6 for the Mobile User Plane

- RFC 9487 – Export of Segment Routing over IPv6 Information in IP Flow Information Export (IPFIX)

- RFC 9602 – Segment Routing over IPv6 (SRv6) Segment Identifiers (SIDs) and the IPv6 Addressing Architecture

- RFC 9259 – Operation, Administration & Maintenance (OAM) in SRv6 Networks

- More draft standards in progress

While these standards collectively establish a comprehensive framework for implementing and deploying SRv6 technology in modern networks, identifying the core SRv6 standards is essential for the scope of this technical report. These core standards serve as the basis for verifying whether a device conforms to SRv6, helping operators determine if a device has fundamental SRv6 support while ensuring compliance with standards, and enabling regulators to assess adherence to SRv6 specifications for certification and policy enforcement.

After a careful study of the above-mentioned standards, only three standards (RFC 8402, RFC 8754, and RFC 8986) are considered the core SRv6 standards that are crucial in the context of SRv6 conformance and for this technical report. These RFCs define the fundamental architecture, encapsulation, and network programming model required for SRv6 functionality.

The other RFC standards, such as RFC 8665, RFC 8666, RFC 8667, RFC 9252, RFC 9352, and RFC 9433, primarily deal with routing protocol extensions (OSPF, IS-IS, BGP), service overlays, mobile user plane integration, and operational aspects of SRv6 deployment. While these standards are critical for specific use cases, optimizations and interoperability, they are not fundamental requirements for verifying whether a device conforms to the core SRv6 protocol specifications.

  - Identification of Testing Requirements from Core SRv6 standards

The following test requirements are identified across RFC 8402, RFC 8754 and RFC 8986 to verify a device’s conformance with SRv6 technology.

- Verify if SRv6 is Enabled – Mandatory

- Outside Domain Traffic – Mandatory

- Leak prevention– Mandatory

- Processing SRH with flag 0 – Mandatory

- Packet Tagging and Tag Processing – Mandatory

- Segment Order in the Segment List  – Mandatory

- TLV Processing - Optional

- Validation of Pad1 TLV – Optional

- Validation of PadN TLV – Optional

- Processing PadN TLV with Zero and Non-Zero Padding – Optional

- HMAC Verification – Optional

- HMAC Digest Truncation – Optional

- HMAC SHA-256 Implementation Verification – Mandatory

- SR Nodes Behaviour – Mandatory

- Processing Segments Left Value – Mandatory

- Decreasing Hop Limit Value – Mandatory

- Invalid Packet Handling in Segment Routing – Mandatory

- Processing Upper-Layer Header – Mandatory

- Securing the SR Domain – Mandatory

- Processing PMTU in SR Domain – Mandatory

- SR Nodes using Flow Label – Mandatory

- SID Format – Mandatory

- SID Arg Value Unchanged  – Mandatory

- SR Endpoint Behavior - End.X (L3 Cross-Connect) – Optional

- SR Endpoint Behavior - End.T (Specific IPv6 Table Lookup) – Optional

- SR Endpoint Behavior - End.DX6: Decapsulation and IPv6 Cross-Connect – Optional

- SR Endpoint Behavior - End.DX4: Decapsulation and IPv4 Cross-Connect– Optional

- SR Endpoint Behavior - End.DT6: Decapsulation and Specific IPv6 Table Lookup – Optional

- SR Endpoint Behavior - End.DT4: Decapsulation and Specific IPv4 Table Lookup – Optional

- SR Endpoint Behavior - End.DT46: Decapsulation and Specific IP Table Lookup – Optional

- SR Endpoint Behavior - End.DX2: Decapsulation and L2 Cross-Connect– Optional

- SR Endpoint Behavior - End.DX2V: Decapsulation and VLAN L2 Table Lookup– Optional

- SR Endpoint Behavior - End.DT2U: Decapsulation and Unicast MAC L2 Table Lookup– Optional

- SR Endpoint Behavior - End.DT2M: Decapsulation and L2 Table Flooding– Optional

- SR Endpoint Behavior - End.B6.Encaps: Endpoint Bound to an SRv6 Policy with Encapsulation– Optional

- SR Endpoint Behavior - End.B6.Encaps.Red: End.B6.Encaps with Reduced SRH – Optional

- SR Endpoint Behavior - End.BM: Endpoint Bound to an SR-MPLS Policy – Optional

- SR Policy Headend Behavior - H.Encaps: SR Headend with Encapsulation in an SR Policy – Optional

- SR Policy Headend Behavior - H.Encaps.Red: H.Encaps with Reduced Encapsulation – Optional

- SR Policy Headend Behavior - H.Encaps.L2: H.Encaps Applied to Received L2 Frames – Optional

- SR Policy Headend Behavior - H.Encaps.L2.Red: H.Encaps.Red Applied to Received L2 Frames – Optional

- Traffic Counters – Optional

- Flow-Based Hash Computation – Mandatory

## Test Topologies for Verifying Conformance to SRv6

    - Common Test Topology – 1

Figure-1: Common Test Topology-1

In this test topology, the Router Under Test (RUT) is an intermediary node within the Segment Routing Domain (SR-domain).

    - Common Test Topology – 2

Figure-2: Common Test Topology-2

In this test topology, the Router Under Test (RUT) is an ingress/egress node at the edge of the Segment Routing Domain (SR-domain).

## SRv6 Conformance Testing – Testing Methodology, Test Procedures and Expected Results

[TBD]

# Relationship with other related standard groups for SRv6

The IETF is the primary body responsible for establishing the standards for SRv6, including its architecture, implementation, deployment, and Operations, Administration, and Maintenance (OAM). The work presented in this technical report only complements the IETF’s efforts by identifying the testing requirements derived from IETF standards for SRv6. It is important to emphasize that this technical report does not seek to specify any aspects of SRv6 nor does it introduce any new requirements for SRv6 outside of IETF’s work.

# Conclusion and Future Work

By specifying the testing requirements, procedures, and expected results for verifying a device’s conformance to SRv6 standards, this technical report provides manufacturers, operators, regulatory agencies, and standardization bodies with the first step towards SRv6-compliant devices. This work item can be further enhanced in future with detailed test procedures and expected outcomes to provide a technical test specification for verifying SRv6 Conformance of a network device.

# Bibliography

- IETF RFC 8402 – Segment Routing Architecture

- IETF RFC 8754 – IPv6 Segment Routing Header (SRH)

- IETF RFC 8986 – SRv6 Network Programming

- IETF RFC 8665 – OSPFv3 Extensions for SRv6

- IETF RFC 8666 – IS-IS Extensions for SRv6

- IETF RFC 8667 – BGP Prefix-SID Extensions for SRv6

- IETF RFC 9252 – BGP Overlay Services Based on Segment Routing over IPv6 (SRv6)

- IETF RFC 9352 – IS-IS Extensions to Support Segment Routing over the IPv6 Data Plane

- IETF RFC 9433 – Segment Routing over IPv6 for the Mobile User Plane

- IETF RFC 9487 – Export of Segment Routing over IPv6 Information in IP Flow Information Export (IPFIX)

- ITU-T Y.3216 (09/2024): Fixed, mobile and satellite convergence – Distributed core network for IMT-2020 networks and beyond

- ITU-T Q.4141 (12/2023): Requirements and signalling of intelligence control for the border network gateway in computing power networks

- ITU-T Y.3657 (12/2023): Big data driven networking – Requirements and capabilities of network visibility

- ITU-T Q.4070 (02/2023): Test suite for interoperability testing of virtualized broadband network gateways

- ITU-T TR.MPM-SRv6: Method for Performance Monitoring of SRv6

_______________________