|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG12-C0112R1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 12 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 12/12 |  | Geneva, 18-26 January 2023 |  |
| CONTRIBUTION |  |  |  |  |  |
| Source: |  | India |  |  |  |
| Title: |  | Revised baseline text ITU-T E.MVS "Mapping and visualization strategies for the assessment of connectivity and QoS" |  |  |  |
| Contact: |  | Venkata Rama Raju Chelle Telecommunication Engineering Centre Ministry of Communications India |  |  | Tel: +91 9821979899 E-mail: cvramaraju.v@gov.in |
|  |  | Abdul Kayum Telecommunication Engineering Centre, Ministry of Communications India |  |  | Tel: +918874209603 Email: Abdul.kayum@gov.in |
|  |  | Rajith Ali SATCOM industry association of India, India |  |  | Tel: +91 8860903836 Email: rajith.ali@sia-india.com |

| Abstract: | This contribution proposes some modifications to the base text of ITU-T E.MVS based on TD172. |
| --- | --- |

Draft new Recommendation ITU-T E.MVS

Mapping and visualization strategies for the assessment of connectivity and quality of service (QoS) and quality of experience (QoE) aspects

Summary

Mapping the QoS and QoE of the telecommunication services is crucial to the work of regulators, government entities and operators since it provides inputs that facilitate the decision-making process to reduce the digital divide. Therefore, this recommendation provides guidance, especially when evaluating and communicating connectivity status and its relation to QoS and QoE across different topologies [including in-building scenarios] and population densities within the country. Where connectivity should be understood not only as the presence or not of telecommunications services, but as a concept that should be built upon different perspectives through relevant sources of information.

Therefore, this work item defines a high-level framework for mapping and visualization strategies that include guidelines to define first how to identify and measure the level of connectivity and second which tools and functionalities are useful in communicating the status of this connectivity.

Editor's Note:

- The Summary needs to be aligned with the scope and other chapters of the draft recommendation.

- Check the proposal of Tahiry (Madagascar) during the review of the summary and scope: "Therefore, this recommendation provides guidance to evaluate and communicate the connectivity status and its relation to the QoS and QoE taking into consideration technical and non-technical aspects that may impact it ."

Keywords

Connectivity assessment, mapping, visualization, QoS, QoE, GIS, connectivity index.

Introduction

<Optional – This clause should appear only if it contains information different from that in Scope and Summary>

# 1	Scope

This recommendation proposes selected guidelines for connectivity assessment in a country for different use cases through the use of mapping tools and relevant sources of information.

The recommendation outlines a series of toolboxes including relevant parameters and identifies useful guidelines to construct a composite connectivity index. Besides, it describes how connectivity can be visualized including in-building connectivity assessment guidelines.

This recommendation also provides guidance on spatial analysis including stratification strategies to classify geographical area, data granularity, collection and post processing mechanisms and possible mapping and visualizations depending on target audience (end users, regulators, operators, etc).

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title.

[ITU-T E.802 Amd2]   Recommendation ITU-T E.802 Amd2 Framework and methodologies for the determination and application of QoS Parameters.

[ITU-T E.805]	Recommendation ITU-T E.805 (2019), Strategies to establish quality regulatory

frameworks.

[ITU-T E.805.1]	Recommendation ITU-T E.805.1 (2021), Quality of service operational strategy for improved regulatory supervision of providers of mobile telecommunication services.

[ITU-T E.806]	Recommendation ITU-T E.806 (2019), Measurement campaigns, monitoring systems and sampling methodologies to monitor the quality of service in mobile networks.

[ITU-T E.812]	Recommendation ITU-T E.812 (2020), Crowdsourcing approach for the assessment of end-to-end quality of service in fixed and mobile broadband networks.

[ITU-T E.840]	Recommendation ITU-T E.840 (2018), Statistical framework for end-to-end network performance benchmark scoring and ranking.

# 3	Definitions

<Check in the ITU-T terms and definitions database at www.itu.int/go/terminology-database whether the term has already been defined in another Recommendation. It would be more consistent to refer to such a definition rather than to redefine the term>

## 3.1	Terms defined elsewhere

<Normally, terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document. Before defining a new term, verify whether it has already been defined in the official ITU terminology database, at www.itu.int/go/terms. >

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>.

3.1.2	<Term 2> [Reference]: <optional quoted definition>.

## 3.2	Terms defined in this Recommendation

Editor’s Note: contributions are invited with a view to improve definitions in this section, if needed.

This Recommendation defines the following terms:

3.2.1	Mapping: The process of illustrating values or parameters in a geographic information system (GIS).

3.2.2	Connectivity: is the overall degree of access to telecommunications services of a network with regard to performance and usability.

[alternate: The ability to link, communicate and control the flow of information in a telecommunication network to have access to quality, reliable, and affordable services

Editor's Note: proposals for Connectivity Definition

- Wolfgang Proposal: Definition from Global connectivity 2022

- Proposal from James: A4AI originally developed the concept, it's unpacked in more detail here: https://a4ai.org/meaningful-connectivity-a-new-standard-to-measure-internet-access/

- Al Proposal: check the following IETF reference, RFC 2498. This is a very exact set of definitions at the IP-layer.

- Stavroula proposal: ￼https://www.berec.europa.eu/en/document-categories/berec/regulatory-best-practices/guidelines/handbook-of-berec-guidelines-on-geographical-surveys-of-network-deployments

Editor’s Note: group to work on a connectivity definition, taking into account description provided in clause 6.3

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

<abbr>	<expansion>

GIS		Geographic information system

<Include all abbreviations and acronyms used in this Recommendation>

# 5	Conventions

<Mandatory clause. Describe any particular notation, style, presentation, etc. used within the Recommendation, if any. If none, write "None.">

# 6	Clause 6 of the Recommendation

<Clause 6 onwards contains the body of the Recommendation>

## 6.1	Connectivity Assessment

Editor’s note: Contributions are invited; for editors to see how to restructure clause number 6.

Connectivity Assessment is primarily essential to understand the optimal connectivity needs, opportunities and challenges of the target community in a particular location or a specific geographical area based on demographic analysis. Therefore, a more holistic approach to connectivity assessment helps to gather information on current and future needs, perform gap and risk analysis, boost investment and ensure productivity and growth to the targeted community or area.

Therefore, the practical way to make a workable connectivity assessment is to collect, collate and apply a flexible and reliable data/information pertaining to digital ecosystem from relevant and multiple sources. The elements of such data should, inter alia, be on telecommunications infrastructure, topography or terrain characteristics, socioeconomic conditions, regulatory and administrative constraints, required QoS etc. Such a meaningful exercise towards connectivity assessment will invariably help to prioritize the infrastructure deployment or target public policies that can help close the digital divide.

Editor’s Note: include new subclause in the document on connectivity assessment use cases

6.1.1 Connectivity assessment use cases

Editor’s note: Contributions are invited

-To accelerate infrastructure deployment

-To resolve any QoS and QoEissues

-To provide informed choices to end users in terms of services

-To have updated information related to connectivity

-To improve the regulatory frameworks in terms of prices, subsidies (universal fund), competition.

### 6.1.2	Sources of information

Editor’s note: Contributions are invited

The main focus should be on gathering and analyzing accurate data from a variety of relevant sources to evaluate the connectivity needs, make informed decisions and ensure desired QoS. Therefore, the methods of collection of desired information should ensure data accuracy and quality to find workable answers to the problems and evaluate the outcomes.

:

Accordingly, the process of gathering and analyzing the data should consider following elements while considering the sources of information.

- Data should be related directly or indirectly to the telecommunication services

A comprehensive and multi-dimensional approach may require to gather data that targets multiple and relevant sources. The sources and elements of information to undertake such connectivity assessment may be directly or indirectly concerned with the telecommunication services. It may be pertinent to mention that The Global Connectivity Report 2022, (xxx) identifies five enablers of connectivity or the meaningful dimensions of connectivity as infrastructure, device, affordability, skills, and safety and security. Some of the enablers may apparently appear as not directly linked to  telecommunication variables but their impact to assess the  connectivity needs can never be ignored.

- Data should be representative of the investigated geographical area

Editor’s note: This section should include guidance on sample representation, trade-offs, parameters to be considered when evaluating a geographic area

In order to have  information on the connectivity status that will be comparable across the studied geographical areas, it is important that all sources of information have the same representation in the whole area. Representativeness is not about the number of data from each source of information but the presence of each source of information in the same geographical area, for example, if the connectivity assessment is going to be performed at a municipality level, then all sources of information should have that level of granularity. The geographical level to be studied could depend on the  objective of the analysis and who is carrying it out (regulators, government entities, operators, others); hence geographical areas could vary such a little town, a suburban area, a region or a country. It should also be taken into account that there are statistical techniques to account for under or overrepresentation of the samples available, for example, data imputation techniques can be used to increase the number of samples in some areas or sampling techniques can also be applied to avoid overrepresenting certain areas.

- Data should be obtained from official/collaborative initiatives.

The composite index to reflect the connectivity will be as accurate as sources of information from which it was obtained. Official or collaborative initiatives said to have high accuracy and wider acceptability. Access to official information such as of central government departments or public bodies can improve confidence of the result obtained due to data source authenticity. Same way, source of information from collaborative initiatives, such as crowdsourcing data or data from any type of surveys, will have a big impact in the result obtained thanks to the number of information gathered but also due to the quality of information collected.

- Data should be obtained periodically

Data analysis to assess connectivity is not a simple task and shouldn’t be performed only once. It needs to be performed periodically. Therefore, data should be followed weekly, monthly or whatever the best frequency is for the intended target. Furthermore, data obtained periodically can be used to build a historical raw database that will allow to look at the evolution of the levels of connectivity through this composite index.

- Data sources should not be redundant

As data is the basis to understand the status of  connectivity , it must be carefully chosen to avoid situations where the same piece of data exists in multiple places. Data redundancy can cause inconsistency, which can provide unreliable and/or meaningless information. Besides, data redundancy may increase the size and complexity of a database making it more of a challenge to maintain. However, when data from two different sources are complimentary to each other, redundancy can be useful. The last case refers, for example, when having two datasets of KPI that contain different samples, then using both datasets will increase the number of samples available for consideration.

6.1.3 Data Tools

Assessment and evaluation of the connectivity status depend on different informational resources or data tools and targets. For example, required targets can range from new market entry, filling coverage holes, and densification related matters. Such informational resources can dimension and scale telecommunication solutions if it is according to a set target.

For purposes of this recommendation, data from different sources of information can be viewed as toolboxes.

KPIs obtained from data sources of information could be associated to each toolbox. The following toolboxes are suggested to begin the analysis, however the use of any or various of the toolboxes may differ according to the use case that is being investigated.

Editor’s Note: Link toolboxes with use cases to add context. To be completed when the use cases subclause is developed.  Suggest inserting (later, after stabilization of content) a graphical overview showing the 5 toolboxes and their principal content.

Performance Toolbox

The Performance toolbox includes KPIs that measure the level of QoS and QoE achieved or delivered to the customer. These parameters are measurable (with instruments, surveys or observations) and allow to know the ability of a whole or portion of the network to provide the functions related to communications between users.

Editor’s Note:

- there is a need to discuss on the comment of Janusz: “How should the QoE assessment be included within the framework if QoE KPIs are not present in the list? For that reason, I added a few KPIs. However, the challenge is that often we have lot of representative QoS metrics but QoE data volume is not sufficient, or not up to the same level as QoS”.

- While reviewing this section, check if we are not in contradiction of the ITU-T E.800.

KPIs that could be considered in this toolbox but not limited to the followings:

- Download/Upload Throughput, Packet Loss, Jitter and Latency

- Unsuccessful Call Ratio, Dropped Call Ratio, Call set up time

- Voice MOS, Video MOS

Editor's Note: Regarding the QoE aspect, it is suggested to put in place a survey based on the level of satisfaction of the end users for any particular telecommunication services.

Telecom Market Trends Toolbox:

Market Trend Analysis is an effective tool for evolving market such as telecom industry to stay and grow.  Since market trends are dynamic, such analysis enables companies to identify, forecast and strategize for current and upcoming markets including market maturity and future market potential  .

The KPIs relevant to this toolbox are:

- Mobile Users Penetration (MUP) (telephony, Internet access) per neighborhood polygon

- Fixed Users Penetration (FUP) (telephony, Internet access) per neighborhood polygon

- Presence of any kind of service provider such as mobile operators, fixed operators, mobile virtual operators, wireless Internet Service Provider

- Fixed broadband basket

- Mobile cellular low-usage basket

- Mobile broadband Internet prices per month

- Satellite broadband internet prices per month (if any)

- Consumer vs Geographical market trends

- Opportunities and fluctuations in the market and their impact

- Churn and revenue loss (in terms of previous view of density population / capacity issues).

Editor’s Note: check if there is a need to expand the point (There is a loss of revenue)

Geographical Network Coverage Toolbox

This toolbox gathers all those metrics that help to identify those geographic areas where users can connect to a network, stay connected and reliably communicate with optimal network conditions and speech quality. Geographical coverage usually represents a single layer such as roads, lakes, inhabited areas, and land use, etc., as well as an integration of these layers (see ITU-T Y.4562 ).

The KPIs relevant to this toolbox are:

- Signal strengths: RxLev (received signal level) for GSM, RSCP (received signal code power) for UMTS and RSRP (reference signal receive power) for LTE, SSRSRP for NR.

- Availability of line-of-sight (LoS)

- Availability of wired connections (optical fiber, DSL, coaxial) for internet:

Editor’s note: proposal to add this under the availability or under the infrastructure toolbox: “Household accessibility, measuring the number of households passed by an infrastructure that enables a cable modem Internet connection”

The satellite KPIs should also be considered as there are satellite service providers.

Editor's Note: to add the toolbox for wired networks (Fiber optic, ..) also. Here we added only the wireless.

Telecommunication Infrastructure Toolbox:

The development of adequate telecom infrastructure (masts and towers, Optical Fibre Cables and other media, bandwidth and backhauls etc) is an important phase in the life cycle of the telecommunication industry to bring out enable quality services to any corner of a country.

The KPIs relevant to this toolbox are:

Editor's Note: Editors will review the below points, and if possible, provide clear definition. .

- Mobile access technologies availability (2G, 3G, LTE, 5G)|(with the corresponding type of power supply deployed  )

- Fixed access technologies availability (optical fiber, DSL, coaxial)

- Availability of backhauling

- Aerial accessibility via existing light poles and transmission tower infrastructure

- Points of interconnection (border gateway HUBs, Data centers, IXP)

- Availability of electricity

- Ease of right of way for laying cable or installing pole/tower

- Ease of right of use of existing street furniture

- Security aspects to protect telecom infrastructure

Toolbox Consumer Behavioral KPI

This toolbox reflects  consumer behaviors that are helpful to study consumers and the processes they use to choose, use (consume), and dispose of telecommunications products and services. Studying consumer behavior is important because it helps to understand what influences consumers’ buying decisions, how consumers decide on a product to fill the digital divide in the market to identify the products that are needed and the products that are obsolete.

The KPIs relevant to this toolbox are:

- Smartphones Penetration

- Computers Penetration by household

- Online shopping Penetration

- Digital financial services usage

- Traffic and Hours spent surfing Internet

- Internet access penetration (fixed and mobile)

- Sensitivity to tariff

- Sensitivity to the QoS (based on complaints/periodic feedbacks)

- Trendspotting across industries and geographies

- Consumer value perception to offset growing competition

Socio-economic factors  Toolbox

Socio-economic toolbox refers to classifying people by their social, economic and educational status. Examining demographics not only provide an understanding of the composition of the market area, but they also help to understand the potential range of impacts of a proposed change, and the likely responses of those impacted if the change occurs.

The KPIs relevant to this toolbox are:

- Demographic parameters: For example, age, education level, income level, number of members by household, genre, occupation, urban or rural population.

- Business parameters: Demographic data about enterprises, number of enterprises, size of enterprise measured by the number of employees, earnings, sector, among others.

- Socio-Economic Levels: Data that allows grouping and classifying households based on availability basic infrastructure (power supply grid, lighting, water, paving streets), health infrastructure, connectivity, and entertainment, among others, the level of details available depends on each country.

GIS Data Toolbox

A geographic information system (GIS) Data facilitates to understand patterns, relationships, and geographic context by visualizing in map forms and remains a powerful decision-making tool that enables effective environmental, demographic, and topographic analysis. Therefore, GIS data-based telecom network analysis facilitates efficient decision-making like where network components should be placed and how they should be connected, what size components to use and how to create a strategy on meeting capacity requirements and maintaining network reliability, working condition of a particular network, optimization of network performance, maximising the optimal use of existing infrastructure and minimizing the cost of deployment.  Thus, GIS data helps to enhance telecom processes through better customer-relationship management and location services. Accordingly, the GIS data toolbox will include KPIs related to demographic and topographic information.

The KPIs relevant to this toolbox are:

- Terrain layers / topographic Layer

- Building layers (2D and 3D)

- Regulatory aspects (Restriction on tower heights near airports, signal level at international borders, near schools/hospital/Prisons/ defense establishment, RoW etc.)

- Protected area layers (National Parks, Conservation Reserves, Community Reserves, Sanctuaries, Biosphere, Monuments).

- Site selection and tower placement mapping criteria

### 6.2	Connectivity indicators

Editor’s note: Contributions are invited

- OECD has a guideline for the construction of composite indicators. Perhaps it could help in the discussions: https://www.oecd.org/els/soc/handbookonconstructingcompositeindicatorsmethodologyanduserguide.htm

The purpose of creating connectivity indicators is to provide actionable insights to improve the status of connectivity depending on the use cases. Then, connectivity indicators can be compared geographically, based on the stratification of the studied areas.

A composite index can be obtained through a combination of connectivity indicators using statistical methodologies. This index represents an even broader view of the connectivity status. It is beneficial when a single dimensionless value is required to assess the connectivity status in simple way, so that decision makers can have a complete picture in mind. Then, concrete, targeted action plans can be derived to improve the connectivity based on the connectivity indicators that assess the dimensions of interest.

### 6.3 Connectivity index

Even if there is no universal method for constructing composite indices, their construction is mainly determined by the application or question that is going to be investigated.

The following guidelines are recommended for the construction of a connectivity index that uses a set of variables (connectivity indicators) that can be obtained from the toolboxes previously described:

- KPIs from multiple toolboxes should be selected according to the considerations defined by the objective of the analysis. The selection step is the result of a trade-off between possible redundancies caused by overlapping information and the risk of losing information.

- Normalization can be required prior to any data aggregation as the KPI in a data set often have different measurement units. Therefore, it is necessary to bring the KPI to the same standard, by transforming them into pure, dimensionless, numbers.

- The composite index is constructed through the  combination of all the different KPI  from the chosen Toolboxes. For this combination, different aggregation methods are possible. The choice of the ‘best’ aggregation method also depends on the aim and the users of the analysis (regulators, operators, or the end users).

- The methodology to obtain the index should be transparent and documented to allow for validation/verification of the results.

Editor's Note:

- Wolfgang will contribute some suggestions on explanation "graphic.

- See how to address Wolfgang Comment: " I see the changes, but I still think it is unclear: a) Aggregation can be done in the native value space of a KPI, so nromalization could be done on aggregated values of the same type also. b) Normalization is not all which is usually done. Typically, one would combine some kind of nonlinear transformation (e.g., using a sigmoid curve on the input quantity, expressing "totally unacceptable" values, a range where scores are prportional, and a "saturation" region. Normalization is then integrated in the computation process, i.e., all output values of such transformations are on the same value range".

5. The use of multivariate methods is suggested for index construction. Multivariate methods allow for a deeper exploration into possible patterns that exist in the data, enable complex inter-relationships between many variables to be represented graphically, and provide ways of reducing the dimensionality of the data for summary and further analysis.

6. Among the multivariate methods available, cluster analysis can be used to identify natural groupings amongst the sampling units so that units within each group (cluster) are similar to one another while dissimilar units are in different groups. Furthermore, when dealing with large datasets (for example, crowdsourcing information), principal component analysis (PCA) can be useful to reduce the dimensionality of the data while considering as much as possible of the variation of the original data. In this way, a smaller number of variables can summarize the data by a linear combination of the original variables.

7. The question of the choice of a system of weights in order to weigh the individual indicators, according to their different importance in expressing the considered phenomenon, necessarily involves the introduction of an arbitrary component. For example, subjective weights can be set by a group of specialists (e.g., regulators, operators, policy makers) or maybe social surveys about how important individual indicators are to the people. A suitable statistical methodology that assigns a weight depending on the variation of the data can also be used (for example, PCA).

### 6.4	Data granularity, periodicity and collection mechanisms

Editor’s note: Contributions are invited

6.4. 1 Granularity

The granularity level is relevant to define a criterion to segment geographical areas to identify different groups of users. Considering that it is common that users have different preferences or interests according to the geographical area they lived.

A careful examination of the data structure to identify different sectors or sub-strata should be undertaken, for example, urban and rural data cannot be combined due to inherent heterogeneity  . As another, a division of 10x10 meters grid could be suggested to divide each country. Then, a value of connectivity could be associated to each square. While, this level of detail may be sufficient for urban areas, whereas for a rural area, a 1x1Km may be more appropriate due to lower KPI density, KPI availability and typically less infrastructure buildup covering larger areas.

6.4.2 Periodicity

Editor's Note: comments: “Importance of having higher samples for daily collection allows regulators to visualize capacity. Some indicators such as throughput, numbers of test, peak capacities. For FTTH coverage can be accomplished by looking at longer time periods and hence sample accumulation”

6.4.3 Collection mechanisms

Editor’s note: This section should highlight the basis of criteria which allow comparability among different geographical areas.

### 6.5	In-building assessment of connectivity

Since typical users spent most of their time indoors it is important to be able to understand the QoE in these environments. Within the buildings, QoE measurements should take into account the effect of built structures, often the best way of achieving this will be via the direct measurement of KPIs from devices involving users in defining and achieving the optimum outcomes for themselves. With more detailed and accurate information about building and clutter environment, and that too in a digital format that is interoperable, would be of great help for tools predicting quality of services inside the building.

It shall be possible to mark the availability of the connectivity within a building, at each floor and at each room through a suitable labelling.

Editor's Note: did we address the two comments from James (That’s certainly the ideal! This level of detail (i.e. connectivity by room) may be aspirational but is very hard to do at scale and perhaps this should be flagged) and Shagufta Parveen (Maybe clarify what the traditional methodologies and more specific interventions are here — or perhaps give some examples. I’m unclear on what is being suggested atm.)?

Connectivity quality assessment may start from a particular spot for a variety of services to be measured at that spot. Assessment of quality in an apartment may be composed of a number of such spots. Further assessment at building level may or may not be aggregation at floor level depending upon closeness of the variations across apartments on the same floor or across floor on the same side of the tower.

The indoor sample validation is essential primary process to assess in-building connectivity, hence, evaluation of parametric conditions using a validation methodologies provide a high probability differentiation between indoor and outdoor samples. Moreover, combination of indoor sample indicators enhances confidence level of indoor identification and provides beneficial use cases.

The following methodologies are recommended:

1. Indoor / Outdoor sample velocity comparison, a threshold for indoor sample velocity < 5 km/hr can be set as a primary filter before further validation takes place. for example, it is impossible to have a proven high velocity sample within an indoor building layer.

2. Validating and filtering out indoor samples can be achieved if coordinates are compared to the geospatial building polygon. Such process can only be efficient if systematic approach is adopted for neighbourhoods polygons and not per building basis.

3. Detection of rapid change for received power (SS RSRP, RSRP, RSCP, Rxlvl) in comparison with building layer polygons. Often multiple detected second window samples are useful for indoor sample validation as degraded power values are obvious.

4. In the crowdsourcing context, there are modules and sensors available in the mobile devices which are widely penetrated across many device models and can be used to determine indoor/outdoor probability such as wi-fi signals, cellular signals, GPS signals, light sensors, barometric pressure, battery status (e.g. plugged) compass, time stamp, etc.

5. Z values is an indicator by itself as collected samples with X, Y, Z coordinates indicates human activity at an elevation referenced from ground level.

6. Hence, In-building connectivity assessment per level is possible if elevation values (Z values) is collected, processed and considered as part of sample coordinates X, Y, Z. The confidence level of valid Z value can be impacted by rough terrain and sample size. As indoor and outdoor samples (X,Y,Z) coordinates are accumulated within dense population, the following use cases can be detected:

- 3D building dimensional modelling which also serves as an indoor/outdoor validation source.

- Spherical antenna beam dimensioning and indoor coverage penetration visuality.

- In-building service optimization and quality assessment per level visuality.

## 6.6	Requirements and characteristics of Mapping and Visualization tools

Editor’s note: Contributions are invited

Assessing the level of connectivity of any particular geography can take many forms, ranging from literal mapping using geographic information system tools to data collection and analysis or polling that helps a region better understand its needs and opportunities. In general, visualization tools are needed to communicate information about the status of connectivity for different purposes and from different perspectives.

Visualization is a summary term for presenting data in visual form, such as bar graphs, pie charts, time-axis diagrams etc.

A special case of visualization is termed "mapping" or geo-visualization, meaning the visualization of data onto geographical maps.

In order to support this, data needs to have respective geographical information with it. For instance, demographic information may be related to postcode areas; in that case geo visualization could display this information with respective polygons on a map; in this case data is related to a given area. It is also possible that primary data is linked to a specific point (given by e.g. GPS co-ordinate; for example locations where actual measurements showed a call drop). In that case, visualization can either be direct (drawing respective points on the map) or data can be aggregated. In that case, the unit of aggregation would be a polygon (a tile, a postcode areas, ...) and the quantity to be visualized would be e.g. a color-coded representation of an indicator, in the example case of a call-drop rate.

    - Mapping

  - Improving knowledge of the different services availability in a defined geography.

  - Providing information to attract investment or generating support initiatives.

  - Exposing common misperceptions.

  - Reducing duplication of efforts by identifying gaps and potential ways to fill them

  - Helping to prioritize and allocate resources

  - Serving as the basis for tracking and demonstrating impact over time

  - Understanding where there are systemic inequities and opportunities.

Editor's Note:

- This section needs contributions

- There is need to address Wolfgang comment: "With the context and definitions given, "mapping" is a special case of "visualization" where information is shown in geographic or other maps (e.g. floor plans of buildings). This section looks very much like a construction site anyway; I promise to try to contribute some actual text here'.

    - Visualization

Visuality for regulators should consider high sample location accuracy in reference to topography and time dimensions.

Editor’s note:  *Types of maps (open signal presentation)

## 6.7	Strategies for communicating the status of connectivity

Editor’s note: Contributions are invited

6.7.1	   From Regulators’ view

Connectivity assessment Interface for regulators should be able to accommodate performance and coverage KPI derived from different database sources, such as crowdsourcing, drive or walk testing and operator network elements (OSS data) databases, operator’s official declaration.   Such informational data can provide composite indicators such as optimization priority, service availability, ranking and partial geo-marketing guidance. Furthermore, Socio-economic, GIS, and infrastructure data such as all base station parameters.

6.7.2	   From Final Users’ view

It is critical to develop and enhance end user applications as it generates raw data constantly and reflects the quality of experience directly from users. It is considered as the main stream for data generation, hence features such as the following is recommended to have for end users:

- General Performance KPI such as Throughput and Latency

- Video Streaming Analytics: Load time (ms), Maximum Resolution (p) and Buffering %

- Service coverage with auto scale heat Map per technology

- Service availability

Connectivity assessment from final user perspective should put more emphasis on the actual QoS / QoE of the given user’s internet connection and less on the aggregate information across many users for a larger geographical area (e.g. suburb, city, region). The reasoning behind is that individual assessment will be more relevant, accurate and actionable. Individual connectivity assessment for the particular user would be more valuable with more detailed breakdown of factors impacting the QoE. Identifying and presenting bottlenecks could suggest factors which can be influenced by the user or not:

    - Poor performing device models

    - Local wi-fi issues

    - In-building wiring or signal penetration issues

    - ISP last-mile issues

    - Poor interconnections between ISPs and content/application providers

Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

<Appendix Title>

(This appendix does not form an integral part of this Recommendation.)

<Body of appendix I>

Bibliography

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title.

____________________