|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG2-C046 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 2 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 1/2 |  | Geneva, 16-20 May 2022 |  |
| CONTRIBUTION |  |  |  |  |  |
| Source: |  | Ministry of Communications, India |  |  |  |
| Title: |  | Proposed revised baseline text for TR.MMWF: "Methodologies to mitigate Wangiri Fraud" |  |  |  |
| Contact: |  | Piyush Chetiya Telecommunications Engineering Centre, Ministry of Communications, India |  |  | Tel: +919435000033 E-mail: piyush.chetiya@gov.in |

| Abstract: | This proposal proposes modifications to the baseline text of SG2-TD 1441 titled Draft Text of Technical Report: “Methodologies to mitigate Wangiri Fraud” |
| --- | --- |

- Introduction

Baseline text of TD no SG2-TD1441 titled Draft Text of Technical Report: “Methodologies to mitigate Wangiri Fraud” was discussed in the ITU-T SG2 virtual meeting from 31 May to 11 June 2021 as given in SG2 TD 1441. The baseline text is as per SG2-TD1441.

Wangiri is a common fraud activity targeting subscribers as well as telecom operators, in this type of fraud, fraudsters call but hang up after the first ring hence the name Wangiri, which is Japanese for “one (ring) and cut.” When victims call back, they don’t realise that it’s a premium rated number, and they’re hit with very high charges. This problem is growing rapidly in different countries but in general cases these premium numbers seems to be from developing countries.

A large volume of calls are delivered to the target networks with no apparent intention of establishing a conversational call setup (zero duration calls).  Also no CDRs are generated because the calls are not answered (missed calls only).

- Proposal:

This contribution proposes modifications/ updates to the baseline text of SG2-TD1441 at Annexure-I

Annexure-I

|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  | SG2-TD1441 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 2 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 1/2 |  | Virtual, 31 May – 11 June 2021 |  |
| TD |  |  |  |  |  |
| Source: |  | Editor, TR.MMWF |  |  |  |
| Title: |  | Draft Text of Technical Report: “Methodologies to mitigate Wangiri Fraud” |  |  |  |
| Purpose: |  | Proposal |  |  |  |
| Contact: |  | Ahmed Tajelsir Atya MOHAMMED Telecommunications and Post Regulatory Authority Sudan |  |  | Tel: +249187171277 Fax: +249183484486 E-mail: ahmed.atyya@tpra.gov.sd |
| Keywords: |  | Wangiri; premium rated numbers ; |  |  |  |
| Abstract: |  | This TD is containing a draft text of the new work item of Technical Report “Methodologies to mitigate Wangiri Fraud” which is trying to describe the problem of Wangiri fraud and suggest some methodologies to prevent subscribers. |  |  |  |

Summary:

The new work item has been approved during Q ½ meeting to draft a Technical Report: “Methodologies to mitigate Wangiri Fraud”. The baseline text is found below in Annex 1 and A.13 justification form is in Annex 2.

|  |  | International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | Technical Report |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (dd MMM YYYY) |
|  | Methodologies to mitigate Wangiri Fraud |  |  |  |

Summary

This technical report aims to define Wangiri fraud in telecommunications and to provide the most effective methodologies to mitigate this type of fraud

Keywords

[Wangiri; Premium rated numbers; Fraud]

Change Log

This document contains Version 1 of the ITU-T Technical Report on “Methodologies to mitigate Wangiri Fraud” approved at the ITU-T Study Group 02 meeting held virtually, 31 May -11 June 2021

| Editor: | Name Company Country | Tel: Fax: Email: |
| --- | --- | --- |

CONTENTS

Page

1 Scope 6 2 References 6 3 Terms and definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined here 6 4 Abbreviations 6 5 Specific contents: could be introduction or something more substantive Error! Bookmark not defined. Annex A Example of annex Error! Bookmark not defined. Appendix I Example of appendix Error! Bookmark not defined.

List of Tables

Page

Table 1: Example of a table 7

List of Figures

Page

Figure 1: Example of a figure 8

Technical Report ITU-T Methodologies to mitigate Wangiri Fraud

Technical Report ITU-T
Methodologies to mitigate Wangiri Fraud

- Summary

This technical report aims to define Wangiri fraud in telecommunications and to provide the most effective methodologies to mitigate this type of fraud.

- Introduction

Wangiri is a common fraud activity targeting subscribers as well as telecom operators, in this type of fraud, fraudsters call but hang up after the first ring hence the name Wangiri, which is Japanese for “one (ring) and cut.” When victims call back, they don’t realise that it’s a premium rated number, and they’re hit with very high charges. This problem is growing rapidly in different countries but in general cases these premium numbers seems to be from developing countries.

A large volume of calls are delivered to the target networks with no apparent intention of establishing a conversational call setup (zero duration calls).  Also no CDRs are generated because the calls are not answered (missed calls only).

- Scope

This technical report provides a definition of Wangiri fraud and the most effective methodologies to mitigate the risk of this type of telecommunications fraud. Moreover this technical report will describe the role of member state to fight this fraud globally.

- References

- Add references here.

- Terms and definitions

## Terms defined elsewhere

This Technical Report uses the following terms defined elsewhere:

    - term [reference]: definition

    - [reference]: definition

## Terms defined here

This Technical Report defines the following terms:

5.2.1. term [reference]: definition

5.2.2.  term [reference]: definition

- Abbreviations

| ABC | spell it out |
| --- | --- |

- Wangiri Definition and description of the fraud

'Wangiri' is a Japanese word meaning 'one ring and drop'. And a Wangiri call does just that. Calls usually cut off just as the phone rings, leaving a missed call message from an international, or unusual number.  In Wangiri fraud the scammers generate single beep phone calls to many phone users using an autodialer. When the user calls back out of curiosity on receiving  a missed call , the scammers try to keep the victims busy on the line for as long as possible. The calls are directed to either a real person or an Interactive Voice Response(IVR) system to extract as much money as possible since the call is routed to a premium-rate number.

- Proposed Methodologies

All member states are invited to consider the following measures to mitigate the risk of Wangiri fraud:

  - Routing Management of Carrier: When a fraudster carries out the Wangiri fraud and gives missed calls to multiple subscribers, the augmented amount of traffic can be perceived by the carrier who routes these calls. If an operator examines this activity, there will be a rhythmic trend of increased traffic observed on the same carrier to route these calls. In such cases, an operator must take necessary protections to route all the traffic through an alternative carrier. Routing the calls through a different carrier will help in breaking the chain between the fraudster and the linked carrier. An appropriate regulations of international traffic by member states might help in relieving the problem to a greater extent.

  - Customer Experience Management: As the customer is the main target of any business, telecom operators need to manage their customer complaints efficiently, which will reduce customer churn and also loss of revenue. All employees in telecom operator should be informed about Wangiri and how to manage complaints related to this fraud.

  - Subscriber Awareness: A pro-active approach to minimize Wangiri fraud would involve making the consumers aware of the risk. Customers may be alerted through Information, Education and Communication (IEC) campaign by the operators about he risk involved in calling back missed calls received from International Numbers.There are several free apps which have active community platforms where users continuously report spam numbers. These apps can be used to quickly check suspicious numbers. This would tell the customer if the number is a part of any ongoing scams or not and will help them in blocking the spam numbers used for Wangiri calls.

  - Interactive Voice Response (IVR) Facility: Whenever a subscriber calls back to a subspecies destination upon receiving a missed call, the telecom operator should have an IVR voice informing him about his called destination. This IVR warning would help the subscriber to decide if they really want to make the call or not through the IVR announcement that the number you are calling is a premium rate number and involves high calling charges.

  - Using Technology: Telecom Operators might have an Artificial Intelligence system to make alerts to the customers via different means when receive missed calls from subspecies numbers.

  - Using Blockchain: Telecom consortium can use blockchain technology to share intelligence in near to real-time about the actual Wangiri calls that have already occurred. This  data can help the Telecom operators to improve the algorithms and reference data used for the proactive management of future calls, increasing the likelihood of correctly identifying a Wangiri call before the intended victim suffers harm.

  - Numbers Blocking: Regulators in different countries might allow telecom operators to block all numbers that were repeated in Wangiri fraud. This data can be available in real time if the blockchain technology at 8.6 is used.

  - Information, Education and Communication(IEC) campaign by the regulators is very much essential for dissemination of information about the charging plan of Premium Rate numbers because a large segment of  customers are not aware about the high rates of such premium rate numbers.

  - Sharing of Black Lists: Regulators in different countries might set up a mechanism to share the most frequent numbers committing Wangiri fraud.

  - The proper National Numbering Plans management: Regulators might prepare a mechanism to ensure the proper use of NDCs with MNOs in order to avoid different types of fraud including Wangiri, call masking and spoofing.

  - The Application of WTSA Res. 91: All member states are invited by this Resolution to “pursuant to the relevant ITU T Recommendations, to make available information on the presentation of their national numbering plans and amendments thereto in a timely manner, so as to ensure that the electronic repository remains up to date.”

Table 1: Example of a table

| Number | Observation |
| --- | --- |
| 1 | Sample table |
| 2 | Table header (1st row) uses style Table_head |
| 3 | Other table rows use style Table_text |
| 4 | Heading with table number uses style "Table_No & title" |

| A |  | B |
| --- | --- | --- |

Captions for figures use WinWord style "Figure_No & title"

Figure 1: Example of a figure

[ADD TEXT, IF ANY].

Annex 2

A.13 justification for new TR.MMWF "Methodologies to Mitigate Wangiri Fraud"

| Question: | Q 1/2 | Proposed new ITU-T Technical report | Virtual 31 May- 11 June 2021 |  |
| --- | --- | --- | --- | --- |
| Reference and title: | TR.MMWF "Methodologies to Mitigate Wangiri Fraud" |  |  |  |
| Base text: | T17-SG02-C-0326 |  | Target date: | 2022-12 |
| Editor(s): | Ahmed Tajelsir Atya Mohammed TPRA Sudan ahmed.atyya@tpra.gov.sd |  | Approval process: | Agreement |
| Purpose and scope This technical report provides a definition of Wangiri fraud and the most effective methodologies to mitigate the risk of this type of telecommunications fraud. Moreover this technical report will describe the role of member state to fight this fraud globally. |  |  |  |  |
| Summary This technical report aims to define Wangiri fraud in telecommunications and to provide the most effective methodologies to mitigate this type of fraud |  |  |  |  |
| Relations to ITU-T Recommendations or other documents (approved or under development): |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |
| Supporting members that are committing to contributing actively to the work item: |  |  |  |  |
| Sudan, Egypt, Sierra Leone, Uganda, UAE. |  |  |  |  |

______________