|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  |  |
| --- | --- | --- | --- |
|  |  |  | T22-SG09-C-0067 STUDY GROUP 9 |
|  |  |  | Original: English |
| Question(s): |  | 1/9 | Bangalore, 9-18 May 2023 |
| Contribution Source:; Ministry of Communications, India; Centre for Development of Telematics, India. Title:; Proposal for updating the baseline text of the draft new ITU-T Technical Report J.TR.WIFI-TV "Secondary distribution of digital television and audiovisual content to portable devices using WiFi”. Contact:; Avinash AgarwalMinistry of Communications India; Tel: +91-9868171222 E-mail: avinash.70@gov.in. Contact:; Pallab Dutta Centre for Development of Telematics India; Tel: +91-9343795475 E-mail: pallab@cdot.in. Contact:; Manavdeep Singh Centre for Development of Telematics India; Tel: +91-9971350079 E-mail: msingh@cdot.in. |
| Abstract: |  | This contribution proposes to update the baseline text of draft new Technical Report J.TR.WiFiTV of “Secondary distribution of digital television and audiovisual content to portable devices using Wi-Fi.” |  |

At the last SG9 meeting in September 2022, it was agreed to start a new work item J.TR.WIFI-TV "Secondary distribution of digital television and audiovisual content to portable devices using WiFi”.

The baseline text agreed during the SG9 Meeting (2022-09) is available as SG9-TD180/Gen (2022-09). Then work was further progressed during the Rapporteur Group Meeting of Q1/9 held on 09 December 2022. The output document of the RGM (221201) is available as Q1/9-DOC4-A1 (221213). In this contribution, India proposes to add more details to progress the work further.

The proposed baseline text is placed at ANNEX.

|  |  | International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | Technical Report |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (dd MMM YYYY) |
|  | J.TR.WiFiTV Secondary distribution of digital television and audiovisual content to portable devices using Wi-Fi |  |  |  |

Summary

The Technical Report addresses aspects related to delivering unencrypted television and other audiovisual signals to smartphones and other portable devices using Wi-Fi constituting the last mile of secondary distribution TV network. In other words, the scope of the draft Technical Report is intended to cover the last mile connection via Wi-Fi interconnected with the conventional TV transmission modes of satellite, cable, or terrestrial.

This concept has high value propositions in rural areas, public utility places such as railway stations and airports, and even in moving vehicles such as buses, taxis, etc.

SG9 is going to develop this Technical Report from the view point of integrated broadband cable television technology and also would like to refer to or collect other necessary information by collaborating with relevant Study Groups or SDOs. While the proposed Technical Report would cover the network architecture, transcoding requirements, other technical requirements, mechanism to facilitate local content distribution, local advertisement insertion, logging end user statistics, case studies, and more. The Technical Report would not cover the radio communication broadcasting.

Keywords

Converged assess node, Wi-Fi, secondary distribution, smartphone

Change Log for development

This document contains the draft ITU-T Technical Report J.J.TR.WiFiTV on “Secondary distribution of digital television and audiovisual content to portable devices using Wi-Fi” approved as the new work item at the ITU-T Study Group 9 meeting held in Virtual, 06-14 Sept. 2022.

| Editor: | Avinash Agarwal Ministry of Communications India | Tel: +91-9868171222 Email: avinash.70@gov.in |
| --- | --- | --- |
|  | Pallab Dutta Centre for Development of Telematics India | Tel: +91-9343795475 E-mail: pallab@cdot.in |

CONTENTS

Page

1 Scope 1 2 References 1 3 Terms and definitions 1 3.1 Terms defined elsewhere 1 3.2 Terms defined here 1 4 Abbreviations 2 5 Introduction 2 6 Network architecture for secondary distribution using Wi-Fi or WLAN 2 7 Converged access node requirements 4 8 Transcoding requirements 5 9 Requirements to meet regulatory compliance for encrypted channels 5 10 Local content distribution 5 11 Local advertisement insertion 5 12 End user statistics logging 5 13 Case studies and global best practices 5 14 Concluding Remarks 5 15 Bibliography 5 Annex A Example of annex 6 Appendix I Example of appendix 6

List of Tables

Page

Table 1: Example of a table 5

List of Figures

Page

Figure 1: Example of a figure 6

Technical Report ITU-T J.TR.WiFiTV

ITU-T Technical Report on Secondary distribution of digital television and audiovisual content to portable devices using Wi-Fi

Summary

The Technical Report addresses aspects related to delivering unencrypted free to view television and other audiovisual signals to smartphones and other portable devices using Wi-Fi constituting the last mile of secondary distribution TV network. In other words, the scope of the draft Technical Report is intended to cover the last mile connection via Wi-Fi interconnected with the conventional TV transmission modes of satellite, cable, or terrestrial.

This concept has high value propositions in rural areas, public utility places such as railway stations and airports, and even in moving vehicles such as buses, taxis, etc.

SG9 is going to develop this Technical Report from the view point of integrated broadband cable television technology and also would like to refer to or collect other necessary information by collaborating with relevant Study Groups or SDOs. While the proposed Technical Report would cover the network architecture, transcoding requirements, other technical requirements, mechanism to facilitate local content distribution, local advertisement insertion, logging end user statistics, case studies, and more. The Technical Report would not cover the radio communication broadcasting.

# Scope

The Technical Report addresses aspects related to delivering unencrypted free to view  television and other audiovisual signals to smartphones and other portable devices using Wi-Fi constituting the last mile of secondary distribution TV network. In other words, the scope of the draft Technical Report is intended to cover the last mile connection via Wi-Fi interconnected with the conventional TV transmission modes of satellite, cable, or terrestrial.

The objective of the Technical Report is to collect information that may drive future standardization to enhance the use of widely available Wi-Fi networks and Wireless LAN networks for the last mile delivery of linear television and audiovisual content to portable devices in a manner that is cost-effective for the end users and does not load the mobile networks.

# References

- To be added as the study progresses.

# Terms and definitions

## Terms defined elsewhere

This Technical Report uses the following terms defined elsewhere:

3.1.1	term [reference]: definition

3.1.2	term [reference]: definition

## Terms defined here

This Technical Report defines the following terms:

3.1.1	term [reference]: definition

3.1.2	term [reference]: definition

[To be added as the study progresses.]

# Abbreviations

| WLAN | Wireless Local Area Network |
| --- | --- |
| IRD | Integrated Receiver Decoder |
| DTV | Digital television |
| DVB | Digital Video Broadcasting |
| UDP | User Datagram Protocol |
| OTT | Over the Top |
| AVC | Advanced Video Coding |
| HEVC | High Efficiency Video Coding |
| AAC | Augmentative and alternative communication |
| WAV | Waveform Audio File Format |
| MPEG | Moving Picture Experts Group |

# Introduction

Mobile handsets, tablets and laptops are becoming the preferred devices for viewing linear television and audiovisual content. This however consumes a lot of mobile data and is costly for the consumers. Also, many concurrent users may create network congestion in typical mobile networks, thus limiting the Quality of Service. Harnessing the digital satellite and digital cable platforms requires the receiving device to have an integrated or separate DVB-T2/S2 receiver module. This is not feasible for smartphones and other mobile devices.

The solution exists in using a converged access node as a middleware, which receives the linear television and other signals through the conventional modes such as satellite, cable or terrestrial, and then distributes them using Wi-Fi or other WLAN. The end users can view the television or other audiovisual content on their portable devices without consuming mobile data and without requiring any additional hardware or plugin, etc.

This concept has high value propositions in rural areas, public utility places such as railway stations and airports, and even in moving vehicles such as buses, taxis, etc.

The proposed Technical Report aims to provide guidance on technical aspects and also share global best practices through case studies.

# Network architecture for secondary distribution using Wi-Fi or WLAN

[Editor’s note: Text to be drafted – contributions encouraged.]

A converged access node consists of various network components responsible for operations such as reception, demodulation, encoding etc.

Cable/Terrestrial/Satellite content is received, demodulated, decoded, and finally selectively streamed in appropriate formats over the wifi. A local content server is hosted for various offline services.

The content is served in a format that can be consumed by end-users using browsers in smartphones and laptops by accessing a web portal through the Wifi without using any special player. Additionally, the end-user can switch to regular OTT services if the access point has been connected to internet backhaul.

Functional blocks are shown in Figure 1

IRD

Video & Media Server

Web Server

Browser on Smartphone/Tablet/Laptop

Local Content Server

Web

Fig. 1

# Converged access node requirements

[Editor’s note: Text to be drafted – contributions encouraged.]

- IRD (Integrated Receiver Decoder):

An IRD should be able to connect to cable or antenna feed. A dish antenna on the ground at the node location should receive the downlinked signals from the satellite signals. Terrestrial signals can similarly be received via a smaller antenna. An amplification device at the antenna should amplify and convert the signals to a suitable band before consumption by IRD devices for demodulation.

An IRD should convert the signals to baseband signals containing multiple DTV or Radio channels. It should be noted that the IRD is usually tuned only for a particular frequency and can only receive a certain number of channels only. Hence multiple IRDs might be required.

- Video and Media Server :

Streams generated by and IRD are not playable by any industry standard browser directly and would need encoding by a Video server block. Encoding details are discussed in point 9.

Also, it should be ensured that the audio/video content is consumed by the end user without having to rely on any kind of specialized software or plugin. Having to install third party plugins or players is not only an inconvenience but also a risk factor for the user.

The Video Server block should be able to solve the above problem by transcoding the UDP stream into a form ingestible by Media server. The Media server further converts this stream into files which can be hosted over the web and played by in a web browser.

- Web Server:

The function of a web server can be two-fold:

a.       To act as an HTML/Web server for serving the audio/video content generated in previous step to the end-users’ browser &

b.       To learn from the user viewership statistics and come up with a list of the number of channels as the most watched/desired channels.

The channels are thus selectively streamed initially for a period of time followed by a self-learning algorithm which determines the most desired/watched channels at a given location. After the algorithm converges to a decisive list over the time, the channels are finalized.

This brings down the cost terms of reliance on the number of demodulating devices. As conveyed in the “IRD” section, the number of such devices can ideally be a lot but the same is not desired and that purpose is solved by the algorithm inside the web server business logic layer.

- End-User Browser:

The smartphone/laptop/tab user should be able to consume the content without having to install any special app or plugin.

After connecting to the wifi, the user would access the web portal using an appropriate link. User should be able to navigate through the content served by algorithm in the previous step easily. The user shall be able to play the content of the served channels directly in the browser window.

# Transcoding requirements

[Editor’s note: Text to be drafted – contributions encouraged.]

The video codecs such as MPEG-2, H.264/AVC, and H.265/HEVC etc. and audio codecs such as MPEG-1 audio, mp3, AAC and AC3 etc. are widely used codecs in input MPTS streams. Other codecs for input may also be supported.

Video Server shall support de-multiplexing and transcoding of individual SPTS streams to H.264, H.265 video codecs and AAC audio codecs. The transcoded streams may further be ingested by Media Server block for streaming using DASH/HLS protocols.

# Local content distribution

[Editor’s note: Text to be drafted – contributions encouraged.]

The local content  , such as video, audio  can be distributed.. The supported formats for the local content is .mp4, AV1 for video, .mp3 for audio respectively.

There is no limitation or restriction of content file size and numbers of content files.

# Local advertisement insertion

[Editor’s note: Text to be drafted – contributions encouraged.]

Advertisements can be inserted locally at the equipment site, either at a dedicated channel or can also be done whenever any channel is switched (changed) by the end users.

# End user statistics logging

[Editor’s note: Text to be drafted – contributions encouraged.]

The system shall support the logging of end user statistics in a database. Parameters like number of live users, view minutes per channel, clients per channels etc. may be captured. Data shall be presented in various formats such as pie/bar chart or table.

The system shall also provide for viewing of statistics on daily, monthly or on custom date range basis.

# Case studies and global best practices

[Editor’s note: Text to be drafted – contributions encouraged.]

# Concluding Remarks

[Editor’s note: Text to be drafted – contributions encouraged.]

# Bibliography

[To be added as the study progresses.]

Annex A
Example of annex

[ADD TEXT].

Appendix I
Example of appendix

[ADD TEXT, IF ANY].

___________________