|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG15-C0368 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 15 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 6/15 |  | Geneva, 17-28 April 2023 |  |
| CONTRIBUTION |  |  |  |  |  |
| Source: |  | Ministry of communication, Government of India |  |  |  |
| Title: |  | Proposal for a new work item “System, Functional and Engineering requirements for establishing Free Space Optical (FSO) transmission systems” |  |  |  |
| Contact: |  | Abdul Kayum Ministry of Communications India |  |  | Tel: +918874209603 Email: Abdul.kayum@gov.in |
| Contact: |  | Rakesh Goyal Ministry of Communications India |  |  | Tel: +919015365488 Email: rakesh.goyal91@gov.in |
| Contact: |  | Avadhesh Singh Ministry of Communications India |  |  | Tel: +919868139334 Email: Avadhesh.singh48@gov.in |
| Contact: |  | Rakesh Desai Ministry of Communications India |  |  | Tel + 919426203403 Email: rakesh.desai@gov.in |

| Abstract: | This contribution proposes to start a new work item about System, Functional and Engineering requirements for establishing Free Space Optical (FSO) transmission systems. |
| --- | --- |

# Scope

Free-Space Optics offers high data rates and low latency, making it an attractive alternative for telecommunication applications. However, designing and implementing FSO systems can be complex and challenging due to the unique characteristics of the technology and the environment in which it operates. This Recommendation defines system, functional and engineering requirements for establishing Free Space Optical (FSO) transmission systems including co-located FSO systems. It may also help to ensure interoperability between different FSO systems from different manufacturers and operators, facilitating wider adoption of the technology.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-R P.1817-1]	Recommendation ITU-R P.1817-1 (02/2012), Propagation data required for the design of terrestrial free-space optical links.

[ITU-T G.640]	Recommendation ITU-T G.640 (03/2006): Co-location longitudinally compatible interfaces for free space optical systems.

[ITU-T G.957]	Recommendation ITU-T G.957 (2006), Optical interfaces for equipments and systems relating to the synchronous digital hierarchy

[ITU-T G.Sup39]	Recommendation ITU-T G.Sup39 (02/2016), Optical system design and engineering considerations.

[IEC 60825-1:2001]	Recommendation IEC 60825-1 (2001), Safety of laser products – Part 1: Equipment classification, requirements and user's guide.

[IEC 60825-2:2005]	Recommendation IEC 60825-2 (2005), Safety of laser products – Part 2: Safety of optical fibre communication systems (OFCS).

[IEC 60825-12:2005]	Recommendation IEC 60825-12 (2005), Safety of laser products – Part 12: Safety of free space optical communication systems used for transmission of information.

# Definitions

<Check in the ITU-T terms and definitions database at www.itu.int/go/terminology-database whether the term has already been defined in another Recommendation. It would be more consistent to refer to such a definition rather than to redefine the term>

## Terms defined elsewhere

<Normally, terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document. Before defining a new term, verify whether it has already been defined in the official ITU terminology database, at www.itu.int/go/terms. >

This Recommendation uses the following terms defined elsewhere:

    - <Term 1> [Reference]: <optional quoted definition>.

    - <Term 2> [Reference]: <optional quoted definition>.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>.

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

<abbr>	<expansion>

APC	Adaptive power control

BER	Bit Error Ratio

CPRI	Common public radio interface

FSO	Free Space Optical

RMS	Root-Mean-Square

<Include all abbreviations and acronyms used in this Recommendation>

# Conventions

<Mandatory clause. Describe any particular notation, style, presentation, etc. used within the Recommendation, if any. If none, write "None.">

# Clause 6 of the Recommendation

<Clause 6 onwards contains the body of the Recommendation>

## Classification of Systems (optical interfaces) (G.957)

A free‑space optical (FSO) communication system is a point‑to‑point wireless optical communication system between transceivers, using free space as the transmission medium instead of optical fiber. By appropriate combinations of transmitters and receivers, power budgets for free space optical systems can be achieved which are optimized in terms of attenuation and cost with respect to the various applications. FSO is gaining popularity due to its high bandwidth (~ Gbps), easy installation, better security than RF access and unlicensed spectrum.

A short-range FSO system (< 1 Km) may require low-power transmitters and receivers and has comparative low attenuation. Such systems can be cost-effective and suitable for indoor applications such as wireless communication in homes, offices, and public spaces. A long-range FSO system (> 1 Km) may require high-power transmitters and sensitive receivers to overcome the higher attenuation and signal loss over longer distances for applications such as Telecom network extensions, backhaul connectivity, etc. Furthermore, the selection of appropriate modulation techniques and channel coding schemes can also impact the power budget and performance/throughput of FSO systems.

Classification can be done on the basis of link distance, throughput and optical window.

| S.No. | Wavelength | Link Distance | Throughput | Applications |
| --- | --- | --- | --- | --- |
| A |  |  |  |  |
| B |  |  |  |  |
| C |  |  |  |  |
| … |  |  |  |  |

Classification can also be done on the basis of link reliability, total transmitted power, transmission beam width, modulation scheme, receiving optics collecting area, and receiver sensitivity.

- System Requirements:

## Criterion:

- to satisfy minimum requirements for worst case scenarios

- different system requirements to be made for each of the classes mentioned in 6.1.

| ## Parameters | ## Unit | ## Values |
| --- | --- | --- |
| ## Digital signal Nominal bit rate | ## kbit/s |  |
| ## Application code |  |  |
| ## Operating wavelength range | ## nm |  |
| ## Transmitter at reference point S |  |  |
| ## Source type |  |  |
| ## Spectral characteristics: |  |  |
| ## maximum RMS width (σ) | ## nm |  |
| ## maximum −20 dB width | ## nm |  |
| ## minimum side mode suppression ratio | ## dB |  |
| ## Mean launched power: |  |  |
| ## maximum | ## dBm |  |
| ## minimum | ## dBm |  |
| ## Minimum extinction ratio | ## dB |  |
| ## Optical path between S and R |  |  |
| ## Attenuation range | ## dB |  |
| ## Receiver at reference point R |  |  |
| ## Minimum sensitivity | ## dB |  |
| ## Minimum overload | ## dBm |  |
| ## Maximum optical path penalty | ## dB |  |
| ## Maximum reflectance of receiver, measured at R | ## dB |  |
| ## Receiving optics collecting area |  |  |
| ## Modulation Technique |  |  |
| ## Range at Max beam divergence |  |  |
| ## Bit Error Ratio (BER) |  | ## BER of 10−12 or better (ITU-T Rec. G.826) |
| ## Interferometric crosstalk penalty (G.640) | ## dB |  |
| ## Co-location longitudinal compatibility |  |  |
| ## Minimum separation where the Systems can be at the same wavelength | ## m |  |
| ## Minimum separation where the Systems can be at the different wavelength | ## m |  |
| ## Adaptive optics |  |  |
| ## …………… |  |  |

## Parameters specified for free space optical interfaces (G.957)

(There will be multiple tables according to the classification above)

## Aging of Receiver

## The aging of the receiver components can affect the sensitivity and reliability of the FSO system. Typical margins between the beginning-of-life, nominal temperature receiver and its end-of-life, worst-case counterpart are desired to be in the 2 to 4 dB range.

## Physical Obstructions

## The system is to be designed to deploy widely spaced redundant transmitters and large receive optics to eliminate interference concerns from objects such as birds.

## Pointing Stability – Building Sway, and Tower Movement

## Building sway, and tower movement can cause misalignment between the transmitter and receiver. The system must be capable of handling the vast majority of movement found in deployments on buildings/tower. A combination of Pointing and acquisition tracking and a well matched receive Field-of-View (FOV) can provide the required stability.

## Scintillation

## The performance of FSO wireless systems is adversely affected by scintillation and the atmospheric turbulence can cause changes in the refractive index of the air. The system design to be capable of handling this situation using a unique combination of diversity, widely spaced transmitters, finely tuned receive filtering, and automatic gain control characteristics etc. so as to eliminate the effects of atmospheric scintillation and jitter transference.

## Solar Interference

## The system design to include measures to reduce/eliminate background noise induced by solar interference around the operating wavelength of the system.

## Co-location longitudinal compatibility (G.640)

## The system should be designed with optical interfaces for co-location longitudinally compatibility, thereby enabling interference-free coexistence of more than one point-to-point free space optical system at a location.

## Reliability

## System design to include an as adaptive coded modulation technique and/or adaptive laser power (Adaptive Power Control or APC) scheme to dynamically adjust the laser power in response to weather conditions.

## May include backup measures with other technologies such as RF or optical fiber systems to maintain SLA uptime.

## User side interfaces

| ## Interface type (Name/bit rate) | ## Standard |
| --- | --- |
| ## Data Interface |  |
| ## Ethernet |  |
| ## CPRI |  |
| ## eCPRI |  |
| ## Synchronization interface |  |
| ## Any other/proprietary interface |  |

## Functional Requirements:

## Criterion:

## to satisfy minimum performance requirements of the users for different applications:

## Coverage

## Throughput

## Latency

## to satisfy security requirements of users for different applications

## to provide resilience (diversity) and backup to ensure uninterrupted communication

- to meet synchronization requirement of application

- to provide EMS for network operation and maintenance

- to ensure compatibility with different protocols and interfaces used by different applications

- to support encryption and authentication mechanism.

- to support adaptive power control to optimize power consumption while maintaining reliable communication.

## Engineering Requirements:

## Criterion:

## to meet EMI/EMC requirements in different deployment conditions.

## to meet Electrical and Optical Safety requirements in different deployment conditions.

# ANNEX 1
A.1 Justification for the proposed draft new recommendation

| Question: | Q6/15 | Proposed new ITU-T Recommendation |  | 17-28 April 2023 |  |
| --- | --- | --- | --- | --- | --- |
| Reference and title: | Proposal for a new work item “System, Functional and Engineering requirements for establishing Free Space Optical (FSO) transmission systems” |  |  |  |  |
| Base text: | SG15-C0368 |  | Timing: |  | 2023-04 |
| Editor(s): | Rakesh Goyal |  | Approval process: |  | AAP |
| Scope (defines the intent or object of the Recommendation and the aspects covered, thereby indicating the limits of its applicability): |  |  |  |  |  |
| This proposal describes the System, Functional and Engineering requirements for establishing Free Space Optical (FSO) transmission systems. |  |  |  |  |  |
| Summary (provides a brief overview of the purpose and contents of the Recommendation, thus permitting readers to judge its usefulness for their work): |  |  |  |  |  |
| Free-Space Optics offers high data rates and low latency, making it an attractive alternative for telecommunication applications. This contribution proposes the establishment of a new work item that defines system, functional, and engineering requirements for Free Space Optical (FSO) transmission systems classified based on link distance, throughput, and optical window. The recommendation aims to provide guidelines for designing and implementing FSO systems, ensure interoperability between different FSO systems from different manufacturers and operators, and facilitate wider adoption of the technology. |  |  |  |  |  |
| Relations to ITU-T Recommendations or to other standards (approved or under development): 1. [ITU-R P.1817-1] Recommendation ITU-R P.1817-1 (02/2012), Propagation data required for the design of terrestrial free-space optical links. This Recommendation will refer to the relevant content. 2. [ITU-T G.640] Recommendation ITU-T G.640 (03/2006): Co-location longitudinally compatible interfaces for free space optical systems. This Recommendation will refer to the relevant content. 3. [ITU-T G.957] Recommendation ITU-T G.957 (2006), Optical interfaces for equipments and systems relating to the synchronous digital hierarchy. This Recommendation will refer to the relevant content. 4. [ITU-T G.Sup39] Recommendation ITU-T G.Sup39 (02/2016), Optical system design and engineering considerations. This Recommendation will refer to the relevant content. 5. [IEC 60825-1:2001] Recommendation IEC 60825-1 (2001), Safety of laser products – Part 1: Equipment classification, requirements and user's guide provides guidance on the safe use of laser products. This Recommendation will refer to the relevant content. 6. [IEC 60825-2:2005] Recommendation IEC 60825-2 (2005), Safety of laser products – Part 2: Safety of optical fibre communication systems (OFCS) provides safety guidelines for optical fiber communication systems that use lasers. This Recommendation will refer to the relevant content. 7. [IEC 60825-12:2005] Recommendation IEC 60825-12 (2005), Safety of laser products – Part 12: Safety of free space optical communication systems used for transmission of information ensures the safe use of FSO systems by providing guidelines for designing and implementing them.. This Recommendation will refer to the relevant content. |  |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |  |
| Supporting members that are committing to contributing actively to the work item: |  |  |  |  |  |
| Ministry of Communications, India |  |  |  |  |  |

Bibliography

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title.

ANSI Z136.1.6: is specific to outdoor laser deployments: it standardizes the rules for deploying

FSOC in public locations (rooftops, cell towers etc.) from a public safety perspective.

___________________