|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG17 |  |
| --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 17 |  |
|  |  |  | Original: English |  |
| Question(s): |  | 15/17 | Goyang, Korea, 29 August – 8 September 2023 |  |
| Contribution |  |  |  |  |
| Source: |  | Editor |  |  |
| Title: |  | Threat analysis and management of security profile of trusted nodes of QKD systems |  |  |
| Contact: |  | Venkata Rama Raju Chelle Ministry of Communications, (India ) |  | Tel: +011-23321992 Fax: +91-9821979899 E-mail: cvramaraju.v@gov.in |

| Abstract: | This new draft proposal for analysing systems for threat analysis and assessment management methods for security profile of trusted QKD systems. |
| --- | --- |

- Introduction :

Several deliverable standards have been published for QKD architecture and key delivery management in QKDNs (Quantum Key Distribution Network). However, key management has been discussed in SG17 is key management in QKDN including key generation with QKD (Quantum Key Distribution) and key relay with trusted nodes.

There are other approaches for quantum-safe communications such as PQC (Post-Quantum Cryptography). Quantum-safe refers to the use of methods that are resistant to attacks by quantum computers, in order to keep information assets secure even after a large-scale quantum computer has been built. In short, quantum-safe means safe against quantum computing attack.

Although there are several deliverables are under discussion in SG17 to deal with QKDN key management, there have been no discussion to assess how QKD node to keep trusted and how to measure assessment to prove quantum application is theoretically secure.

This assessment to ensure   keys emanating from QKD to application layer is secure presuming communication   to be quantum-safe possibly with other possible means such as PQC.

- Gap analysis

In IT-T SG 13, recommendations on QKD,  QKDN fundamental sets have been completed, including Y. 3800 “Overview on networks supporting QKDN”, Y.3801 “Functional requirements for QKDN”, Y.3802 Functional architecture of QKDN.

Based on the architecture, reference points defined in the Y.3800 series of SG 13, SG11 has initiated a series of QKDN protocols related recommendations, including Q.QKDN_profr “Quantum key distribution networks – Protocol framework”, which covers general aspects of protocols for QKDNs, as well as specific interface protocols in classical layers in QKDN, such as Q.QKDN_CK, Q.QKDN_Kq-1, Q.QKDN_AK, Q.QKDN_Kx

Beyond ITU-T, ISO/IEC JTC 1 SC27 WG3 has developed ISO/IEC 23837 on Security requirements, test and evaluation methods for quantum key distribution Part 1: Requirements; Part 2: Evaluation and testing methods. The DIS has been approved and the standards are being prepared for publication.

. Draft GS QKD 005 “Security Proofs” is one of the supporting documents, which specifies structure of QKD protocols, security framework for QKD systems, security proof framework, assumptions for security, and post-processing for key generation. GS QKD 005 focuses on the security proof but it also generally covers processes of key generation such as raw key exchange and post-processing. Meanwhile, other work items include GR QKD 003 on QKD components, GS QKD 008 on QKD module security, GS QKD 013 on QKD module deployment. In ETSI ISG QKD,  GS QKD 016 “Common Criteria Protection Profile - Pair of Prepare and Measure Quantum Key Distribution Modules” and supporting documents released in April 2023. GS QKD 016 will specify security requirements of QKD module

ITU-T SG17 published several deliverables related to key management in QKDN as follows:

(1) X.1712 ‘Security requirements and measures for QKD networks – key management’

X.1712 specifies security threats and security requirements for key management in quantum key distribution networks (QKDNs), and security measures of key management to meet the security requirements. This Recommendation also provides support for the design, implementation, and operation of key management in QKDNs. This recommendation deals with the key management inside QKDNs, which is all activities, performed on keys during their life cycle starting from their reception from the quantum layer, the storage, formatting, relay, synchronization, authentication, to supply to cryptographic application and delete or preserve depending on the key management policy.

(2) X.1714 ‘Key combination and confidential key supply for quantum key distribution networks’

Recommendation ITU-T X.1714 describes key combination methods for quantum key distribution network (QKDN) and specifies security requirements for both the key combination and the key supply from QKDN to cryptographic applications.  In particular, this Recommendation addresses the following points:

–	The security of the combination of keys exchanged through a QKDN and keys exchanged through other key exchange methods;

–	The security of the key supply from a QKDN to cryptographic applications.

Figure 1 – Key combination method for QKDN

Figure 1 in ITU-T X.1714 shows a general description of the key combination considered in this Recommendation. It consists of a key combiner, which transforms two or more input keys supplied by different key exchange methods into a combined key. One of the input keys is supplied by a QKD-based key exchange.

When the key combiner is located outside the QKDN and used in the service layer, the QKD-based key exchange is a key exchange performed by the QKDN. When the key combiner is in the QKDN, the QKD-based key exchange is a key exchange between QKD modules with or without key relays. The other input keys can be provided by any kind of method allowing the exchange of symmetric keys between two nodes, including cryptographic means and physical means (e.g., subscriber identity module (SIM) card).

If the key combiner is located outside the QKDN and is used as a cryptographic application, the requirements fully depend on the policy of the user of the application. Specification of the security requirements in this case is outside the scope of X.1714.

(3) XSTR-HYB-QKD ‘Overview of hybrid approaches for key exchange with quantum key distributions’

This technical report provides a landscape of the standardization activities on hybrid approaches for migration towards quantum-safe algorithms or protocols within international, regional and national organizations. The hybrid approach that is covered by this technical report is for key exchange.

Hybrid approaches for key exchange consist of generating a key exchange functionality by combining at least two different key exchange methods. This technical report studies the possible way forward to accommodate quantum key distribution protocols in the context of hybrid approaches for key exchange. This compatibility is studied for generic hybrid key exchange and hybrid key exchange that is specific to certain communication protocols. The scope and the objective of this technical report is to study how QKD keys can be used for other protocols for key exchange which are already existing and have been being standardized.

Although the several deliverables have been published for the key management in QKDNs, all deliverables are for the key management inside of QKDNs which means they are key management specific to QKDNs. XSTR-HYB-QKD is not specific for QKDNs, however it is for existing (or being standardized) cryptographic protocols to accommodate QKD keys inside of their protocols.

None of these deliverables deals with the key management for quantum-safe communication with hybrid approaches with QKD and PQC from user perspective to leverage all available solutions including QKD for their benefits depending on the use cases.

(4) ETSI TC Cyber QSC WG published several deliverables related to PQC case studies, use cases, threat analysis, and analysis of quantum-safe primitives and so on as follows:

- ETSI GR QSC001 Analysis of Quantum-Safe Primitives

- ETSI GR QSC003 Quantum-Safe Case Studies & Use Cases

- ETSI GR QSC004 Quantum-Safe Threat Analysis

- ETSI TR 103 570 Quantum-Safe Key Exchanges

- ETSI TR 103 617 Quantum Safe Virtual Private Networks

- ETSI GS QKD 016 V1.1.1 (2023-04)

Initial Draft proposal on QKD Security profile

Over view  threat analysis and management of security profile of trusted nodes of QKD systems

Summary

This initial draft proposal for addressing overall security measures of QKD and assessment methods to keep theoretically information secure by ensuring key generation and distribution is secure in trusted node.  This proposal ultimately addresses key management to application layer is secure for hybrid approaches of quantum-safe communication.

Keywords

QKD (quantum key distribution), QKDN (QKD network), security requirements, PQC (post quantum cryptography), quantum-safe communications, Quantum Authentication Key (QAK), authentication, authorization, PKI, security requirements, security measures

Table of Contents

1.	Scope	7

2.	References	8

3	Definitions	9

4.	QKD subsytems	10

4.1	QKD module	10

4.2	KMA	11

4.3	KSA	11

4.4	QKDN controller	12

5.	Abbreviations and acronyms	12

6.	Conventions	12

7.	Quantum -safe communications with quantum Key management and service scenarios	13

Considerations	14

8.	IDs and their management in QKDN	14

8.1	IDs for functional components in QKDN	14

8.2	IDs for key operation and key relay	14

8.3	IDs for key operation and key supply	15

8.4	IDs for key generation in QKD modules	15

9.	Reference points in QKDN in functional subsystem	15

Figure 3- Functional elements and associated reference points in a QKDN Entitiy authentication procedures	16

10.	Public key certification supported by PKI	16

11.	Authentication and authorization in QKDN	17

12.	Security requirements	17

12.1	Key acquisition, synchronization, storage in KMAs	17

12.2	Key relay between KMAs	18

12.3	Key supply from KSAs to cryptographic applications	18

12.4	Access Control:	18

12.5	Asset Management Controls:	19

12.6	Physical and Environmental Security Controls:	19

12.7	Operational Security Controls	19

12.8	Audit Data	19

12.9	Reaching and preserving secure states	19

12.10	Authenticated classical channel of QKD link	19

12.11	QKD Key Establishment	19

12.12	Management	19

12.13	Evaluation Assurance Level	19

12.14	Trusted User Interfaces with Authentication	19

12.15	Target of Evaluation (TOE)  self-protection	19

12.16	Provisioning and re-personalization after delivery	19

12.17	Local Authentication of Users	19

12.18	Guidance for SFR for RNG	19

Bibliography	20

# Scope

This initial draft proposal for analysing   existing system   for threat analysis and assessment management methods for security profile of trusted QKD systems.

This Protection Profile (PP) describes the security requirements for Quantum Key Distribution modules (QKD modules) that use a Prepare and Measure QKD protocol (P&M protocol) and entanglement based protocol. This PP considers the case, where both modules are located in environments with identical security requirements.

QKD can guarantee the information-theoretical security for key exchange, but it requires special devices and network infrastructure. PQC algorithms have been developed to be safe against quantum computer attacks, but they are still based on computational security, which means they can be broken by new emerging algorithms in the future. Furthermore, there are no widely accepted commercial systems that use PQC yet.

Therefore, it is natural to use both methods QKD and PQC are crucial for information theoretically secure using quantum  phenomenon.   Security and maintain trusted QKD node is crucial to safe guard information security through application presuming key is secured and need assessment of vulnerability keys and node as a whole and ensure by assessment of various possible scenarios till the delivery of key to application and taking high entropy quantum source from Quantum Random Number Generator(QRNG), wherever it is applicable.

In ITU-T deliverables, QKDN deals with the key management inside of QKDN to supply to the cryptographic applications. To realize quantum-safe communications with hybrid approaches with QKD and PQC, it is important for service providers to consider all available technologies and have the option to take them and assess security profile.

Information assets in QKDN (i.e. key data, metadata and control and management information) can be breached and altered during key generation, key relay and key storage etc. Confidentiality and integrity of information assets need to be protected by appropriate security measures. Such a functionality can be built on cryptographic protocols (public key cryptography, hash functions etc.) that are computationally secure or even IT-secure (e.g., Wegman-Carter message authentication).

# References

[ITU-T X.1710] 	Recommendation ITU-T X.1710 (2021), Security framework for quantum key distribution networks

[ITU-T X.1712] 	Recommendation ITU-T X.1712 (2021) Corrigendum 1, Security requirements and measures for quantum key key distribution networks – key management

[ITU-T X.1714] 	Recommendation ITU-T X.1714 (2020), Key combination and confidential key supply for quantum key distribution networks

[ITU-T XSTR-HYB-QKD] 	Technical Report ITU-T XSTR-HYB-QKD (2022), Overview of hybrid approaches for key exchange with quantum key distribution

[ITU-T X.509] 	Recommendation ITU-T X.509 (2016), Information technology – Open Systems Interconnection – The Directory: Public-key and attribute certificate frameworks

[ITU-T Y.3800] 	Recommendation ITU-T Y.3800 (2019)/Cor.1 (2020), Overview on networks supporting quantum key distribution.

[ITU-T Y.3801] 	Recommendation ITU-T 3801 (2020) Functional requirements for quantum key distribution network.

[ITU-T Y.3802] 	Recommendation ITU-T Y.3802 (2020), Functional architecture of the quantum key distribution network.

[ITU-T Y.3803] 	Recommendation ITU-T Y.3803 (2020), Key management for quantum key distribution Networks

[ITU-T Y.3804] 	Recommendation ITU-T Y.3804 (2020), Control and Management for Quantum Key Distribution Networks.

# 3	Definitions

3.1 control [b-ISO/IEC 27000]: measure that is modifying risk.

Key Manager (KM) [ITU-T Y.3800]: A functional module located in a quantum key distribution (QKD) node to perform key management in the key management layer.

Key Management [ITU-T Y.3800]: All activities performed on keys during their life cycle starting from their reception from the quantum layer, the storage, formatting, relay, synchronization, authentication, to supply to cryptographic application and delete or preserve depending on the key management policy.

Quantum Key Distribution (QKD) [b-ETSI GR QKD 007]: Procedure or method for generating and distributing symmetrical cryptographic keys with information theoretical security based on quantum information theory.

Quantum Key Distribution link (QKD link) [ITU-T Y.3800]: A communication link between two quantum key distribution (QKD) modules to operate the QKD.

NOTE – A QKD link consists of a quantum channel for the transmission of quantum signals, and a classical channel used to exchange information for synchronization and key distillation.

Quantum Key Distribution module (QKD module) [ITU-T Y.3800]: A set of hardware and software components that implements cryptographic functions and quantum optical processes, including quantum key distribution (QKD) protocols, synchronization, distillation for key generation, and is contained within a defined cryptographic boundary.

NOTE – A QKD module is connected to a QKD link, acting as an endpoint module in which a key is generated. These are two types of QKD modules, namely, the transmitters (QKD-Tx) and the receivers (QKD-Rx).

quantum key distribution network (QKDN) [ITU-T Y.3800]: A network comprised of two or more quantum key distribution (QKD) nodes connected through QKD links.

NOTE – A QKDN allows sharing keys between the QKD nodes by key relay when they are not directly connected by a QKD link.

quantum key distribution network controller (QKDN controller) [ITU-T Y.3800]: A functional module, which is located in a quantum key distribution (QKD) network control layer to control a QKD network.

quantum key distribution network manager (QKDN manager) [ITU-T Y.3800]: A functional module, which is located in a quantum key distribution (QKD) network management layer to monitor and manage a QKD network.

quantum key distribution node (QKD node) [ITU-T Y.3800]: A node that contains one or more quantum key distribution (QKD) modules protected against intrusion and attacks by unauthorized parties.

NOTE – A QKD node can contain a key manager (KM).

3.1.10  QKD Authentication Key (QAK): shared secret used for authentication mechanisms between both QKD modules

# QKD subsytems

## QKD module

All the QKD module IDs are registered in a registry of the access control function in the QKDN controller, and authenticated by it via the control and management links, preventing unauthorized QKD modules from accessing to the quantum layer. Based on this registry, the QKDN controller issues certificates to the registered functional components including QKD modules, and supports authentication between the QKD modules.

The QKD modules authenticate each other before performing QKD protocols, and authenticate a KMA before sending generated QKD-keys to the KMA.

## KMA

All the KMA IDs are registered in a registry of the access control function in the QKDN controller, and authenticated by it via the control and management links, preventing unauthorized KMAs from accessing to the key management layer. Based on this registry, the QKDN controller issues certificates to the registered functional components including KMAs, and supports authentication between the KMAs.

The KMAs authenticate each other at certain frequencies via the KMA links. Thus controlled access and authorization is realized. The authenticated KMAs is able to perform key relay by OTP encryption to protect the required confidentiality.

A KMA acquires QKD-keys from QKD module/modules in the same trusted node. The acquired QKD-key data is temporarily stored in a buffer and compared with the hash value stored in the matching KMA via the KMA link for synchronization and authentication. The authenticated QKD-keys are resized in an appropriate format, supplemented with the metadata, and are then stored in the KMA storage.

## KSA

All the KSA IDs are registered to a registry of the access control function in the QKDN controller, and authenticated by it at a certain frequency via the management links, preventing unauthorized KSAs from accessing to the key management layer.

Based on this registry, the QKDN controller issues certificates to the registered functional components including KSAs, and supports authentication between the KSAs.

A KSA receives a required amount of KMA-key. Then the KSA calculates a hash value of it, and compare it with that of the matching KSA for synchronization and authenticity test for individual key request, by using MAC.

Candidates of MAC include computationally secure schemes such as hash-based MAC and cipher-based MAC, and IT-secure schemes such as Wegman-Carter authentication scheme based on a family of universal hash functions.

The authenticated key is supplied as the KSA-key with the KSA-key ID to the cryptographic application. The cryptographic application identifies the key data based on the KSA-key ID, and uses it.

## QKDN controller

The access control function in a QKDN controller provides capabilities to verify the claimed identity of functional components under control and support by the QKDN controller and to restrict the functional components to pre-authorized activities or roles.

The access control function has an access control repository of QKD nodes, QKD modules, KMs, and cryptographic applications. The access control repository also contains authorized roles, access rights of these functional components, and their priorities based on enforced policies. Based on this repository, the access control performs the following functions:

- To register and delete IDs of functional components;

- To issue certificates to the registered functional components;

- To perform authentication and authorization between the QKDN controller and a functional component, referring to their certificates;

- To support authentication and authorization between the functional components according to their certificates and the access control repository.

# Abbreviations and acronyms

This Document uses the following abbreviations:

IT-secure	Information-theoretically secure

KM 	Key Manager

PQC	Post Quantum Cryptography

QKD.  	    Quantum Key Distribution

QKDN	QKD Network

QRNG.        Quantum Random Number Generator

SDLC              Software Development Life Cycle

# Conventions

This Recommendation applies the following verbal forms for the expression of provisions:

a)	"Shall" indicates a requirement,

b)	"Should" indicates a recommendation,

c)	"May" indicates a permission,

d)	"Can" indicates a possibility and a capability.

# Quantum -safe communications with quantum Key management and service scenarios

Figure 2 depicts the concept of the global key management of hybrid approaches compared to the key management of QKDN defined in ITU-T X.1712. The key management described in ITU-T X.1712 is how to relay the keys to locally inside of QKDNs among the trusted nodes to finally deliver the keys to the cryptographic applications. The key management resides in the use network which can be the existing key management for existing cryptographic functions to manage the keys from QKDNs (QKD keys) and other keys which can be PQC ones. For figurative purposes, only two ends ( one is transmitter/sender and other one receiver)  are depicted in Figure 2.

Node ‘A’ (sender/transmitter)                                Node ‘B’ Receiver

Figure 1 – Concept of Quantum -safe communications including quantum Key management

# Considerations

# IDs and their management in QKDN

QKDN ID: This ID is assigned as a unique ID to identify the QKDN. This clause will describe IDs in a QKDN. They are essential elements for authentication and authorization in a QKDN. This clause includes list of ID, and further specifications will be studied such as hieratic tree of IDs and their attributes.

The following is a basic list of IDs, which are used in a QKDN. Other IDs might be defined for various implementations.

## IDs for functional components in QKDN

This list of IDs is assigned by a QKDN manager to identify a functional component for management and security purposes such as configuration, authentication and authorization.

- QKD node ID: This ID is assigned as a unique ID to identify the QKD node in the QKDN.

- QKDN controller ID: This ID is assigned as a unique ID to identify the QKDN controller in the QKDN.

- QKDN manager ID: This ID is assigned as a unique ID to identify the QKDN manager in the QKDN.

- KM ID: This ID is assigned as a unique ID to identify a KM in the QKDN.

- KMA ID: This ID is assigned as a unique ID to identify a KMA in the QKDN.

- KMA link ID: This ID is assigned as a unique ID to identify a KMA link in the QKDN.

- KSA ID: This ID is assigned as a unique ID to identify a KSA in the QKDN.

- KSA link ID: This ID is assigned as a unique ID to identify a KSA link in the QKDN.

- QKD module ID: This ID is assigned as a unique ID to identify a QKD module in the QKDN.

- Application ID: This ID is assigned as a unique ID in the QKDN to identify a application which requests and acquires the keys from QKDN.

## IDs for key operation and key relay

This list of IDs is assigned by a KMA to perform key operations and key relay.

- Matching KMA ID: This ID is a KMA ID to identify a matching KMA.

- Source KMA ID: This ID is a KMA ID to identify a source KMA of key relay.

- Destination KMA ID: This ID is a KMA ID to identify a destination KMA of key relay.

- KMA-key ID: This ID is assigned as a unique ID in the QKDN to identify a KMA-key.

NOTE – Details of these IDs and operations are specified in [ITU-T Y.3803].

## IDs for key operation and key supply

This list of IDs is assigned by a KSA to perform key operations and key supply.

- KSA-Key ID: This ID is assigned as a unique ID in the QKDN to identify a KSA-key.

- Application source ID: This ID is an application ID to identify a source application.

- Application destination ID: This ID is an application ID to identify a destination application.

NOTE – Details of these IDs and operations are specified in [ITU-T Y.3803].

## IDs for key generation in QKD modules

This list of IDs is assigned by a QKD module to perform QKD protocols and key generation.

- Matching QKD module ID: This ID is a QKD ID to identify a matching QKD module.

- QKD-key ID: This ID is assigned as a unique ID in the QKDN to identify a QKD-key.

NOTE – Details of these IDs and operations are specified in [ITU-T Y.3803].

# Reference points in QKDN in functional subsystem

Figure 3 illustrates functional elements and associated reference points in a QKDN, which are defined in [ITU-T Y.3802], and [ITU-T Y.3803] except KMi reference point.

- Reference points associated with QKD module: Qdist, Qsync, Qqc

- Reference points associated with KM: Kx-1, Kx-2, Kq-1, Kq-2, KMi

- Reference points associated with QKDN controller: Cq, Ck, Cx, Cqrp, Cops,

- Reference points associated with QKDN manager: Mq, Mk, Mc, Mqrp, Mops

NOTE 1 – QKD links and reference points connecting to them (i.e. Cqrp, Cops, Mqrp, Mops) are not indicated in Figure 3.

NOTE 2 – A Cx reference point connecting two QKDN controllers is not indicated in figure 3. This reference point is used in a distributed QKDN.

NOTE 3 - KMi reference point is introduced in this Recommendation. This reference point is an internal reference point in KM, and connects KSA and KMA.

# Figure 3- Functional elements and associated reference points in a QKDN Entitiy authentication procedures

# Public key certification supported by PKI

The public-key infrastructure (PKI) is the infrastructure established to support the issuing, revocation and validation of digital certificates. Details of the PKI are specified in [ITU-T X.509]. In a PKI, certification authority (CA) is a functional component that issues certificate. CAs form a tree structure to construct trust chains. A CA in the top of the tree is called a root CA and it may be a trust anchor. A QKDN manger receives certificates from the PKI and the QKDN manger can be a root CA in the QKDN. The root CA in the QKDN manger issues certificates for the next CAs, which are located in functional components in the QKDN such as QKDN controllers, KMs and QKD modules. Functional components, which receive certificates, can use them for validation of digital signatures in public-keys. Digital certificates, which are provided by CAs, can also be used for entity and message authentications in the QKDN.

# Authentication and authorization in QKDN

This clause will describe procedures for entities and messages authentication and authorization in a QKDN. First, it identifies reference points where functional entities in the QKDN authenticate each other. Second, it describes procedures of entity authentications before communication starts. Third, it describes procedures of message authentications during communication.

# Security requirements

# Key acquisition, synchronization, storage in KMAs

KMA acquires QKD-key. To protect the integrity of QKD-key data, a hash value of the QKD-key is included in the metadata. The QKD-key is synchronized and authenticated in the quantum layer in two trusted nodes, and is transferred to the KMA. The integrity protection can be made by using message authentication code (MAC) via the classical channel.

Candidates of MAC include computationally secure schemes such as hash-based MAC and cipher-based MAC, and IT-secure schemes such as Wegman-Carter authentication scheme based on a family of universal hash functions.

Appropriate schemes may be selected based on policy of QKDN service providers, e.g., considering required time scales of integrity and authenticity protections, implementation costs, etc. Eventually MAC schemes are hopefully quantum-safe cryptographic schemes or IT-secure schemes in the post-quantum computer era.

NOTE 3 - Quantum-safe cryptographic schemes: Cryptographic schemes which are expected to be hard to decrypt in polynomial time even by quantum computing algorithms.

All the KMA IDs are registered in a registry of the access control function in the QKDN controller, and authenticated by it via the control and management links, preventing unauthorized KMAs from accessing to the key management layer. Based on this registry, the QKDN controller issues certificates to the registered functional components including KMAs, and supports authentication between the KMAs.

The KMAs authenticate each other at certain frequencies via the KMA links. Thus controlled access and authorization is realized.

Candidates of MACs for this purpose are the same as in the sub-clause 10.1.

A KMA acquires QKD-keys from QKD module/modules in the same trusted node. The acquired QKD-key data is temporarily stored in a buffer and compared with the hash value stored in the matching KMA via the KMA link for synchronization and authentication. The authenticated QKD-keys are resized in an appropriate format, supplemented with the metadata, and are then stored in the KMA storage.]

# Key relay between KMAs

The authenticated KMAs is able to perform key relay by OTP encryption to protect the required confidentiality.

After key relay, a pair of KMA-keys at the source and destination KMAs is put to synchronization and authenticity test by using MAC. Candidates of MACs for this purpose are the same as in the sub-clause 10.1.

# Key supply from KSAs to cryptographic applications

The KSAs are between the cryptographic applications and the KMAs.

All the KSA IDs are registered to a registry of the access control function in the QKDN controller, and authenticated by it at a certain frequency via the management links, preventing unauthorized KSAs from accessing to the key management layer.

Based on this registry, the QKDN controller issues certificates to the registered functional components including KSAs, and supports authentication between the KSAs.

A KSA receives a required amount of KMA-key. Then the KSA calculates a hash value of it, and compare it with that of the matching KSA for synchronization and authenticity test for individual key request, by using MAC. Candidates of MACs for this purpose are the same as in the sub-clause 10.1.

The authenticated key is supplied as the KSA-key with the KSA-key ID to the cryptographic application. The KSA metadata are sent to the QKDN manager for key life cycle management. The cryptographic application identifies the key data based on the KSA-key ID, and uses it.

# Access Control:

Adequate access control mechanism is required to ensure that only authorized user can access the System and applications. The access to the system may be provided and controlled by secure log-on procedure.

Additionally, security roles and access processes need to defined for creating, reading, deleting, updating or deactivating the data on phone.

# Asset Management Controls:

Guidelines to control the ownership of Hardware Tokens (signing keys) and other systems carrying the security infrastructure may be defined.

# Physical and Environmental Security Controls:

Processes defining the key Management procedures regarding use of Hardware Security Modules along with Physical Security mechanism’s such as physical protection of QKD module, auditing physical protection mechanism, Installation of CCTV or other gated Security Controls.

# Operational Security Controls

This includes the incorporation of standard Processes such as Vulnerability Assessment, Back up management, Password Management. Since Permissioned Models works on Server within the Private Network, standard processes related to security of Virtual Private Network will be applicable within QKDN.

# Audit Data

# Reaching and preserving secure states

# Authenticated classical channel of QKD link

# QKD Key Establishment

# Management

# Evaluation Assurance Level

# Trusted User Interfaces with Authentication

# Target of Evaluation (TOE)  self-protection

# Provisioning and re-personalization after delivery

# Local Authentication of Users

# Guidance for SFR for RNG

# Bibliography

[1]     ISO/IEC 27001:2022   Information Technology-Security Techniques Information Security Management Systems –Requirements

[2]     ETSI GS QKD 016 V1.1.1 (2023-04)   Quantum Key Distribution (QKD); Common Criteria Protection Profile - Pair of Prepare and Measure Quantum Key Distribution Modules

ISO/IEC 15408 for security requirements characterization and can be considered as their applications in specialization area of QKD technology.

ISO/IEC ISO/IEC 19790 for security requirements testing methods description, and can be considered as their applications in specialization area of QKD technology.