# Report on Terrestrial Broadcast in 5G Era Challenges and Opportunities

| Sr.no. | Topic | Page No. |
| --- | --- | --- |
| 1. | Introduction | 3 |
| 2. | What is “Terrestrial Broadcasting in 5G Era "? | 3 |
| 3. | What is the need of Terrestrial Broadcasting in 5G Era? | 4 |
| 4. | Purpose Behind Terrestrial Broadcasting in 5G Era | 4-5 |
| 5. | What is the Importance of Terrestrial Broadcasting in 5G Era for the Future of Content Distribution | 5-6 |
| 6. | Challenges for Terrestrial Broadcasting in 5G Era | 6 |
| 7. | What is The Future of Terrestrial Broadcasting in 5G Era | 7 |
| 8. | What is 3gpp for terrestrial broadcasting in the 5G era | 7-8 |
| 9. | Architecture of Terrestrial Broadcasting in 5G Era | 8-10 |
| 10. | Physical layer OF Terrestrial Broadcasting in 5G Era | 10 |
| 11. | Standardization initiatives and limitations of Terrestrial Broadcasting in 5G Era | 10-11 |
| 12. | Advantages of Terrestrial Broadcasting in 5G Era | 11-12 |
| 13. | Disadvantages of Terrestrial Broadcasting in 5G Era | 12 |
| 14. | Specifications for Terrestrial Broadcasting in 5 Era | 12-13 |
| 15. | Used case of Terrestrial broadcasting in the 5G era | 13-14 |
| 16. | Companies Testing broadcasting in 5g era | 14-15 |
| 17. | Standards available for Terrestrial Broadcasting in 5G Era | 15-19 |
| 18. | Characteristics of Terrestrial Broadcasting in 5G Era | 19-21 |
| 19. | Conclusion | 22 |
| 20. | REFRENCES | 23 |

# Abstract

The emergence of 5G technology has brought about new opportunities and challenges for terrestrial broadcasting. In this paper, we analyse the impact of 5G on terrestrial broadcasting and identify the key challenges and opportunities that arise. First, we discuss the technical aspects of 5G technology that have the potential to enable new services and enhance the quality of terrestrial broadcasting. Next, we examine the challenges that terrestrial broadcasters face in adapting to the new technology, including the need for spectrum, investment in infrastructure, and the need for new business models. Finally, we discuss the opportunities for terrestrial broadcasting in the 5G era, including the ability to offer new services and enhance the viewing experience for consumers.

# Introduction

The fifth generation of mobile communication technology (5G) is expected to have a profound impact on the media and broadcasting industries. In particular, the introduction of 5G will provide opportunities for terrestrial broadcasters to offer new services and enhance the quality of their existing services. At the same time, 5G also presents a number of challenges for terrestrial broadcasters, including the need for spectrum and investment in infrastructure. In this paper, we analyse the challenges and opportunities that 5G presents for terrestrial broadcasting.

# What is “Terrestrial Broadcasting in 5G Era "?

Terrestrial broadcasting in the 5G era refers to the use of radio waves to distribute audio and video content to a large audience, using the latest 5G wireless technology. Terrestrial broadcasting is a traditional method of broadcasting, where signals are transmitted from a broadcasting station to terrestrial receivers, such as televisions and radios.

In the 5G era, terrestrial broadcasting can take advantage of the higher bandwidth and faster data speeds offered by 5G wireless networks to deliver higher quality

content to viewers. 5G technology can also enhancethe user experience by providing interactive features and personalized content delivery.

Terrestrial broadcasting in the 5G era can also provide wider coverage, especially in remote areas where access to high-speed internet is limited. It can also be a more reliable means of delivering content, as it is not affected by issues such as internet congestion, bandwidth limitations, or buffering.

# What is the need of Terrestrial Broadcasting in 5G Era?

Terrestrial broadcasting continues to play an important role in the 5G era, despite the emergence of newer digital technologies. Here are some reasons why terrestrial broadcasting is still needed:

a)Wide coverage:Terrestrial broadcasting can provide wide coverage to a large audience, including those in remote areas, where access to high-speed internet is limited. This makes it an important tool for delivering content to a wide range of viewers, especially for live events such as sports, news, and entertainment programs. b)Reliability:Terrestrial broadcasting is a reliable means of delivering content, as it is not affected by issues such as internet congestion, bandwidth limitations, or buffering. It can also provide uninterrupted service during emergencies or natural disasters, making it an important tool for public safety and emergency communication.

- Cost-effectiveness:Terrestrial broadcasting can be more cost-effective than other distribution methods, as it does not require an internet connection or subscription free. This makes it accessible to a wider audience, regardless of their income level.

- Localization:Terrestrial broadcasting can be used to deliver localized content, such as regional news, weather updates, and advertisements. This allows broadcasters to target specific regions or demographics, providing relevant and timely content to their audience.

- Compatibility:Terrestrial broadcasting can be received on a wide range of devices, including radios, TVs, and mobile phones. This compatibilitymakes it an accessible and convenient way to deliver content to a broad audience .

# Purpose Behind Terrestrial Broadcasting in 5G Era:

Terrestrial broadcasting refers to the distribution of audio and video content through radio waves, which are transmitted through the air to terrestrial receivers, such as televisions and radios. In the 5G era, terrestrial broadcasting continues to play an important role in the distribution of media content.

One of the main purposes behind terrestrial broadcasting in the 5G era is to provide reliable and high-quality audio and video content to a large audience. While streaming services have become increasingly popular, they can be affected by internet congestion and bandwidth limitations, leading to buffering or degraded picture quality. Terrestrial broadcasting, on the other hand, can deliver content to a large audience simultaneously, without requiring an internet connection.

Another purpose of terrestrial broadcasting is to ensure emergency communication and public safety. In the event of a natural disaster or emergency, terrestrial broadcasting can provide critical information to the public, even if internet and cellular networks are down.

Additionally, terrestrial broadcasting can be used to deliver localized content, such as news, weather updates, and advertisements. This allows broadcasters to target specific regions or demographics, providing relevant and timely content to their audience.

Overall, terrestrial broadcasting remains an important distribution method in the 5G era, providing reliable, high-quality content to a large audience, as well as critical emergency communication and localized content delivery.

# What is the Importance of Terrestrial Broadcasting in 5G Era for the Future of Content Distribution:

Terrestrial broadcasting still plays an essential role in the distribution of content, even in the 5G era. While 5G technology offers high-speedinternet access to users, it is notwithout limitations such as coverage, signal strength, and data caps. Terrestrial broadcasting, on the other hand, offers wider coverage, free-to-air content, and high- quality broadcast reception.

Here are some reasons why terrestrial broadcasting remains important for content distribution in the 5G era:

  - Broad Coverage: Terrestrial broadcasting has a broad coverage area that can reach remote and rural areas, where 5G network coverage might not be available. This makes it possible for people to access content regardless of their location.

  - Free-to-air Content: Terrestrial broadcasting provides free-to-air content, making it accessible to everyone regardless of their financial status. This is particularly important in developing countries where access to paid content is limited.

  - High-quality Reception: Terrestrial broadcasting provides high-quality reception of content without buffering or interruptions. This is crucial for live events such as news, sports, and concerts where uninterrupted transmission is necessary.

  - Emergency Services: Terrestrial broadcasting is a vital communication tool during emergencies, such as natural disasters or terrorist attacks. It provides critical information to the public when other communication channels might be unavailable.

  - Efficient Use of Spectrum: Terrestrial broadcasting uses spectrum more efficiently than 5G networks, making it possible to broadcast content to millions of people simultaneously.

While 5G technology offers many benefits for content distribution, terrestrial broadcasting remains important in providing broad coverage, free-to-air content, high-quality reception, emergency services, and efficient use of spectrum.

# Challenges for Terrestrial Broadcasting in 5G Era:

Despite the potential benefits of 5G technology, terrestrial broadcasters face several challenges in adapting to the new technology. One of the biggest challenges is the need for spectrum. With the introduction of 5G, there is increasing demand forspectrum to support mobile services, which may limit the amount of spectrum available for terrestrial broadcasting. This could result in increased competition for spectrum and higher costs for terrestrial broadcasters.

Another challenge for terrestrial broadcasters is the need for investment in infrastructure. To take advantage of the benefits of 5G, broadcasters will need to

invest in new equipment and infrastructure, such as new transmitters and antennas. This investment can be costly, especially for smaller broadcasters.

Finally, terrestrial broadcasters will need to adapt their business models to take advantage of the new opportunities presented by 5G. This may require new partnerships and collaborations with other industry players, as well as new revenue streams, such as advertising and sponsorships.

# What is Future of TerrestrialBroadcasting in the 5G Era:

The future of terrestrial broadcasting in the 5G era is promising, as this traditional method of broadcasting continues to evolve and adapt to new technologies and changing consumer behaviours. Here are some potential developments in the future of terrestrial broadcasting in the 5G era:

  - Enhanced interactivity: With the higher bandwidth and faster data speeds offered by 5G, terrestrial broadcasting can provide enhanced interactive features and personalized content delivery, such as on-demand services and real-time interactivity.

  - Augmented Reality and Virtual Reality: 5G technology can provide the necessary bandwidth for transmitting high-quality augmented reality and virtual reality content over terrestrial broadcasting, leading to new possibilities for immersive media experiences.

  - Mobile broadcasting: Terrestrial broadcasting can take advantage of the mobility offered by 5G wireless networks to deliver content to mobile devices, such as smart phones and tablets. This allows viewers to access content on-the- go, without relying on a stable internet connection.

  - Improved efficiency: The use of 5G technology in terrestrial broadcasting can also improve efficiency, such as reducing the power consumption of broadcasting equipment and increasing the capacity of transmission.

  - Hybrid broadcasting: In the future, terrestrial broadcasting may also integrate with other broadcasting technologies, such as satellite broadcasting and IP-based streaming, to provide a more flexible and versatile distribution system.

Overall, the future of terrestrial broadcasting in the 5G era is expected to bring new possibilities and opportunities for delivering high-quality, interactive, and personalized content to a wide audience, while maintaining the reliability and wide coverage of this traditional broadcasting method.

# What is 3gpp for terrestrial broadcasting in the 5G era:

3GPP (Third Generation Partnership Project)[1] is a collaboration between groups of telecommunications standards organizations, responsible for the development and maintenance of various mobile communication standards, including those for 5G wireless networks.

In the context of terrestrial broadcasting in the 5G era, 3GPP has developed standards for the delivery of multimedia content over broadcasting networks, known as 5G Broadcast. This standard is designed to enable the efficient distribution of multimedia content, such as live sports, news, and entertainment programs, to a large audience over the 5G network.

The 3GPP 5G Broadcast standard enables content to be delivered over a single frequency network, allowing broadcasters to reach a wide audience with minimal infrastructure. It also provides a range of features, such as dynamic content insertion, which allows broadcasters to personalize content to specific regions or audiences.

The 3GPP 5G Broadcast standard also includes a mechanism for providing emergency warning messages to the public, ensuring that critical information is delivered quickly and efficiently during emergencies or natural disasters.

# Architecture of Terrestrial Broadcasting in 5G Era:

Terrestrial Broadcasting in 5G support in 5G architecture has been detailed in 3GPP technical specification TS 23.247 provides an overview of the major components in the 5GS architecture along with the new functions introduced to support multicast broadcast services. Access and Mobility Management Function (AMF) primarily provides control plane interface towards the NG-RAN to serve the User Equipment’s (UEs). Session Management Function (SMF) establishes the data path between the UEs and the User Plane Function (UPF) to facilitate UE data transfer. Unified Data Management (UDM) facilitates centralized user data management and Policy Control Function (PCF) uses the policy related information to configure policy rules in the 5G network. Network Exposure Function (NEF) allows external Application Functions (AFs) to interact with the 5G core network functions. Listed below are the new functions for supporting Terrestrial Broadcasting in 5G enhancements.[2] [3]

## MulticastSession Management Function (MB-SMF):

MB-SMF primarily supports MBS session management related functionalities along with controlling Quality of Service (QoS) parameters of those sessions. MB-SMF configures the Multicast/Broadcast-UPF (MB-UPF) to apply multicast and broadcast data transport rules based on the policies received from PCF. MB- SMF also interacts with the NG-RAN (via AMF) to establish data transmission resources (in the case of multicast) or to control data transport (in the case of broadcast) for 5GC shared MBS traffic delivery method. It interacts with SMF to modify the existing Protocol Data Unit (PDU) sessions associated with a

multicast session and controls multicast data transport using 5GC individual MBS traffic delivery method. [2] [3]

## Multicast User Plane Function (MB-UPF):

MB-UPF is primarily responsible for the delivery of multicast and broadcast data to NG-RAN nodes using 5GC shared/individual MBS traffic delivery methods. MB-UPF interacts with Multicast/Broadcast Service Transport Function (MBSTF)/AF to receive multicast and broadcast data to be delivered to the UEs. It also ensures QoS enforcement based on the policies configured by the MB- SMF.[2] [3]

- Multicast Service Function (MBSF):MBSF provides service level functionality to support multicast broadcast services and it also facilitates inter working with LTE’s MBMS. It interacts with AF and MB-SMF for MBS session related operations along with selecting a serving MB-SMF for a particular MBS Session. MBSF also controls MBSTF and determines the destination Internet Protocol (IP) multicast address for MBS sessions.[2] [3]

- Multicast Service Transport Function (MBSTF):MBSTF acts as the media anchor for MBS data traffic along with sourcing of IP multicast address if required. It configures generic packet transport functionalities like framing and forward error correction.[2] [3]

- Delivery methods:Following are the two delivery methods defined in 5G MBS architecture: 5GC Shared MBS traffic delivery method: This delivery method is applicable to both multicast and broadcast sessions where a single copy of data packet is delivered to the NG-RAN, which then delivers the data packets to all the UEs in that particular session. Between NG-RAN and UE, MBS data packets can be delivered over the radio using two possible methods.

  - Point-To-Point (PTP) delivery method: In this method separate copy of data packets are delivered to individual UEs by NG-RAN.

  - Point-To-Multipoint (PTM) delivery method: In this method, NG-RAN delivers single copy of data packets to multiple UEs. 5GC Individual MBS traffic delivery method: This delivery method is applicable only for multicast session where MB- UPF receives a single copy of data packet. MBUPF handles the replication of those data packets and those are delivered to individual UEs via their dedicated PDU sessions. So, for each UE, a PDU session is required to be associated with the multicast session.

## Radio aspects of MBS delivery:

In 5G MBS New Radio (NR), broadcasting is applicable to all the available UEs irrespective of their connected state, and acknowledged delivery mode is not applied for the same. However, multicasting can be only made possible for UEs which are in Radio Resource Control (RRC)-active state. Besides, broadcasting of MBS session identifier is required to activate the MBS session. Sublayers for 5G MBS NR. Service Data Adaptation Protocol (SDAP) layer does map of incoming QoS flows into MBS Radio Bearers (MRBs) which can be multicast or broadcast MRBs based on the QoS policies. Protocol Data Convergence Protocol (PDCP) layer performs reordering of the packets and further link these packets to

Radio Link Layer (RLC) entities. RLC layer segments these packets into small sub-packets. There are two types of RLCs, PTM RLC or PTP RLC. PTM RLC is used to send same packet to multiple users with the use of Group Radio Network Temporary Identifier (G-RNTI). PTP RLC uses Cell Radio Network Temporary Identifier (C-RNTI) and supports acknowledged mode of delivery. Further multiplexing/de-multiplexing is performed at the Medium Access Control (MAC) layer.

## Standardization initiatives and limitations of 5G MBS Architecture:

3GPP specifications for multicast/broadcast enhancements in cellular networks are listed in Table I. 3GPP initiated multicast/broadcast service integration by defining group communication system enablers for LTE in TS 23.468. Various options and solutions with respect to architectural changes and procedures for MBS enhancements in 5GS were discussed and presented in 3GPP TR 23.757. The MBS architecture presented is specified in 3GPP TS 23.247.

Specifications on architectural enhancements for 5G MBS started including the aspects of UE-AF application-level details and service announcements mechanisms. In spite of specifications supporting multicast/broadcast enhancements in LTE and 5G architectures, there are certain limitations that need to be addressed to serve upcoming usage requirements. Improving resource efficiency for MBS reception is required along with group event management reporting for MBS sessions. It is also necessary to expose MBS sessions related statistics. To overcome some of these limitations, few work items are initiated and are open for contributions in 3GPP.

# Physical layer OF Terrestrial Broadcasting in 5G Era:

The physical layer of terrestrial broadcasting in the 5G era is expected to be based on the latest version of the DVB-T2 (Digital Video Broadcasting-Terrestrial Second Generation) standard. DVB-T2 is a widely used standard for digital terrestrial television broadcasting and is designed to provide high-quality video and audio services with improved spectral efficiency and robustness.

One of the key features of the DVB-T2 standard is the use of advanced modulation techniques, such as COFDM (Coded Orthogonal Frequency Division Multiplexing) and QAM (Quadrature Amplitude Modulation), which enable higher data rates and improved performance in noisy environments. The standard also supports multiple- input multiple-output (MIMO) technology, which can improve the coverage and capacity of terrestrial broadcasting systems.

In addition to the advanced modulation and MIMO techniques, the physical layer of terrestrial broadcasting in the 5G era is expected to incorporate other technologies, such as advanced error correction coding, adaptive equalization, and interference mitigation techniques, to ensure high-quality and reliable services.

The physical layer of terrestrial broadcasting in the 5G era is expected to be based on the latest version of the DVB-T2 standard, with the incorporation of advanced modulation techniques, MIMO, and other technologies to provide high-quality and reliable services. The use of 5G NR technology can further enhance the physical layer of terrestrial broadcasting by providing improved spectral efficiency and coverage.

# StandardizationinitiativesandlimitationsofTerrestrial Broadcasting in 5G Era:

The standardization of terrestrial broadcasting in the 5G era is being carried out by various organizations such as the International Telecommunication Union (ITU), the European Telecommunications Standards Institute (ETSI), and the Digital Video Broadcasting (DVB) Project. These organizations are working to develop new standards and specifications for broadcasting in the 5G era.

One of the key standardization initiatives for terrestrial broadcasting in the 5G era is the development of the next generation broadcast (NGB) standard, which is being led by the DVB Project. The NGB standard is designed to provide a flexible, efficient, and future-proof platform for broadcasting services in the 5G era. It is expected to support a range of broadcasting services, including linear and non-linear services, and to provide improved spectral efficiency and robustness.

Another standardization initiative for terrestrial broadcasting in the 5G era is the development of the 5G Broadcast (5G-B) standard, which is being led by the ETSI. The 5G-B standard is designed to provide a new broadcast service using 5G technology, which can provide high-quality video and audio services with low latency and high reliability.

## Limitations:

Despite the opportunities presented by the 5G technology, terrestrial broadcasting still faces some limitations in the 5G era. One of the main limitations is the competition from over-the-top (OTT) services, which are becoming increasingly popular among consumers. OTT services offer on-demand content with flexible access and personalized options, which can be challenging for terrestrial broadcasters to compete with.

Another limitation is the availability of spectrum for broadcasting services. As the demand for wireless services continues to grow, there is a risk that the available spectrum for broadcasting services may become limited, which can have an impact on the quality and availability of broadcasting services.

Moreover, terrestrial broadcasting is also facing limitations in terms of the coverage and capacity of broadcasting systems. Although the use of advanced technologies such as MIMO and 5G NR can improve the coverage and capacity of broadcasting systems, there are still challenges in providing high-quality and reliable services in

areas with low population density or difficult terrain.

# Advantages of Terrestrial Broadcasting in 5G Era:

There are several advantages of terrestrial broadcasting in the 5G era, including

  - Wide coverage: Terrestrial broadcasting has the ability to provide wide coverage across large geographic areas, reaching people in both urban and rural areas. This is particularly important in regions where internet connectivity is limited or unreliable.

  - Reliability: Terrestrial broadcasting is less susceptible to disruptions caused by weather, natural disasters or other factors that can affect internet connectivity. It provides a stable and reliable platform for broadcasting critical information, such as emergency alerts.

  - Cost-effective: Terrestrial broadcasting infrastructure is already in place in many areas, and upgrading it to support 5G technology is often less costly than building new infrastructure for wireless networks.

  - Low latency: 5G terrestrial broadcasting can offer low latency, which means that there is minimal delay between when a signal is sent and when it is received. This is especially important for real-time applications such as live sports broadcasts.

  - High-quality video: Terrestrial broadcasting can deliver high-quality video content with better picture and sound quality than many streaming services. This is because broadcasting networks have more bandwidth and are not affected by internet traffic congestion.

  - Energy-efficient: Terrestrial broadcasting networks are designed to be energy- efficient and can operate using low power consumption, making them a more sustainable option than some wireless networks.

# Disadvantages of Terrestrial Broadcasting in 5G Era:

While terrestrial broadcasting has many advantages in the 5G era, there are also some disadvantages to consider, including:

- Limited interactivity: Terrestrial broadcasting is a one-way communication channel, which means that it does not support interactivity or two-way communication. This limits the ability to offer personalized content or to receive feedback from viewers.

- Limited capacity: Terrestrial broadcasting networks have limited capacity, which can make it difficult to deliver large amounts of data, such as high-quality video content. This can result in lower picture and sound quality or longer buffering times.

- Fixed infrastructure: Terrestrial broadcasting networks are based on fixed infrastructure, which means that they cannot be easily adapted to changing needs or user demands. This can make it difficult to keep up with evolving technologies and consumer preferences.

- Regional differences:Terrestrial broadcasting networks may vary in terms of coverage and availability depending on geographic location. This can result in unequal access to information and entertainment for people in different regions.

- Spectrum allocation: The allocation of spectrum for terrestrial broadcasting networks can be a complex and politically charged issue, with limited spectrum available for broadcasting in some regions. This can limit the expansion of terrestrial broadcasting services and lead to increased competition for available spectrum.

- Vulnerability to interference: Terrestrial broadcasting signals can be vulnerable to interference from other sources, such as other broadcast signals, electrical equipment, or weather conditions. This can result in signal disruptions or reduced coverage in certain areas.

# Specifications for Terrestrial Broadcasting in 5G Era:

Some of the key technical specifications of Terrestrial Broadcasting in 5 Era:

  - Frequency Bands: 5G broadcast services can be delivered over various frequency bands, including sub-6 GHz and millimetre-wave (mmWave) bands. The choice of frequency band depends on the specific use case, network architecture, and local regulations.

  - Modulation Schemes: 5G broadcast services can use various modulation schemes, such as 16-QAM, 64-QAM, and 256-QAM, to encode the multimedia content for transmission over the network.

  - Core Network Architecture: The core network architecture for Terrestrial Broadcasting in 5G Era services includes a broadcast/multicast service centre (BM-SC), which is responsible for managing the delivery of broadcast/multicast content over the network.

  - Media Delivery Methods: 5G broadcast services can use various media delivery methods, such as File Delivery over Unidirectional Transport (FLUTE), MPEG-2 Transport Stream (TS), and MPEG-DASH, to deliver multimedia content to end- users.

  - Latency: 5G broadcast services are designed to have low latency, typically between 10 and 100 milliseconds, to enable real-time multimedia experiences such as live streaming of sports events or concerts.

  - Quality of Service (QoS): Terrestrial Broadcasting in 5G Era services support various QoS levels, including best-effort and guaranteed bit rate, to ensure that the multimedia content is delivered with the appropriate level of quality and reliability.

  - Security: Terrestrial Broadcasting in 5G Era services are designed with various security features, such as encryption and authentication, to ensure the privacy and integrity of the multimedia content and the network.

# Used case of Terrestrial broadcasting in the 5G era:

There are several potential use cases for terrestrial broadcasting in the 5G era, some of which are:

- Live sports events: Terrestrial broadcasting in the 5G era can be used to deliver live sports events to a large audience in real-time. The high bandwidth and low latency of 5G technology can provide a high-quality viewing experience, with interactive features such as multiple camera angles, replays, and statistics.

- News broadcasts: Terrestrial broadcasting in the 5G era can also be used for delivering live news broadcasts to a large audience, with the ability to personalize content based on regional or audience preferences. The 5G network can also provide a reliable means of delivering emergency warnings and public safety messages.

- Entertainment programs: Terrestrial broadcasting in the 5G era can be used for delivering a range of entertainment programs, such as music concerts, cultural events, and movie premieres. The high bandwidth and low latency of 5G technology can provide a high-quality viewing experience, with interactive features such as voting, chat, and social media integration.

- Education and training: Terrestrial broadcasting in the 5G era can also be used for delivering educational and training programs to a wide audience, such as distance learning programs, corporate training, and professional development courses.

- Advertising: Terrestrial broadcasting in the 5G era can be used for delivering targeted advertising to a wide audience, with the ability to personalize ads based on user preferences and behaviours.

# Companies Testing Broadcasting in 5G Era:

Tests Terrestrial Broadcasting in 5 Era emissions at ISE and MWCPilot project to watch DTT on mobile without consuming data.

The project is part of the company’s commitment to the European audio-visual innovation roadmap The contents of the test broadcasts that can be accessed via mobile include RTVE’s Canal 24h, La1 and Radio5.Rohde Schwarz, Atama and Qualcomm Technologies are also taking part in the project. Barcelona, 3 February 2023. Cellnex Telecom has launched a pilot to test “5G Broadcast” emissions together with Rohde Schwarz, Atama and Qualcomm Technologies. Terrestrial Broadcasting in 5G Era will allow viewers to receive live TV and radio content on their mobile phones with low battery consumption, without the need to consume data and high-quality network coverage.

The tests, which began in mid-January, were performed during the ISE international audiovisual fair and will end after the MWC Barcelona 2023 to be held at Firas Gran Via de Hospitalet from 27 February until 2 March. In this way, users can access the contents through their mobile devices compatible with the Terrestrial Broadcasting in 5 Era signal.

(Service) was compared with the current transmission standard for terrestrial television, DVB-T2. In cooperation with the ORS, the Vienna University of Technology (TU Wien) expanded their link-level simulator for FeMBMS capabilities. Simulations with a multitude of parameters were performed to get data rates and reception quality for different coverage situations. The ORS compared the simulation results with the results from stationary and mobile field measurements.

In summer 2020, ORS measuring vehicles had been out and about in Vienna to simulate different walking and driving speeds and to test the reception quality of the signal. Transmission of new media formats, such as HTTP Live Streaming (HLS), and simultaneous processing of mobile communication, Wi-Fi and FeMBMS data on mobile devices was also studied.

Second test phase tests the interplay between broadcasting and mobile communications.

## SaankhyaLabs:

Saankhya Labs gets 5G trial spectrum, ties up with US broadcaster Sinclair for live TV services on mobile phones Saankhya’s core BRH product, which is already certified by US Federal Communications Commission (FCC), enables convergence of broadcast and mobile broadband networks, allowing digital terrestrial broadcasters to boost their reach and, potentially, market share. Kolkata: Homegrown wireless communication and semiconductor chipset company Saankhya Labs has received Terrestrial Broadcasting in 5G Era trial spectrum from the telecom department to test in house products that enable convergence of broadcast and mobile broadband networks before launching them in the US.

## AT&T, LiveU test 5G HEVC mobile broadcasting:

AT&T and Live U will be testing out 5G-enabled features at AT&T’s Foundry AT&T, Live U test 5G HEVC mobile broadcasting | Fierce VideoBy teaming up with Live U to trial 5G with their portable HEVC broadcast units, they will learn how we can help broadcasters increase production capabilities and expand possibilities for future innovations - as well as help them create compelling new engagement models with consumers. Just imagine the possibility of broadcasters being able to live stream breaking news in 8K or in an AR/VR 360 format over mobile 5G, said Mo Kibbeh, chief marketing officer at AT&T Business, in a statement.

- The ORS started a Terrestrial Broadcasting in 5G Era trial in Vienna in 2020:In order to establish Terrestrial Broadcasting in 5G Era in Austria, the ORS started a trial in Vienna at the beginning of 2020. The new transmission standard FeMBMS will be studied over the next few years in two phases using two high-tower-high- power transmitters located at Kahl Enberg and Lie sing, Vienna.

## First trial phase compares DVB-T2 with Terrestrial Broadcasting in 5G Era

In the first phase until Q2 2021, the mobile transmission technology FeMBMS (Further evolved Multimedia Broadcast Multicast applications including hybrid services with interaction between broadcast network operators and mobile network operators as well as the distribution of disaster and crisis information together with collaboration partners.

The ORS extends an open invitation to interested parties who would like to take part in the second phase of the Terrestrial Broadcasting in 5G Era test: "To achieve the best possible results in the test phase we need collaboration and joint testing of the new digital television standard. The ORS is delighted to have keen participants and questions are welcome", says ORS Project Manager Dr. Johann Mika.

# Standards available for Terrestrial Broadcasting in 5G Era:

According to Report REPORT ITU-R BT.2295-3 - Digital terrestrial broadcasting systems. These are some standards available currently

## ATSC3.0

According to REPORT ITU-R BT.2295-3 - Digital terrestrial broadcasting systems and Home - ATSC : NextGen TV

ATSC 3.0 is a next-generation broadcasting standard for terrestrial television, developed by the Advanced Television Systems Committee (ATSC). This standard is designed to provide broadcasters with more flexibility, efficiency, and interactivity in delivering high-quality audio and video content to viewers over the air.

ATSC 3.0 includes several features that improve upon the previous ATSC 1.0 standard, such as:

## Improved video and audio quality:

ATSC 3.0 uses the latest video and audio coding technologies, such as HEVC and Dolby AC-4, to provide higher resolution and improved sound quality.

## Enhanced mobility:

ATSC 3.0 is designed to work with mobile devices, such as smartphones and tablets, allowing viewers to watch live television on-the-go.

## Personalization:

ATSC 3.0 includes features for delivering personalized content to viewers, such as targeted advertising and interactive programming.

## Interactivity:

ATSC 3.0 supports interactive applications and services, such as voting, gaming, and social media integration, providing a more engaging viewing experience.

## Emergency alerts:

ATSC 3.0 includes improved emergency alerting capabilities, allowing broadcasters to deliver critical information to viewers in a timely and efficient manner.

ATSC 3.0 provides broadcasters with a more efficient and flexible means of delivering high-quality audio and video content to viewers over the air. It also provides viewers with a more engaging and personalized viewing experience, with enhanced interactivity and mobility features.

## DABsystem

DAB system, also  known as the  Eureka 147 digital audio broadcasting  (DAB) system,	has

beendevelopedforbothsatelliteandterrestrialbroadcastingapplicationsinordertoallown commonlow-cost receiver to be used. The system has been designed to provide vehicular, portable and fixedreception with low gain omni-directional receive antennas located at 1.5 m above ground. DABsystem allows for complementary use of satellite and terrestrial broadcast transmitters resulting inbetter spectrum efficiency	and	higher	service	availability	in	all	receiving	situations.	It especiallyoffers improved performance in multipath and shadowing environments which		are		typical	of	urbanreceptionconditionsbytheuseofon- channelterrestrialrepeatersto services gap-fillers.DAB system capable of offering various levels of sound quality up to high quality sound comparable to thatobtained from consumer digital recorded media. It can also offer various data services and differentlevels of conditional access and the capability of dynamically re-arranging the various servicescontainedin themultiplex.

## DRMsystem

Digital Radio Module (DRM) is a digital radio broadcasting system that delivers audio content over terrestrial radio networks using digital audio compression techniques. In the context of the 5G era, DRM can complement terrestrial television

broadcasting, providing a means of delivering high-quality audio content to listeners over the air.

DRM has several advantages over traditional analog radio broadcasting, including:

## Improved audio quality:

DRM uses advanced audio compression techniques, such as AAC+ audio coding, to deliver higher-quality audio than traditional analog radio broadcasting.

## More efficient use of spectrum:

DRM uses a more efficient coding scheme than analog radio broadcasting, allowing more channels to be broadcast over the same spectrum.

## More reliable reception:

DRM uses error correction techniques to ensure more reliable reception, reducing the impact of signal interference and multipath distortion.

## More interactivity:

DRM supports a range of interactive features, such as scrolling text, song and artist information, and program guides.

## Better coverage:

DRM can provide better coverage than analog radio broadcasting, as digital signals are less affected by terrain and other environmental factors.

In the 5G era, DRM can work alongside other broadcasting technologies, such as terrestrialtelevision broadcasting, to provide a more comprehensive multimedia experience for viewers and listeners. DRM can also complement mobile networks, providing a means of delivering high-quality audio content to mobile devices over the air. Additionally, DRM can support a range of digital rights management (DRM) systems, enabling broadcasters to protect their content from unauthorized copying and distribution.

## DTMB-Asystems

DTMB-A (Digital Terrestrial Multimedia Broadcast - Audio) is a digital television broadcasting system developed in China. In the context of the 5G era, DTMB-A can complement terrestrial television broadcasting, providing a means of delivering high-quality audio and video content to viewers over the air.

DTMB-A has several advantages over traditional analog television broadcasting, including:

Improved audio and video quality: DTMB-A uses advanced audio and video coding technologies, such as MPEG-4 and Dolby Digital Plus, to deliver higher-quality audio and video than traditional analog television broadcasting.

More efficient use of spectrum: DTMB-A uses a more efficient coding scheme than analog television broadcasting, allowing more channels to be broadcast over the same spectrum.

More reliable reception: DTMB-A uses error correction techniques to ensure more

reliable reception, reducing the impact of signal interference and multipath distortion.

More interactivity: DTMB-A supports a range of interactive features, such as electronic program guides, on-screen messaging, and interactive advertising.

Better coverage: DTMB-A can provide better coverage than analog television broadcasting, as digital signals are less affected by terrain and other environmental factors.

In the 5G era, DTMB-A can work alongside other broadcasting technologies, such as DAB and DRM, to provide a more comprehensive multimedia experience for viewers. DTMB-A can also complement mobile networks, providing a means of delivering high-quality audio and video content  to mobile devices over the air. Additionally, DTMB-A can support a range of digital rights management (DRM) systems, enabling broadcasters to protect their content from unauthorized copying and distribution.

## DVB-T2system

DVB-T2 (Digital Video Broadcasting - Second Generation Terrestrial) is a digital television broadcasting system that delivers high-quality audio and video content over terrestrial radio networks. In the context of the 5G era, DVB-T2 can complement terrestrial television broadcasting, providing a means of delivering high-quality audio and video content to viewers over the air.

DVB-T2 has several advantages over its predecessor DVB-T and traditional analog television broadcasting, including:

Improved audio and video quality: DVB-T2 uses advanced audio and video coding technologies, such as HEVC (High-Efficiency Video Coding) and MPEG-H Audio, to deliver higher-quality audio and video than DVB-T and traditional analog television broadcasting.

More efficient use of spectrum: DVB-T2 uses a more efficient coding scheme than DVB-T and traditional analog television broadcasting, allowing more channels to be broadcast over the same spectrum.

More reliable reception: DVB-T2 uses error correction techniques and advanced modulation schemes, such as 256-QAM, to ensure more reliable reception, reducing the impact of signal interference and multipath distortion.

More interactivity: DVB-T2 supports a range of interactive features, such as electronic program guides, on-screen messaging, and interactive advertising.

Better coverage: DVB-T2 can provide better coverage than traditional analog television broadcasting, as digital signals are less affected by terrain and other environmental factors.

In the 5G era, DVB-T2 can work alongside other broadcasting technologies, such as DAB and DRM, to provide a more comprehensive multimedia experience for viewers. DVB-T2 can also complement mobile networks, providing a means of delivering high-quality audio and video content  to mobile devices over the air. Additionally, DVB-T2 can support a range of digital rights management (DRM)

systems, enabling broadcasters to protect their content from unauthorized copying and distribution.

## IBOC system

In-band on-channel (IBOC) digital sound broadcasting (DSB) system was designed to provide vehicular, portable and fixed reception using terrestrial transmitters. Although IBOC system can be implemented in unoccupied spectrum, a significant feature of the system is its ability to offer simulating of analogue and digital signals in the existing AM and FM sound broadcasting bands.

The system offers improved performance in multipath environments resulting in greater reliabilitythan is offered by existing analogue AM and FM operations. IBOC system (also called HD Radiosystem) offers enhanced audio quality comparable to that obtained from consumer digital recordedmedia, new data-casting services in addition to the enhanced audio programming. In addition, thesystem allows for allocation of bits betweenaudio and data-casting capacity to maximize thedata- castingcapabilities.

## ISDB-Tsystemsfamily

ISDB-T (Integrated Services Digital Broadcasting-Terrestrial) is a digital terrestrial television broadcasting system that was developed by Japan. While it is not specifically designed for the 5G era, it can still be used in conjunction with 5G networks to provide high-quality television broadcasting.

ISDB-T is designed to provide high-quality digital broadcasting signals with better resistance to interference compared to analog TV signals. It uses advanced modulation techniques and error correction to ensure that the transmitted signal is received accurately by the receiver.

Although 5G networks have the potential to deliver video content over the internet, terrestrial broadcasting systems like ISDB-T still have advantages in terms of reliability, scalability, and cost-effectiveness. With the growing demand for high- quality video content, the use of ISDB-T is likely to continue in the 5G era as an effective method of delivering digital television content to viewers.

## T-DMB,AT-DMBsystems

Terrestrialdigitalmultimediabroadcasting(T- DMB)systemistheextendedsystemcompatiblewithdigital sound broadcasting system T-DAB, which enables video services using T-DAB networks forehandheld receivers in   a   mobile   environment.   T-DMB	provides multimedia services including video,audio,and interactivedata.

AT-DMB is the extended system of guaranteeing backward compatibility with T- DMB, whichincreases channel capacity of T-DMB by applying hierarchical modulation mechanism. AT-DMBprovidesascallable videoservice as well as allkinds of T-DMB services provides general system characteristics forT-DMBand AT- DMB(Multimedia systemA)formobilereception byhandheld receivers.

# Characteristics of Terrestrial Broadcasting in 5G Era:

These are some characteristicsof standards available currentlyof Terrestrial Broadcasting in 5G Era :

| Characteri stics | ATSC 3.0 | DAB | DRM | DTMB- A | DVB- T2 | IBOC | ISDB- T family | T-DMB, AT- DMB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Net data rates | Depend ing on modulat ion and code rate a)0.93- 57.9Mb it/s b)1.08- 67.5 Mbit/s c)1.24- 77.2 Mbit/s | 0.576 to 1.728 Mbit/s | Depend ing on modulat ion , code rate, robustn ess mode, channel bandwi dth: 1)Mode s A , B,C,D- 5- 72Kbit/ s 2)Mode E-37- 186 Kbit/s | Depend ing on modulat ion code and frame header: a)3.75- 38 Mbit/s b)4.38- 44.4 Mbit/s c)5.0- 50.73 Mbit/s | 7.5- 50.5 Mbit/s | Dependi ng on band, modulat ion, Code rate: a)(MF)2 0.4 to 40.4 kbit/s b)(VHF Hybrid) 51 to 149kbit/ s c)(VHF all Digital) 51 to 280 kbit/s | N* a)0.28 1 to 1.787 Mbit/s b)0.32 8 to 2.085 Mbit/s c)0.37 4 to 2.383 Mbit/s | T- DMB:0. 576 TO 1.728 Mbit/s AT- DMB:0. 864 TO 2.304 Mbit/s at BPSK over DQPSK AT- DMB:1. 152 to 2.88 Mbit/s at QPSK over DQPSK |
| Single frequency network | Support ed | Suppor ted | Support ed | Support ed | Suppor ted | Support ed | Suppor ted | Support ed |

| Characte ristics | ATSC3.0 | DAB | DRM | DTMB-A | DVB-T2 | IBOC | ISDB- Tfamily | T- DMB, AT- DMB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OuterFEC | None, | – | – | BCH(3072 0, 3051 2)for LDP C1/2 BCH(4096 0, 4075 2)for LDP C2/3 BCH(5120 0, 5099 2)for LDP C5/6 | BCH(1620 0,x, t), there x – depends onLDPC code rate. Error correcti oncapab ility t= 12errors | RS(96,88, | RS(204, | RS(204, |
|  | CRC |  |  |  |  | 4) | 188, | 188, |
|  | (32- bit),orBC H. BCH (16 200, x,t)orBC H(64800, |  |  |  |  | RS(255,2 23, 16) RS(255,1 91, 32) | T=8) | T = 8) code forvideo servicea ndscala blevide oservice |
|  | x,t),there |  |  |  |  |  |  |  |
|  | x – |  |  |  |  |  |  |  |
|  | depends |  |  |  |  |  |  |  |
|  | onLDPC |  |  |  |  |  |  |  |
|  | coderate. |  |  |  |  |  |  |  |
|  | Errorcor |  |  |  |  |  |  |  |
|  | rectionc |  |  |  |  |  |  |  |
|  | apability |  |  |  |  |  |  |  |
|  | uptot= |  |  |  |  |  |  |  |
|  | 12 errors |  |  |  |  |  |  |  |
| Outerinter | Bit | – | – | Symbol interleavin g240 rowsand 4096colum ns | Bit(parity | Upto64 | Byte- | Convolu |
| leaving | (parit |  |  |  | &columnt | codew | wiseco | tionalint |
|  | y,gro |  |  |  | wist)interle | ords | nvoluti | erleavin |
|  | up- |  |  |  | aving |  | onalint | gforvide |
|  | wise, |  |  |  |  |  | erleavi | oservice |
|  | block |  |  |  |  |  | ng,I=1 | andscala |
|  | )interl |  |  |  |  |  | 2 | blevideo |
|  | eavin |  |  |  |  |  |  | service |
|  | g |  |  |  |  |  |  |  |
| Dataran | 16bitPRB | 16bitP | 16bitPR | PRBS | 16bitPRBS | 12bitPRB | PRBS | 16bitPR |
| domizati | S | RBS | BS |  |  | S |  | BS |
| on/energ |  |  |  |  |  |  |  |  |
| ydispers |  |  |  |  |  |  |  |  |
| al |  |  |  |  |  |  |  |  |
| Trans | Boots | Phas | Controlc | Service | Preamblesy | 32 | TMC | Phase |
| missio | trap,P | erefe | ells | channelsig | mbolP1 | bitinform | C | referenc |
| npara | ream | renc |  | nalling is |  | ationsequ | pilotca | esymbol |
| meters | blesy | esy |  | carriedbyc |  | enceprovi | rriers |  |
| ignalli | mbol, | mbo |  | ontrolchan |  | dedcyclic |  |  |
| ng | L1sig | l |  | nelinthesup |  | ally |  |  |
|  | nallin |  |  | erframe |  |  |  |  |
|  | g |  |  |  |  |  |  |  |

# Conclusion:

In conclusion, 5G technology presents both challenges and opportunities for terrestrial broadcasting. While the need for spectrum and investment in infrastructure may pose challenges for broadcasters, 5G also offers new technical capabilities and the potential for new revenue streams. To take advantage of these opportunities.

REFRENCES-

- REPORT ITU-R BT.2295-3 - Digital terrestrial broadcasting systems

- A Survey on Multicast Broadcast Services in 5G and Beyond | IEEE Conference Publication | IEEE Xplore

- 5G Technology for Broadcast TV Applications | IEEE Conference Publication | IEEE Xplore

- Realising a vRAN based FeMBMS Management and Orchestration Framework | IEEE Conference Publication | IEEE Xplore

- A/300, “ATSC 3.0 System” - ATSC : NextGen TV

- A/321, "System Discovery and Signaling" (atsc.org)

- AT&T, LiveU test 5G HEVC mobile broadcasting | Fierce Video

- Evaluation of ATSC 3.0 and 3GPP Rel-17 5G Broadcasting Systems for Mobile Handheld Applications | IEEE Journals & Magazine | IEEE Xplore

- PowerPoint-Präsentation (itu.int)

- MBSFN or SC-PTM: How to Efficiently Multicast/Broadcast | IEEE Journals & Magazine | IEEE Xplore

- Scenarios for 5G networks: The COHERENT approach | IEEE Conference Publication | IEEE Xplore

- Handbook on Digital Terrestrial Television Broadcasting networks and systems implementation (itu.int)

- Transition from analogue to digital terrestrial television broadcasting (itu.int)

- Characteristics of digital terrestrial television broadcasting systems in the frequency band 470-862 MHz for frequency sharing/interference analyses (itu.int)

- 3GPP TR 23.757 – Study on architectural enhancements for 5G multicast-broadcast services – TechSpec (itecspec.com)

- ISDB-T System | DiBEG

- DVB - The global standard for digital television - Home

- Digital Radio Mondiale | The future of global radio (drm.org)

- WorldDAB

- Report ITU-R BT.2383-0

- REPORT ITU-R BT.2295-3 - Digital terrestrial broadcasting systems

- http://www.ors.at/en/

- Driving sustainable telecom connectivity - Cellnex