|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG11-XX |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 11 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 15/11 |  | Geneva, 10-20 October 2023 |  |
| CONTRIBUTION |  |  |  |  |  |
| Source: |  | Ministry of Communications, India |  |  |  |
| Title: |  | Proposal for advancement in the baseline text of draft new Technical Report TR-CF-QoS “Impact of Counterfeit Mobile devices on Quality of Service”. |  |  |  |
| Contact: |  | Arjun Singh Ministry of Communications, India |  |  | Tel: +91-9267933988 E-mail: arjun.singh93@gov.in |
| Abstract: |  | This contribution proposes advancement in the baseline text of draft new Technical Report TR-CF-QoS “Impact of Counterfeit Mobile devices on Quality of Service” (Geneva, 6-15 July 2022). The contribution proposes to include following clauses: i) Addition of clause 6.5 for Network Coverage. ii) Addition of clause 6.6 for Security Vulnerability. |  |  |  |

Draft new Technical Report ITU-T TR-CF-QoS

Impact of Counterfeit Mobile devices on Quality of Service

Summary

According to ITU-T Rec E. 800 Quality of Service is defined as the totality of characteristics of a telecommunications service that bear on its ability to satisfy stated and implied needs of the user of the service. World Trade Organization - WTO also defines counterfeit as the unauthorized representation of a registered trademark carried on goods identical or similar to goods for which the trademark is registered, with a view to deceiving the purchaser into believing that he/she is buying the original goods. Counterfeit mobile devices pose QoS delivery issues. Studies conducted have shown that such mobile devices suffered from low call completion rates, high call drop rates and handover failures. The counterfeit mobile devices can impact the network performance due to their low call completion rates and frequent call drops which in turn can lead to network congestion and reduced speed for the legitimate users. A survey conducted in the Africa region revealed that all categories of mobile phones were perceived to be the most counterfeited ICT devices in the region. This technical report will outline the challenges and impact of counterfeit mobile devices on Quality of Service for the users and networks.

Keywords

Counterfeiting, Quality of Service, Quality of Experience, Impact

Change Log

This document contains Version X of the draft ITU-T Technical Report on “Impact of Counterfeit Mobile devices on Quality of Service” created at the ITU-T Study Group 11 meeting held in Geneva, March, 2019.

CONTENTS

Page

1 Scope 4 2 References 4 3 Terms and definitions 4 3.1 Terms defined elsewhere 4 3.2 Terms defined here 5 4 Abbreviations 5 5 Introduction 5 Error! Bookmark not defined. Error! Bookmark not defined. Error! Bookmark not defined. Error! Bookmark not defined. Error! Bookmark not defined. Error! Bookmark not defined. 6 Impact of Counterfeit – Technical: 6 7 Conclusions 9 1 Nigeria experience on the impact of counterfeiting on quality of service 100 1.1 Overview of the situation 10 1.2 Data on the impact of CF on QoE/QoS on Nigeria 10 1.3 Conclusion 10

Draft new Technical Report ITU-T TR-CF-QoS

Impact of Counterfeit Mobile devices on Quality of Service

Summary

According to ITU-T Rec E. 800 Quality of Service is defined as the totality of characteristics of a telecommunications service that bear on its ability to satisfy stated and implied needs of the user of the service.

World Trade Organization - WTO also defines counterfeit as the unauthorized representation of a registered trademark carried on goods identical or similar to goods for which the trademark is registered, with a view to deceiving the purchaser into believing that he/she is buying the original goods.

Counterfeit mobile devices pose QoS delivery issues. The ITU-T SR-AFR “Survey report on counterfeit ICT devices in Africa region” revealed that all categories of mobile phones were perceived to be the most counterfeited ICT devices in the region. Additionally, the same survey pointed that counterfeit ICT devices suffered from low call completion rates, high call drop rates and handover failures.

This technical report will outline the challenges and impact of counterfeit mobile devices on Quality of Service for the users and networks.

# Scope

This technical report contains a study on negative effects and impact of counterfeit mobile devices on network’s Quality of Service along with the negative effects and service degradation experienced by the mobile subscribers.

# References

[ITU-T Q.5050]	Recommendation ITU-T Q.5050 (2019), Framework for solution to combat counterfeit ICT Devices.

[ITU-T Q.5051]	Recommendation ITU-T Q.5051 (2020), Framework for combating the use of stolen mobile devices.

[ITU-T Q.5052]	Recommendation ITU-T Q.5052 (2020), Addressing mobile devices with duplicate unique identifier.

[ITU-T E.807]	Definitions, associated measurement methods and guidance targets of user-centric parameters for call handling in cellular mobile voice service

[ITU-T K.114]	Electromagnetic compatibility requirements and measurement methods for digital cellular mobile communication base station equipment

[ITU-T SR-AFR]	Survey report on counterfeit ICT devices in Africa region

# Terms and definitions

## Terms defined elsewhere

This Technical Report uses the following terms defined elsewhere:

3.1.1	drop rate or voice service cut-off ratio: [ITU-T E.807]: The probability of a call terminating without the user's action.

3.1.2	Throughput [ITU-T K.114]: The number of payload bits successfully received per second for a reference measurement channel in a specified reference condition.

3.1.3 Quality of Experience (QoE) [ITU-T P.915]: The degree of satisfaction of the user of an application or service. It results from the fulfilment of his or her expectations with respect to the utility or enjoyment of the application or service in the light of the user's personality and current state.

3.1.4 Quality of Service (QoS) [ITU-T E.800]: Totality of characteristics of a telecommunications service that bear on its ability to satisfy stated and implied needs of the user of the service.

3.1.5 Counterfeit ICT Devices [ITU-T Q.5050]: A counterfeit ICT device is a product that explicitly infringes the trademark, copies hardware or software designs, or infringes brand or packaging rights of an original or authentic product and, in general, infringes applicable national and/or international technical standards, regulatory requirements or conformity processes, manufacturing licensing agreements, or other applicable legal requirements.

## Terms defined here

This Technical Report defines the following terms:

### Substandard Device: A substandard device is a product that has lower quality than a branded product and is not submitted to conformance tests with any type of approval and compliance.

# Abbreviations and Acronyms

This Technical Report uses the following abbreviations and acronyms:

MWF 	Mobile Wireless Forum

QoS		Quality of Service

QoE		Quality of Experience

WTO 	World Trade Organization

# Introduction

The rise of counterfeit products can potentially cause businesses to experience an increase in costs and loss of productivity. Most importantly, counterfeiting can negatively impact a company's reputation and may cost millions in revenue losses. Counterfeiting has a damaging effect on business, the economy and the general population.

World Trade Organization - WTO defines counterfeit as the unauthorized representation of a registered trademark carried on goods identical or similar to goods for which the trademark is registered, with a view to deceiving the purchaser into believing that he/she is buying the original goods. Counterfeit mobile devices may impact on Quality of Service.

The ITU-T SR-AFR “Survey report on counterfeit ICT devices in Africa region” revealed that all categories of mobile phones were perceived to be the most counterfeited ICT devices in the region. Additionally, the same survey pointed that counterfeit ICT devices suffered from low call completion rates, high call drop rates and handover failures.

This technical report will outline the challenges and impact of counterfeit mobile devices on Quality of Service for the users and networks.

# Impact of Counterfeit related to QoS/QoE:

Counterfeit devices on the network may affect the following Key Performance Indicators of the Networks:

## Call Drop Rate:

A call may drop for different reasons – however for counterfeit devices using retainability tests, it has been found that the failure due to counterfeit devices are higher when compared to those of genuine phones. Explanations for this behaviour are traceable to elements that impact retainability such as:

  - MS transmit power;

  - Phase frequency error;

  - UL power control time response;

  - Delayed handover and;

  - Device sensitivity.

Note that generally, networks are planned with standard device sensitivity, counterfeit devices especially as we approach the coverage edge are disadvantaged. Given also that they are low sensitivity phones, their overall interference become worst due to the increase of average transmission power to maintain calls – as such they are more susceptible to drop the radio link or experience call blocking. This increased interference also impact the overall noise floor and this can bear negative impact on the overall QoS for especially networks where power is the ultimate resource.

[For example, Studies from one operator network records 77% of counterfeit device-related voices quality of service degradation as being due to “Call reject” failure code. The “Call Reject” failure “happens when the terminal equipment (mobile device) is having issues accepting calls. From the exchange standpoint, the call had been cleared and is successful, because the terminal equipment (mobile device) was neither occupied nor was it incompatible.” Hence this sort of blocking which is commonplace due to a supplementary service constraint flagged as 403 forbidden as a cause code to request failure response.]

## Call Setup:

Generally, accessibility tests show that in areas of poor radio frequency conditions, that counterfeit devices showed no service whereas this was not the case with original devices. This reflected that the sensitivity on such counterfeit devices was worse than those of original phones. This is one of the reasons why counterfeit devices are responsible for call setup issues. For some of the trace monitored failures, the call setup failures are captured to most times be due to bearer capacity. It is either the device does not have the reported capability or it does not have the authority to use such capability due to its cloned nature.

Failure code most often associated with this failure is a “Bearer capacity not authorized” which represents a failure which happens when the mobile station requested a bearer capability that it is not authorized to use.

Using trace records from network A shows a typical case of initial context setup failure for a LTE network. In each case the culprit device had an IMEI that was not found in the type allocation code (TAC) database.

Trace File 1 - Total 35 Drop captured in trace file , 91% of the failures were traceable to  UEs two devices – *IMEI altered for privacy.

Trace File 2 - Total 45 Drop captured in trace file, 87% failure due to only one device:

Trace File 3 - Total 65 Drop captured in trace file, one device responsible for 89%

[Editor’s note: The picture should be revised so we can read it better].

[In the case of failures associated with devices with IMEI not found in the TAC registry, after 1.5 sec of InitialContextSetupRequest, the e-nodeB receives a UEContextReleaseCommand from MME due to no response from UE/ENB. In terms of QoS, the initial Context Setup Failure results “if the security configuration on the eNB is not supported by the UE or sometimes it can happen if the UE cannot handle both the Security Mode Command and the RRC Connection Reconfiguration together” – in this case, the issue was with the counterfeit device.]

The coverage in a mobile network is defined by the maximum allowed path loss (MAPL) in both the uplink and downlink. Counterfeit devices with their substandard receiver sensitivity reduce the DL MAPL. This reduction which is also seen in the uplink MAPL as a result of the phone not being able to set the maximum transmitter output power defined in the 3GPP standards requires the operator to deploy additional network investment to cover for the holes due to these phones.

## Hand Over Rate:

Handover failure rate with counterfeit devices are higher and this can be traced to the longer execution time it takes for the typical counterfeit phone to complete its handover [b-MWF-Publication]. Flowing from the receiver sensitivity of counterfeit mobile devices, it follows that as their maximum allowable path loss reduces due to their low sensitivity, their ability to retain radio links coupled with their extended handover time, is a recipe for increased handover failure and ultimately drop calls.

Against results from RF functional test which evaluates the quality of the engine machine inside phone terminals which puts counterfeit phones as worse off – it follows that service reduction in terms of handover will be perceived by customers with such devices.

## Throughput:

With the advent of smartphones, increased adoption and improvement in network capabilities, the preponderance of traffic on mobile networks has transformed from voice to data. This growth has also exposed a fundamental challenge with the sort of scheduling deployed by networks to ensure fairness among users. In the downlink of networks, the proportional fairness (PF) scheduling mechanism is used to ensure this fairness. Proportional fairness schedulers depend on honest and a priori trust on feedback provided by the mobile station (MS) or User Equipment (UE). These feedback forms the basis for scheduling the mobile device or providing network resources for its utilization. The network performs such scheduling decisions dependent on feedback reports from the channel quality (CQI) and the automatic repeat request (ARQs). Counterfeit / malicious phones explore this trust in a manner that ultimately impacts on the network quality of service. Observations for mobiles in this category, is such that they are either providing:

- erroneous channel quality feedback or

- misreporting of the packet transmission success.

The vulnerability associated with the network scheduler blindly trusting the feedback from the UEs with regards the downlink quality and transmissions is a security backdoor that allows producers of these counterfeit phones to fake CQI or ARQ reports that in many instances come in different forms

- correct CQI - fake ARQ – the most severe case;

- fake CQI and fake ARQ – also very severe impact;

- fake CQI - correct ARQ – mild impact.

The critical impact of these false reports is that, the scheduler trusting on these reports / feedback provides the affected UEs with increased service time / an unfair share of time slots. This starves well behaved UEs of this resource; whereas the counterfeit devices in receipt of either higher transmission block size or very large fraction of time slots than it can actually sustain in the downlink, leads to packet error and waste of scarce resource. Thus the culprit device does not enjoy the expected higher throughput while at the same time, it denies other users of access to same, as such impacting the QoS of the network. These wrong device settings are counterproductive on the network.

## Network Coverage:

The range of the network coverage is restricted by minimum received power and maximum transmitted power of mobile device:

- Minimum Received Power: The mobile devices need a strong enough signal from the tower for a reliable connection.

- Maximum Transmission Power: When mobile phones transmit excessive power, there is a risk that they can create interference with other devices. Conversely, if they transmit insufficient power, their own service quality may be compromised.

## Note1: Studies in [b-MWF-Publication] and [b-IEEE-Report] have revealed that received signal power and transmit power of the counterfeit/substandard mobile devices have been reduced significantly when compared to genuine mobile devices.

## Note2: This reduction in received and transmitted powers leads to reduction in the maximum achievable distance between the antenna tower and the counterfeit/substandard mobile phones to sustain a connection, particularly for voice calls.

## Security Vulnerability:

Counterfeiting mobile devices may not have proper security features and software updates, making them more susceptible to viruses, malwares, and other security threats. These devices can significantly impact the networks performance due to various attacks and can disrupt the service availability to the genuine mobile phone users.

# Conclusions

There’s evidence that the use counterfeit devices lower the overall quality of service (QoS) and causes a negative impact on mobile networks. In the report [b-IEEE-Report], it’s shown results of tests, using 3GPP specification [TS51.010-1], of substandard and counterfeit phones along with original devices. The results of the tests provided findings that show that substandard/counterfeit devices presented a lower sensitivity, lower transmit power, higher absolute frequency error and longer handover time when compared to original devices. Those poor performance characteristics have several impacts on the mobile network as described in section 6.

Annex A

Country Experiences

[Editor’s Note: The experience should be focused on the QoS/QoE impact, for how to remove the CF devices we have the Q.Sup.CFS-Use-Cases].

# Nigeria experience on the impact of counterfeiting on quality of service

A general survey conducted by one of Nigeria’s industry working groups showed that the majority of the operators in the telecommunication industry showed from their answers to survey questions that counterfeit devices are impacting negatively on the quality of service, and that it is necessary to control the influx of counterfeit mobile devices into the country in order to curb the negative effects such devices are having on the quality of service and quality of experience of the consumers and subscribers. This cannot be generalized until other regions or countries also share their own experiences so that this can be juxtaposed and benchmarked to have common recommendations.

## Overview of the situation

From the Nigerian experience, which showed that technical problems affecting QoS can be caused by counterfeit terminals, mobile telephone operators which has been brought to the attention of the Nigerian Communications Commission during a couple of Industry Working Group meetings  in the year 2019.

It is, thus, clear that the use of such terminals in networks has a negative impact on QoS parameters and it is important that influx of counterfeit devices should be minimized.

## Data on the impact of CF on QoE/QoS on Nigeria

[Editors Note: include the information on this topic]

## Conclusion

[Editors Note: Conclusion should focus on the impact of QoE/QoS]

Stricter enforcement of IP laws is essential to curb the growing threat that counterfeiting and piracy pose to society. A first step towards effective actions consists of achieving a clear understanding of the scale of the problem.

It is important to engage directly with national consumer protection agencies, international organizations and the private sector to promote effective anti-counterfeiting strategies.

Campaigns such as FAKES COST MORE – IMPACT ON INDIDUAL AND NATIONAL ECONOMY could be promulgated in different countries to fight counterfeiting.

This contribution shows the mandatory need for regulators to accurately identify the counterfeit devices on the national network and to manage the network until all counterfeit devices are abolished. This can be done through the deployment of an accurate system backed up by the necessary regulation.

# Bibliography

[b-MWF-Publication] Counterfeit/substandard mobile phones - a resource guide for governments Framework for solutions to combat counterfeit. (https://spotafakephone.com/docs/eng/MMF_CounterfeitPhones_EN.pdf)

[b-IEEE-Report] 		Substandard cell phones: impact on network quality and a new method to identify an unlicensed IMEI in the network (https://ieeexplore.ieee.org/document/6766091)

[b-IEEE-Report2]	Policy Framework and Recommendations to Minimize the Usage of Stolen and Counterfeit or Substandard Mobile Communication Devices

[b-GSMarena] https://www.gsmarena.com/glossary.php3?term=network-coverage#:~:text=The%20geographical%20area%20covered%20by,Network%20capacity

[b-TelecomReview] https://www.telecomreview.com/articles/reports-and-coverage/4865-telecommunication-security-ddos-attacks

[b-twobirds.com] https://www.twobirds.com/insights/2019/uae/counterfeit-mobile-devices-more-than-just-a-security-risk

______________