|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  | SG5-Cn |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 5 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 6/5 |  | Geneva, 3-12 June 2025 |  |
| CONTRIBUTION |  |  |  |  |  |
| Source: |  | Ministry of Communications, Department of Telecommunications, India |  |  |  |
| Title: |  | Considerations for update of content of baseline text for draft new Recommendation L.DLEE “Deep Learning Computation Efficiency Evaluation framework and metrics |  |  |  |
| Contact: |  | Vineet Malik TEC, DoT Ministry of Communications, India |  |  | Tel: 9999184213 Fax: E-mail: vineet.malik@gov.in |
| Contact: |  | Kanika Gambhir DoT Ministry of Communications, India |  |  | Tel: 9891056945 Fax: E-mail: kanika.gambhir@gov.in |

| Abstract: | This proposal contains update of content of baseline text for draft new Recommendation L.DLEE “Deep Learning Computation Efficiency Evaluation framework and metrics” which is based on SG5-C489, Wrocław, 17 -21 June 2024,Poland. The existing contribution increasingly acknowledges the environmental concerns surrounding AI such as energy consumption and carbon emissions, however, some, a novel metric: the sustainability-aware efficiency offset has been proposed. This is a supplemental metric that quantifies the ecological cost of AI models through factors such as CO₂ emissions, energy consumption, and renewable energy usage. This allows for more holistic benchmarking that balances accuracy, efficiency, and environmental responsibility. The clause 8.3.5 has been added in track change mode |
| --- | --- |

Table of Contents

Page

1 Scope 8 2 References 8 3 Definitions 8 3.1 Terms defined elsewhere 8 3.2 Terms defined in this recommendation 9 4 Abbreviations and acronyms 9 5. Conventions 9 6. Overview of Deep learning computation energy efficiency evaluation 9 7. Computation Efficiency Evaluation Framework for Deep Learning Computation 10 8. Computation Efficiency Evaluation Framework for Deep Learning Computation 10 8.1 General energy consumption metrics for deep learning 10 8.2 Deep Learning Computation Efficiency: 11 8.3 Other Computation Efficiency Metrics in Multiple Dimensions 11 8.3.1 Measuring Computation Efficiency through Computing Power Scale 11 8.3.2 Measuring Computation Efficiency through Deep Leaning Model Parameters 12 8.3.3 Measuring Computation Efficiency through Task Duration 12 8.3.4 Measuring Computation Efficiency through Training Data volume 12 9. Energy Efficiency Evaluation for varied Scenarios 12 9.1 Metrics of Development Procedure 12 9.2 Metrics of training procedure 12 9.3 Metrics of inference procedure 12 9.3.1 For inference on the GPU Cloud 12 9.3.2 For inference on edge computing 12 9.3.3 For inference on end terminals 13

Draft new Recommendation within ITU-T ITU-T L.DLEE

Deep Learning Computation Energy Efficiency Evaluation Framework and Metrics

Summary

This recommendation clarifies the conceptual framework of deep learning computation energy efficiency. It also introduces the evaluation methods and the general metric designed for deep learning computation computation efficiency. The goal is to enhance the energy efficiency for deep learning scenarios.

Keyword

[Keywords]

# Scope

This draft recommendation specifies computation efficiency evaluation framework and metrics for deep learning computing scenarios.

The scope of the new work item is proposed to include these aspects:

- The conceptual framework for deep learning computation efficiency evaluation.

- The metric for deep learning computation efficiency evaluation.

# References

The following Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[TBD]

# Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1 artificial intelligence (AI) [ETSI GR ENI 004]: Computerized system that uses cognition to understand information and solve problems.

3.1.2 Energy consumption [ISO/IEC 13273-1:2015]: quantity of energy applied.

3.1.3 Energy efficiency [ISO/IEC 13273-1:2015]: the ratio or other quantitative relationship between an output of performance, service, goods or energy, and an input of energy.

3.1.4 Energy saving [ISO/IEC 13273-1:2015]: reduction of energy consumption following implementation of an energy performance improvement action.

[TBD]

## 3.2	Terms defined in this recommendation

This Recommendation defines the following terms:

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

AI                 Artificial Intelligence

CPU	Central Processing Units

DL                Deep Learning

GPU             Graphics Processing Units

[TBD]

# 5. 	Conventions

None

# 6.	Overview of Deep learning computation energy efficiency evaluation

In recent years, the exponential growth of AI technologies, particularly the emergence of deep learning models, has significantly transformed various industries and sectors. The utilization of deep learning for tasks such as natural language processing, computer vision, and data analytics has showcased remarkable advancements in performance and capability. However, this progress has been accompanied by a notable increase in energy consumption throughout the lifecycle of AI models, encompassing their development, training, inference, and application stages.

The development of deep learning computation involves complex algorithms, vast amounts of data, and significant computational resources. Training these models often requires extensive computational power, leading to substantial energy usage and carbon emissions. Moreover, the deployment and inference stages, where deep learning computation are utilized for real-time decision-making and analysis, also contribute to energy consumption, particularly in resource-intensive applications.

The increasing energy demands of deep learning computations pose significant challenges to energy sustainability, cost-effectiveness, and the long-term evolution of AI technologies. To effectively address this issue, it is imperative to adopt a holistic approach aimed at optimizing energy efficiency across all stages of deep learning computation. While energy-saving strategies and best practices have been outlined in standards such as ITU-L.IEDL and L.Cooling_DC, the absence of standardized metrics and evaluation methods remains a notable gap. Deep learning models vary widely in their energy consumption due to factors such as parameter size, pre-training data volume, epoch numbers, and model accuracy. Moreover, different AI models exhibit distinct energy consumption patterns even when handling the same type of data. Some models prioritize achieving higher accuracy through extensive training, leading to diminished overall computational efficiency. For instance, certain models may benefit from training on large datasets, albeit at the expense of increased energy consumption. Additionally, the power consumption of AI tasks can vary significantly across different types of tasks. For instance, after performing 1000 calculations, the power consumption associated with image classification tasks may be considerably lower compared to image generation tasks. The lack of accurate estimation and measurement of AI task energy consumption poses challenges for both computing resource and network resource scheduling in the future.

Therefore, addressing the computation efficiency of deep learning computation requires the development of standardized metrics and evaluation methods to assess energy consumption across diverse models and tasks. It is essential to define the recommendation about the general evaluation framework for  for deep learning computation efficiency, end to end energy consumption metrics.

# 7.	Computation Efficiency Evaluation Framework for Deep Learning Computation

The general computation efficiency evaluation framework includes the following key components. By delineating these layers, the framework provides an end to end evaluating for the computation efficiency of deep learning computation.

Energy consumption evaluation from Infrastructure Layer:

The infrastructure layer comprises the fundamental computing resources essential for executing deep learning computations. This layer reports energy consumption and computing power scale during deep learning computations, providing valuable insights for optimizing efficiency.

Resource monitoring from Platform Layer:

The platform layer encompasses the monitoring systems designed to track vital metrics like temperature, resource utilization, and time duration. These monitoring systems ensure optimal performance and efficient resource management throughout the computational processes.

DL Model complexity evaluation from DL Model Application Layer:

The DL model application layer is the focal point for AI model training, inferencing, and research and development activities. The complexity of DL models is generated, which includes factors such as the number of epochs, parameter quantity, data volume, and the depth of layers.

Based on these inputs, the performance metrics for computation efficiency will be assessed.

# 8.	Computation Efficiency Metrics for Deep Learning Computation

## 8.1 General energy consumption metrics for deep learning

The life cycle of a deep learning model encompasses development, training, inference, and application stages. To ensure a comprehensive evaluation of computation efficiency throughout each stage, a set of general metrics applicable to all phases of the AI model life cycle is essential. Proposed general metrics for a more holistic energy consumption assessment:

Computation Energy Consumption (total)=Kenergy consumption of each unit

This metrics indicates that the total energy of deep learning task is the sum of the energy consumption of each unit. The energy of each unit including the such as CPUs, GPUs, memory, storage, and networking.

Central Processing Units (CPUs): Energy consumption for CPUs involves factors like clock speed, number of cores, and utilization during training or inference. This can be measured in terms of power consumption (Watts) over time.

Graphics Processing Units (GPUs): GPUs are commonly used for deep learning due to their parallel processing capabilities. Energy consumption for GPUs is similar to CPUs but often higher due to their intensive computational tasks.

Memory: Both RAM (Random Access Memory) and VRAM (Video Random Access Memory) contribute to energy consumption. Memory access patterns, data movement, and caching affect energy usage.

Storage: Energy consumption in storage devices (e.g., SSDs, HDDs) involves factors like read/write operations, data transfer rates, and idle power consumption.

Networking: Energy consumption for networking equipment (e.g., switches, routers) is influenced by data transmission rates, packet processing, and network congestion.

To measure the energy consumption of each unit, one can use specialized hardware energy meters or software-based power monitoring tools. Hardware energy meters directly measure power consumption at the device level, providing accurate readings. Software-based tools often rely on performance counters and sensors within the hardware to estimate power consumption.

[The measurement method and metrics will be modified later]

## 8.2 Deep Learning Computation Efficiency:

The deep learning computation efficiency can be evaluated by the computation Task Per Energy(CTPE).

CTPE=Computing Task Performance/ Computation energy consumption

The computing task performance refers to the amount of computing required to execute AI computing tasks, such as the number of images processed during ResNet-50 image classification.

The computation power consumption refers to the total energy consumed during the execution of computing tasks.

## 8.3 Other Computation Efficiency Metrics in Multiple Dimensions

The computation consumption can be defined in multiple dimensions, and the following sections will give the definitions of different scenarios in detail.

## 8.3.1 Measuring Computation Efficiency through Computing Power Scale

Measuring energy efficiency through the computing power scale offers a quantitative measurement that aligns with the computational complexity of AI models. FLOPs, representing Floating Point Operations Per Second, serve as a metric for computing power during both training and inference phases, reflecting the algorithm or model's complexity and computational demands. A higher FLOPs value typically signifies a greater need for floating-point operations, indicating a higher requirement for computational resources. Integrating FLOPs into computation efficiency metrics enables a more comprehensive evaluation of deep learning model computation consumption, allowing for comparisons across various models and streamlining optimization endeavors for enhanced the total energy efficiency.

## 8.3.2 Measuring Computation  Efficiency through Deep Leaning Model Parameters

Measuring the computational efficiency of an AI model involves considering various factors such as the volume of model parameters, batch size, and number of layers.

Model Parameters Volume: The total number of parameters in the model impacts the computational complexity and memory requirements during both training and inference.

Batch Size: The batch size determines the number of samples processed simultaneously during training. It affects the utilization of computational resources such as GPUs and memory.

Number of Layers: The depth of the model, indicated by the number of layers, influences the computational workload. Deeper models generally require more computations.

By analyzing these factors, we can assess the computational efficiency of an AI model. Models with fewer parameters, optimized batch sizes, and efficient layer configurations tend to be more computationally efficient, requiring fewer resources to achieve comparable performance.

## 8.3.3 Measuring Computation Efficiency through Task Duration

Task during represents the time required for the procedure to gain the target deep learning computation accuracy.

To be refined later

## 8.3.4 Measuring Computation Efficiency through Training Data volume

To be refined later

# 8.3.5 Sustainability-Aware Efficiency Offsets (Proposed Supplement)

As AI adoption grows rapidly across sectors, particularly in data- and compute-intensive areas like deep learning, so does the environmental footprint of such systems. Current evaluation methods primarily measure raw computational efficiency (speed, scale, parameter count), but they omit the ecological cost of achieving that performance. This creates a gap: a system might score well on conventional metrics while consuming significant electricity or generating substantial emissions, which is unsustainable in the long term—especially in countries with carbon-intensive grids or limited energy resources.

Sustainability-Aware Offsets

There are several reasons for introducing offset, a few of them are listed below:

1. Environmental Responsibility: Large-scale models can emit CO₂ equivalent to hundreds of flights; without offsets, efficiency rankings ignore this cost.
2. Fair Benchmarking: Comparing two models without considering their energy use can favor high-power, high-emission systems even when greener alternatives exist.
3. Informed Trade-offs: Offsets reveal when marginal accuracy gains come at disproportionately high environmental costs.
4. Equity & Accessibility: Sustainable models are more affordable and feasible in energy-constrained or low-resource regions.

Integrating Sustainability Offsets into the Efficiency Framework:

In section 8.3.1(Measuring Computation Efficiency through Computing Power Scale), the traditional metric is Computing Power Scale, measured in TFLOPs and FLOPs/Watt, and the sustainability offset (supplemental) includes CO₂ emissions per TFLOP, PUE (Power Usage Effectiveness), and Renewable energy share. The offset improves the framework by encouraging use of efficient hardware and green data centers; adds climate accountability.

## In section 8.3.2(Measuring Computation  Efficiency through Deep Leaning Model Parameters

), the traditional metric is Model Parameters, measured by parameter count and model size, and the sustainability offset includes energy (kWh) per million parameters and emissions during training. This promotes parameter-efficient architectures; discourages unnecessary model bloat.

## In section 8.3.3 (Measuring Computation Efficiency through Task Duration), the traditional metric is Task Duration, measured by total training time and inference latency, and the sustainability offset includes emissions per training hour, peak power draw, and carbon-aware scheduling. This highlights environmental cost of long runtimes; incentivizes time-efficient training.

## In section 8.3.4(Measuring Computation Efficiency through Training Data volume), the traditional metric is Training Data Volume, measured by dataset size (samples/GB), and the sustainability offset includes energy consumed per sample processed and data transmission emissions. This encourages leaner data usage; supports local training for reduced transmission footprint.

To calculate and report these offsets, developers may use:
CodeCarbon: Measures CO₂ emissions from Python training scripts.
Green Algorithms: Estimates energy and emissions based on model parameters and hardware type. Cloud Sustainability APIs: Google Cloud, AWS, and Azure now provide CO₂ dashboards at project or region level.

Conclusion & Recommendation

# The inclusion of sustainability-aware offsets makes deep model computational efficiency metrics more complete, responsible, and future-proof. It allows stakeholders to assess trade-offs not just in terms of accuracy and latency, but also in terms of climate impact, affordability, and regional feasibility.

# 9.	Energy Efficiency Evaluation for varied Scenarios

[To be refined later. Note: More features will be defined.]

## 9.1 Metrics of Development Procedure

The energy efficiency of development procedure means the energy that produced before model training, including the cost of data collection, data preprocessing and a series of preparatory work.

## 9.2 Metrics of training procedure

A complete training process contains many epochs, where an epoch means one pass (forward and backward) of the whole training dataset. The metric for energy efficiency in the training process includes the energy efficiency of each epoch.

## 9.3 Metrics of inference procedure

Compared with training process, inference procedure only contains forward process, and AI models run once for test dataset. The inference process is divided into single inference and batch inference, where single inference refers to inferring only one sample at a time, and batch inference means inferring a batch of samples at once. About single inference, the energy efficient definition is as follows.

[Note: More features will be defined.]

## 9.3.1 For inference on the GPU Cloud

[Note: More features will be defined.]

## 9.3.2 For inference on edge computing

[Note: More features will be defined.]

## 9.3.3 For inference on end terminals

[Note: More features will be defined.]

Appendix I

To be refined later

Bibliography

To be refined later

A.1 justification for draft new Recommendation ITU-T L.DLEE “Deep Learning Computing Energy Efficiency Evaluation Framework and Metrics”

| Question: | Q6/5 | Proposed new ITU-T Recommendation | 17-21 June 2024 |  |
| --- | --- | --- | --- | --- |
| Reference and title: | Proposed new work item on ITU-T L.DLEE Deep Learning Computing Energy Efficiency Evaluation Framework and Metrics |  |  |  |
| Base text: | C489 |  | Timing: | 2026-Q4 |
| Editor(s): | Xin Wang, China Unicom, Tel: +86-18601106563, Email: wangxin349@chinaunicom.cn; Jian Yao, China Unicom, Tel: +86-18601722890, Email: yaojian1@chinaunicom.cn LingYun Wan, State Grid Corporation of China, Tel: + 86-15086839960 E-mail: 375676739@qq.com |  | Approving: | AAP |
| Scope (defines the intent or object of the Recommendation and the aspects covered, thereby indicating the limits of its applicability): |  |  |  |  |
| This draft recommendation specifies computing energy efficiency evaluation framework and metrics for deep learning computing scenarios. The scope of the new work item is proposed to include these aspects in a step-wise approach: - Analysis of existing definition of deep learning - The conceptual framework for deep learning computing energy efficiency evaluation. - The metric definition and measurement method for deep learning computing energy efficiency evaluation. |  |  |  |  |
| Summary (provides a brief overview of the purpose and contents of the Recommendation, thus permitting readers to judge its usefulness for their work): The increasing energy demands of deep learning computing pose significant challenges to energy sustainability, cost-effectiveness, and the long-term evolution of AI technologies. To effectively address this issue, it is imperative to adopt a holistic approach aimed at optimizing energy efficiency across all stages of deep learning computing. Addressing the computing efficiency of deep learning computing requires the development of standardized metrics and evaluation methods to assess energy consumption across diverse models and tasks. By doing so, we can better manage energy resources, optimize computing efficiency, and ensure the sustainable evolution of AI technologies. |  |  |  |  |
| Relations to ITU-T Recommendations or to other standards (approved or under development): |  |  |  |  |
| ITU-T L.IEDL , ITU-T L.MM_Computing power, ITU-T L.S_AI |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |
| ISO/IEC JTC1/SC42, SG16 |  |  |  |  |
| Supporting members that are committing to contributing actively to the work item: |  |  |  |  |
| China, China Telecom, ,State Grid Corporation of China (China), Korea (Republic of) |  |  |  |  |