|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  |  |  | SG5-Cxx |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | STUDY GROUP 5 |
|  |  |  |  |  | Original: English |
| Question(s): |  | 3/5 |  |  | Geneva, 03-12 June 2025 |
| CONTRIBUTION |  |  |  |  |  |
| Source: |  | Ministry of Communications, Department of Telecommunications, India |  |  |  |
| Title: |  | Proposal of new work item – new Recommendation K.actual_max "Guidance on implementation of Actual Maximum Approach for assessment, evaluation, compliance and monitoring of RF EMF" |  |  |  |
| Contact: |  |  | Bhoomika Gaur Department of Telecommunications, Ministry of Communications, India | Tel: +91 9315251773 E-mail: bhoomika.gaur@gov.in |  |
| Contact: |  |  | Neha Upadhyay Department of Telecommunications, Ministry of Communications, India | Tel: +91 9868132034 Email: neha.upadhyay86@gov.in |  |

| Abstract: | This document contains a proposed new work item with new Recommendation ITU-T K.actual_max "Guidance on implementation of Actual Maximum Approach for assessment, evaluation and compliance of RF EMF" |
| --- | --- |

The aim of this new Recommendation to provide guidance on implementation of actual maximum approach in assessment, evaluation and compliance of RF-EMF exposure levels. Conventionally, the assessment and compliance of RF-EMF exposure levels have taken into account the worst case scenarios such as taking into account maximum traffic conditions, maximum emission from transmitter etc so as to ensure RF-EMF compliance during maximum possible exposure conditions. However, at the same time, the actual maximum approach (from IEC 62232) suggests evaluation of RF EMF exposure taking into account the actual maximum, i.e. the maximum value reached by the actual value recorded.

While the existing ITU-T Recommendations provide guidance on assessment of RF-EMF via various calculation, simulation and measurement based methods, not enough guidance is available in the existing standards for the regulators to use and implement the actual maximum approach in the assessment, evaluation and compliance of RF-EMF exposure levels.  There is a need to develop a new ITU-T Recommendation which covers the implementation guidelines for adoption of this approach in various phases of telecommunication installations- product compliance, installation and in operations; and offers case studies for reference.

A.1 justification for new work item on new ITU-T Recommendation K.actual_max "Guidance on implementation of Actual Maximum Approach for assessment, evaluation, compliance and monitoring of RF EMF"

| Question: | Q3/5 | Proposal of new ITU-T Recommendation | Geneva, 03-12 June 2025 |  |
| --- | --- | --- | --- | --- |
| Reference and title: | Recommendation K.actual_max "Guidance on implementation of Actual Maximum Approach for assessment, evaluation, compliance and monitoring of RF EMF" |  |  |  |
| Base text: |  |  | Timing: | 2027 |
| Editor(s): | Bhoomika Gaur (bhoomika.gaur@gov.in) Neha Upadhyay (neha.upadhyay86@gov.in) |  | Approval process: | AAP |
| Scope: This new Recommendation to provide guidance on implementation of actual maximum approach in assessment, evaluation and compliance of RF-EMF exposure levels during various phases of telecommunication installations- product compliance, installation and in operations; and offers case studies for reference. |  |  |  |  |
| Summary: |  |  |  |  |
| Conventionally, the assessment and compliance of RF-EMF exposure levels have taken into account the worst case scenarios such as taking into account maximum traffic conditions, maximum emission from transmitter etc so as to ensure RF-EMF compliance during maximum possible exposure conditions. However, at the same time, the actual maximum approach (from IEC 62232) suggests evaluation of RF EMF exposure taking into account the actual maximum, i.e. the maximum value reached by the actual value recorded. While the existing ITU-T Recommendations provide guidance on assessment of RF-EMF via various calculation, simulation and measurement based methods, not enough guidance is available in the existing standards for the regulators to use and implement the actual maximum approach in the assessment, evaluation and compliance of RF-EMF exposure levels. There is a need to develop standards and implementation guidelines focused on adoption of actual maximum approach in various phases of telecommunication installations- product compliance, installation and in operations. |  |  |  |  |
| Relations to ITU-T Recommendations or to other standards (approved or under development): |  |  |  |  |
| [ITU-T K.52], [ITU-T K.61], [ITU-T K.100], [ITU-T K.70], IEC 62232, IEC TR 62669 |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |
| ITU-R, ITU-D, IEC TC106 |  |  |  |  |
| Supporting members that are committing to contributing actively to the work item: |  |  |  |  |
| India |  |  |  |  |

_________________