|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG12-TD172R1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 12 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 12/12 |  | Geneva, 18-26 January 2023 |  |
| TD |  |  |  |  |  |
| Source: |  | Editor E.MVS |  |  |  |
| Title: |  | Draft baseline text ITU-T E.MVS "Mapping and visualization strategies for the assessment of connectivity and QoS" |  |  |  |
| Contact: |  | Rocio Nahiely Velasco Santos Federal Institute of Telecommunications Mexico |  |  | Tel: +52 55 50154000 E-mail: rocio.velasco@ift.org.mx |
|  |  | Abdul Kayum Telecommunication Engineering Centre, Ministry of Communications India |  |  | Tel: +918874209603 Email: Abdul.kayum@gov.in |
|  |  | Zeid ALKADI Telecommunication Regulatory Commission (TRC) Jordan |  |  | Tel: +962791997878 Email: zeid.alkadi@trc.gov.jo |
|  |  | Joachim POMY Opticom GmbH Germany |  |  | Tel: +49177 78 71958 Email: jp@joachimpomy.de |

| Abstract: | This TD provides the baseline text for the work item ITU-T E.MVS " Mapping and visualization strategies for the assessment of connectivity and QoS ". This document is the outcome of discussions done during the Question 12/12 sessions (first and second sessions) and drafting sessions on E.MVS. A version of the text with track changes is available in attachment. |
| --- | --- |

Draft new Recommendation ITU-T E.MVS

Mapping and visualization strategies for the assessment of connectivity and QoS

Summary

Mapping the QoS of the telecommunication services is crucial to the work of regulators, government entities and operators since it provides inputs that facilitate the decision-making process to reduce the digital divide. Therefore, this recommendation provides guidance, especially when evaluating and communicating connectivity status and its relation to QoS across different topologies [including in-building scenarios] and population densities within the country. Where connectivity should be understood not only as the presence or not of telecommunications services, but as a concept that should be built upon different perspectives through relevant sources of information.

Therefore, this work item defines a high-level framework for mapping and visualization strategies that include guidelines to define first how to identify and measure the level of connectivity and second which tools and functionalities are useful in communicating the status of this connectivity.

Editor's Note:

- The Summary needs to be aligned with the scope and other chapters of the draft recommendation.

Keywords

Connectivity assessment, mapping, visualization, QoS, GIS, connectivity index.

Introduction

<Optional – This clause should appear only if it contains information different from that in Scope and Summary>

# 1	Scope

This recommendation proposes selected guidelines for connectivity assessment in a country for different use cases using mapping tools and relevant sources of information.

The recommendation outlines a series of toolboxes including relevant parameters and identifies useful guidelines to construct a composite connectivity index. Besides, it describes how connectivity can be visualized including in-building connectivity assessment guidelines.

This recommendation also provides guidance on spatial analysis including stratification strategies to classify geographical area, data granularity, collection and post processing mechanisms and possible mapping and visualizations depending on target audience (end users, regulators, operators, etc).

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title.

[ITU-T E.802 Amd2]   Recommendation ITU-T E.802 Amd2 Framework and methodologies for the determination and application of QoS Parameters.

[ITU-T E.805]	Recommendation ITU-T E.805 (2019), Strategies to establish quality regulatory

frameworks.

[ITU-T E.805.1]	Recommendation ITU-T E.805.1 (2021), Quality of service operational strategy for improved regulatory supervision of providers of mobile telecommunication services.

[ITU-T E.806]	Recommendation ITU-T E.806 (2019), Measurement campaigns, monitoring systems and sampling methodologies to monitor the quality of service in mobile networks.

[ITU-T E.812]	Recommendation ITU-T E.812 (2020), Crowdsourcing approach for the assessment of end-to-end quality of service in fixed and mobile broadband networks.

[ITU-T E.840]	Recommendation ITU-T E.840 (2018), Statistical framework for end-to-end network performance benchmark scoring and ranking.

# 3	Definitions

<Check in the ITU-T terms and definitions database at www.itu.int/go/terminology-database whether the term has already been defined in another Recommendation. It would be more consistent to refer to such a definition rather than to redefine the term>

## 3.1	Terms defined elsewhere

<Normally, terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document. Before defining a new term, verify whether it has already been defined in the official ITU terminology database, at www.itu.int/go/terms. >

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>.

3.1.2	<Term 2> [Reference]: <optional quoted definition>.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Mapping: The process of illustrating values or parameters in a geographic information system (GIS).

3.2.2	Connectivity index: A scalar level that represents the overall degree of two-way access to telecommunication services related to performance and usability, evaluated at a particular time, and applicable for a specific duration.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

<abbr>	<expansion>

GIS		Geographic information system

<Include all abbreviations and acronyms used in this Recommendation>

# 5	Conventions

<Mandatory clause. Describe any particular notation, style, presentation, etc. used within the Recommendation, if any. If none, write "None.">

# 6	Clause 6 of the Recommendation

<Clause 6 onwards contains the body of the Recommendation>

## 6.1	Connectivity Assessment

Connectivity Assessment is primarily essential to understand the optimal connectivity needs, opportunities and challenges of the target community in a particular location or a specific geographical area. To this end, a more holistic approach to connectivity assessment helps to gather information on current and future needs, perform gap and risk analysis, boost investment and ensure productivity and growth in the targeted community or area.

Therefore, connectivity cannot be measured by a single descriptive indicator nor by multiple indicators disconnected from each other. Instead, it should be represented with the ‘combination’ of different dimensions, to be considered together as the proxy of the phenomenon.

A practical way to assess connectivity is the creation of a composite index (single number) calculated from multiple sources of information related to the digital ecosystem that evaluates the level of connectivity available in a geographical area. This connectivity index can represent the aggregation of different categories of data, such as telecommunications infrastructure, topography or terrain characteristics, QoS parameters, regulatory aspects, demand for telecommunications services and even socio-economic statistics. It is important to emphasize that the sources of information to consider will depend on the objective of the analysis. Indeed, identifying different levels of connectivity in different geographical areas through a connectivity index can lead to greater competition and investment, to the benefit of all the different stakeholders in the digital ecosystem. These connectivity levels can provide investors with granular country-level market intelligence that highlights key intersections of telecommunication development needs.

6.1.1 Connectivity assessment use cases

Editor’s note: Contributions are invited

The following use cases for connectivity assessment collection and data analysis can provide substantial data for many stakeholders to ensure visuality and robustness for  their decisions (business decisions, market decisions, investments decisions).

USE CASE 1 -To accelerate infrastructure deployment

Connectivity remains a challenge in rural and remote areas where many people stillhave an unconnected digital life. Like water and electricity, connectivity too is a matter of  right for individuals, households, businesses, industries and the public sector. There must exist a particular consideration for those who live in remote areas and belong to vulnerable groups.

Nowadays, technology and infrastructure are used increasingly to bring innovative solutions for complex problems in multiple sectors like energy, health, education, transport, and agriculture, among others. Hence connectivity assessment will endow governments and regulatory authorities to enable efficient and timely deployment of resources to provide telecommunication services with proper justification.

USE CASE 2 -To resolve any quality of service issue

The quality of services can differ remarkably depending on the place and time; consequently, an assessment of connectivity helps to focus on those geographical areas where there is a lack of quality or it exists a quality deficiency.

Ensuring the quality of services in this ever-changing environment is increasingly critical. A harmonized and common approach like connectivity assessment would enable greater quality prospects irrespective of the locations of the consumer and service provider.

USE CASE 3 -To provide informed choices to end users in terms of services

Users' decisions must not be only based on the price. Considering a connectivity assessment, their decisions will be sustained by other relevant variables involved in the connectivity index calculation. Therefore, connectivity assessment will help customers be aware of the multiple levels of the connectivity index that allows them to make informed choices. An informed decision will make him a loyal customer as well.

USE CASE 4 -To have updated information related to connectivity.

It is relevant to evaluate constantly the state of digital connectivity to know how close the current scenario is to achieving universal and meaningful connectivity that relies on the “connectivity enablers” of infrastructure, affordability, device, skills, and safety and security. Indeed, a meaningful connectivity implies a level of connectivity that allows users to have a safe, satisfying, enriching, and productive online experience at an affordable cost and with a sufficiently large data allowance according with the Global Connectivity Report 2022.

The United Nations 2030 Agenda for Sustainable Development sets 17 Sustainable Development Goals (SDGs) for humanity and the catalytic and enabling role of connectivity in sustained, sustainable, and inclusive development and growth is recognized among these SDGs. Therefore, it becomes more important to analyze the current state of digital connectivity globally and his progress towards reaching the targets by 2030.

USE CASE 5 -To improve the regulatory frameworks.

Connectivity assessment will have many useful applications to make  regulation more efficient for the benefit of final users. The following are some examples identified:

Market’s competition: Knowldge of the market’s competition dynamic and the status of deployments, may help drafting better public policies and streamlining the obligations. Accordingly, it may also be incorporated into operators’ future licences.

Measuring QoS and coverage also means obtaining the means to stimulate competition and steer the market in the right direction by providing users with the information that will enable them to choose the operator that best matches their needs in an objective manner.

Digital scorecard: Connectivity assessment may help to produce a digital scorecard for the country for  guiding and/or anticipating deployments. It may be used by the regulator/governments to monitoring operators’ compliance with obligations and stimulating competition too. It can also be used to help bolster regional digital development.

InvestorGuidance: Right now, an operators’ network is a Blackbox for an investor. Obtaining a better and objective understanding of operators’ actual coverage in the field, and  the ability to establish a diagnosis will help investors and international financial institutions to better channel their choices.. Also, once investment is done, it will provide a forward-looking view of rollouts and network performance, to be able to assess the efficiency and effectiveness of the investments made.

White Areas: It will help drafting schemes for deploying networks in white areas and defining priority coverage targets. The ability to identify areas where coverage is poor or non-existent thanks to these maps also helps public authorities establish a diagnosis of the national status quo, and in tailoring their policies to satisfy coverage needs and reduce the digital divide.

### 6.1.2	Connectivity indicators

Editor’s note: Contributions are invited

- OECD has a guideline for the construction of composite indicators. Perhaps it could help in the discussions: https://www.oecd.org/els/soc/handbookonconstructingcompositeindicatorsmethodologyanduserguide.htm

The purpose of creating connectivity indicators is to provide actionable insights to improve the status of connectivity depending on the use cases. Then, connectivity indicators can be compared geographically, based on the stratification of the studied areas.

A composite index can be obtained through a combination of connectivity indicators using statistical methodologies. This index represents an even broader view of the connectivity status. It is beneficial when a single dimensionless value is required to assess the connectivity status in simple way, so that decision makers can have a complete picture in mind. Then, concrete, targeted action plans can be derived to improve the connectivity based on the connectivity indicators that assess the dimensions of interest.

### 6.1.3 Connectivity index

Even if there is no universal method for constructing composite indices, their construction is mainly determined by the application or question that is going to be investigated.

In simple terms, a composite index is a single (scalar) number that is constructed from a set of input values. Typically, those input values come with their own ranges (e.g. a MOS scale from 1 to 5). In order to process them into a composite indicator, these values need to be normalized, i.e. brought to the same scale. Each input value has a given weight with which it contributes to the output quantity.

For instance, if the output quantity is calculated as a weighted sum of normalized input values, the set of set of weights is chosen such that the output value has also the desired value range, e.g. 0..100.

Also typically, input values are not used linearly, but are transformed using some nonlinear function, expressing essentially an assessment. If this transformation is done by a function applied to the original input quantity, normalization can be integrated into this transformation already.

Figure 1 – Construction of a composite index

The following guidelines are recommended for the construction of a connectivity index that uses a set of variables (connectivity indicators) that can be obtained from the toolboxes previously described:

- KPI from multiple toolboxes should be selected according to the considerations defined by the objective of the analysis. The selection step is the result of a trade-off between possible redundancies caused by overlapping information and the risk of losing information.

- Normalization can be required as the KPI in a data set often have different measurement units. Therefore, it is necessary to bring the KPI to the same standard, by transforming them into pure, dimensionless, numbers.

- The composite index is constructed through the combination of all the different KPI from the chosen Toolboxes. For this combination, different aggregation methods are possible. The choice of the ‘best’ aggregation method also depends on the aim and the users of the analysis (regulators, operators, or the end users).

- The methodology to obtain the index should be transparent and documented to allow for validation/verification of the results.

- The use of multivariate methods is suggested for the connectivity index construction. Multivariate methods allow for a deeper exploration into possible patterns that exist in the data, enable complex inter-relationships between many variables to be represented graphically, and provide ways of reducing the dimensionality of the data for summary and further analysis.

- Among the multivariate methods available, cluster analysis can be used to identify natural groupings amongst the sampling units so that units within each group (cluster) are similar to one another while dissimilar units are in different groups. Furthermore, when dealing with large datasets (for example, crowdsourcing information), principal component analysis (PCA) can be useful to reduce the dimensionality of the data while considering as much as possible of the variation of the original data. In this way, a smaller number of variables can summarize the data by a linear combination of the original variables.

- The question of the choice of a system of weights in order to weigh the individual indicators, according to their different importance in expressing the considered phenomenon, necessarily involves the introduction of an arbitrary component. For example, subjective weights can be set by a group of specialists (e.g., regulators, operators, policy makers) or maybe social surveys about how important individual indicators are to the people. A suitable statistical methodology that assigns a weight depending on the variation of the data can also be used (for example, PCA).

### 6.2	Sources of information

Editor’s note: Contributions are invited

Gathering the right data becomes the first step and the base to assess the connectivity. Data analysis will be possible through good data collection, useful granularity level and organization.  For this reason, it is essential to identify the relevant sources of information and to insure about the quality and accuracy of the data. The more reliable and better organized the data, the more likely it will produce valuable information that can be transformed into actionable insights.

The next list contains some main characteristics recommended when considering sources of information:

- Data should be related directly or indirectly to the telecommunication services

A comprehensive and multi-dimensional approach may require gathering data that targets multiple and relevant sources of information which may be directly or indirectly concerned with the telecommunication services. It may be useful to mention that [b-ITU GCR] identifies five enablers of connectivity or the meaningful dimensions of connectivity as infrastructure, device, affordability, skills, and safety and security. Some of the enablers may apparently appear as not directly linked to telecommunication variables, however, they may impact the level of connectivity.

- Data should be representative of the investigated geographical area

In order to have information on the connectivity status that will be comparable across the studied geographical areas, it is important that all sources of information have the same representation in the whole area. Representativeness is not about the amount of data from each source of information but the presence of each source of information in the same geographical area, for example, if the connectivity assessment is going to be performed at a municipality level, then all sources of information should have that level of granularity. The geographical level to be studied could depend on the objective of the analysis and who is carrying it out (regulators, government entities, operators, others); hence geographical areas could vary such as a little town, a suburban area, a region or a country. It should also be considered that there are statistical techniques to account for under or overrepresentation of the samples available, for example, data imputation techniques can be used to increase the number of samples in some areas or sampling techniques can also be applied to avoid overrepresenting certain areas. Annex A of [ITU-T E.806] provides a reference to calculate the number of samples to achieve representative results.

- Data should be obtained from official/collaborative initiatives.

The connectivity index will be as accurate as the sources of information from which it was obtained. Official or collaborative initiatives said to have high accuracy and wider acceptability. Access to official information such as of central government departments or public bodies can improve confidence of the result obtained due to data source authenticity. Same way, source of information from collaborative initiatives, such as crowdsourcing data or data from any type of surveys, will have a big impact in the result obtained thanks to the number of information gathered but also due to the quality of information collected.

- Data should be obtained periodically

Data analysis to assess connectivity is not a simple task and shouldn’t be performed only once. It needs to be performed periodically. Therefore, data should be followed weekly, monthly or whatever the best frequency is for the intended target. Furthermore, data obtained periodically can be used to build a historical raw database that will allow to look at the evolution of the levels of connectivity through this composite index.

- Data sources should not be redundant

As data is the basis to understand the status of connectivity, it must be carefully chosen to avoid situations where the same piece of data exists in multiple places. Data redundancy can cause inconsistency, which can provide unreliable and/or meaningless information. Besides, data redundancy may increase the size and complexity of a database making it more of a challenge to maintain. However, when data from two different sources are complimentary to each other, redundancy can be useful. The last case refers, for example, when having two datasets of KPI that contain different samples, then using both datasets will increase the number of samples available for consideration.

6.2.1 Data Tools

Assessment and evaluation of the connectivity status depend on different informational resources or data tools and targets. For example, required targets can range from new market entry, filling coverage holes, and densification related matters. Such informational resources can dimension and scale telecommunication solutions if it is according to a set target.

For purposes of this recommendation, data from different sources of information can be viewed as toolboxes.

KPI obtained from data sources of information could be associated to each toolbox. The following toolboxes are suggested to begin the analysis, however the use of any or various of the toolboxes may differ according to the use case that is being investigated.

Editor’s Note: Link toolboxes with use cases to add context. To be completed when the use cases subclause is developed.  Suggest inserting (later, after stabilization of content) a graphical overview showing the 5 toolboxes and their principal content.

Performance Toolbox

The Performance toolbox includes KPI that measure the level of QoS achieved or delivered to the customer. These KPI are measurable (with instruments or observations) and allow to assess the ability of a whole or portion of the network to provide the functions related to communications between users.

KPI that could be considered in this toolbox, but not limited to, are the followings:

- Download/Upload Throughput, Packet Loss, Jitter and Latency

- Unsuccessful Call Ratio, Dropped Call Ratio, Call set up time

- Voice MOS, Video MOS

Telecom Market Trends Toolbox:

Market Trend Analysis is an effective tool for an evolving market such as the telecommunications industry to stay and grow. Since market trends are dynamic, such analysis enables companies to identify, forecast and strategize for current and upcoming markets including market maturity and future market potential.

Some KPI relevant to this toolbox are:

- Mobile Users Penetration (MUP) (telephony, Internet access) per neighborhood polygon

- Fixed Users Penetration (FUP) (telephony, Internet access) per neighborhood polygon

- Presence of any kind of service provider such as mobile operators, fixed operators, mobile virtual operators, wireless Internet Service Provider

- Fixed broadband basket

- Mobile cellular low-usage basket

- Mobile broadband Internet prices per month

- Satellite broadband internet prices per month (if any)

- Churn (potential revenue loss)

Coverage Toolbox

This toolbox gathers all those metrics that help to identify those geographic areas where users can connect to a network, stay connected and reliably communicate with optimal network conditions . It is  usually represented as single or multiple layers such as roads, lakes, inhabited areas, and land use, etc., (see ITU-T Y.4562 ).

Some KPI that could be considered to determine whether an area is covered:

- Signal strengths: RxLev (received signal level) for GSM, RSCP (received signal code power) for UMTS and RSRP (reference signal receive power) for LTE, SSRSRP for NR.

- Availability of line-of-sight (LoS)

- Availability of wired connections (optical fiber, DSL, coaxial) for internet:

- Availability of satellite access for Internet

Telecommunications Infrastructure Toolbox

Infrastructure toolbox contains those KPI related with deployed telecom infrastructure (masts and towers, optical fiber cables and other media, bandwidth and backhaul, etc.), nevertheless other kind of infrastructure can also be considered in this toolbox.

Some KPI that could be considered in this toolbox:

- Mobile access technologies availability (2G, 3G, LTE, 5G) (with the corresponding type of power supply deployed )

- Fixed access technologies availability (optical fiber, DSL, coaxial)

- Availability of backhauling

- Aerial accessibility via existing light poles and transmission tower infrastructure

- Points of interconnection (border gateway HUBs, Data centers, IXP)

- Availability of electricity (backup batteries / solar)

- Ease of right of way for laying cable or installing pole/tower

- Ease of right of use of existing street furniture

Toolbox Consumer Behavioral KPI

This toolbox reflects consumer behaviors that are helpful to study consumers and the processes they use to choose, use (consume), and dispose of telecommunications products and services. Studying consumer behavior is important because it helps to understand what influences consumers’ buying decisions, how consumers decide on a product to fill the digital divide in the market to identify the products that are needed and the products that are obsolete.

The KPI relevant to this toolbox are:

- Smartphones Penetration

- Computers Penetration by household

- Online shopping Penetration

- Digital financial services usage

- Traffic and Hours spent surfing Internet per consumer / by type of device

- Internet access penetration (fixed and mobile)

- Sensitivity to tariff

- Sensitivity to the quality of services (based on complaints)

Socio-economic factors Toolbox

Socio-economic toolbox refers to classifying people by their social, economic and educational status. Examining demographics not only provide an understanding of the composition of the market area, but they also help to understand the potential range of impacts of a proposed change, and the likely responses of those impacted if the change occurs.

Some KPI that could be considered in this toolbox:

- Demographic parameters: For example, age, education level, income level, number of members by household, genre, occupation, urban or rural population.

- Business parameters: Demographic data about enterprises, number of enterprises, size of enterprise measured by the number of employees, earnings, sector, among others.

- Socio-Economic Levels: Data that allows grouping and classifying households based on availability basic infrastructure (power supply grid, lighting, water, paving streets), health infrastructure, connectivity, and entertainment, among others, the level of details available depends on each country.

GIS Toolbox

A geographic information system (GIS) facilitates to understand patterns, relationships, and geographic context by visualizing in map forms and remains a powerful decision-making tool that enables effective environmental, demographic, and topographic analysis. Therefore, geodata-based telecom network analysis facilitates efficient decision-making like where network components should be placed and how they should be connected, what size components to use and how to create a strategy on meeting capacity requirements and maintaining network reliability, working conditions of a particular network, optimization of network performance, maximising the optimal use of existing infrastructure and minimizing the cost of deployment. Thus, geodata helps to enhance telecom processes through better customer-relationship management and location services. Accordingly, the GIS toolbox will include topographic related information.

Some layers of information relevant to this toolbox are:

- Terrain layers

- Topographic layers

- Building layers (2D and 3D)

- Regulatory aspects (Restriction on tower heights near airports, signal level at international borders, near schools/hospital/Prisons/ defense establishment, rights of way etc.)

- Protected area layers (National parks, conservation reserves, community reserves, sanctuaries, biosphere, monuments).

### 6.3	Data granularity, periodicity and collection mechanisms

Editor’s note: Contributions are invited

6.3.1  Granularity

The granularity level is relevant to define a criterion to segment geographical areas to identify different groups of users. Considering that it is common that users have different preferences or interests according to the geographical area they lived.

A careful examination of the data structure to identify different sectors or sub-strata should be undertaken, for example, urban and rural data cannot be combined due to inherent heterogeneity  . As another, a division of 10x10 meters grid could be suggested to divide each country. Then, a value of connectivity could be associated to each square. While, this level of detail may be sufficient for urban areas, whereas for a rural area, a 1x1Km may be more appropriate due to lower KPI density, KPI availability and typically less infrastructure buildup covering larger areas.

6.3.2 Periodicity

Editor's Note: comments: “Importance of having higher samples for daily collection allows regulators to visualize capacity. Some indicators such as throughput, numbers of test, peak capacities. For FTTH coverage can be accomplished by looking at longer time periods and hence sample accumulation”

6.3.3 Collection mechanisms

Editor’s note: This section should highlight the basis of criteria which allow comparability among different geographical areas.

### 6.4	In-building assessment of connectivity

Since typical users spent most of their time indoors it is important to be able to understand the QoS in these environments. Within the buildings, QoS measurements should take into account the effect of built structures, often the best way of achieving this will be via the direct measurement of KPIs from devices involving users in defining and achieving the optimum outcomes for themselves. With more detailed and accurate information about building and clutter environment, and that too in a digital format that is interoperable, would be of great help for tools predicting quality of services inside the building.

It shall be possible to mark the availability of the connectivity within a building, at each floor and at each room through a suitable labelling.

Connectivity quality assessment may start from a particular spot for a variety of services to be measured at that spot. Assessment of quality in an apartment may be composed of a number of such spots. Further assessment at building level may or may not be aggregation at floor level depending upon closeness of the variations across apartments on the same floor or across floor on the same side of the tower.

The indoor sample validation is essential primary process to assess in-building connectivity, hence, evaluation of parametric conditions using validation methodologies provide a high probability differentiation between indoor and outdoor samples. Moreover, combination of indoor sample indicators enhances confidence level of indoor identification and provides beneficial use cases.

The following criteria are recommended for filtering indoor and outdoor samples:

1. For indoor / Outdoor sample velocity comparison, a threshold for indoor sample velocity < 5 km/hr can be set as a primary filter before further validation takes place. For example, it is impossible to have a high velocity sample within an indoor building layer.

2. Validating and filtering out indoor samples can be achieved if coordinates are compared to the geospatial building polygon. Such process can only be efficient if a systematic approach is adopted for neighbourhood polygons and not per building basis.

3. Detection of rapid changes for received power (SS RSRP, RSRP, RSCP, RxLev) in comparison with building layer polygons. Often multiple detected second window samples are useful for indoor sample validation as degraded power values are obvious.

4. In the crowdsourcing context, there are modules and sensors available in the mobile devices which are widely penetrated across many device models and can be used to determine indoor/outdoor probability such as wi-fi signals, cellular signals, GPS signals, light sensors, barometric pressure, battery status (e.g. plugged) compass, time stamp, etc.

5. Collected samples with X, Y, Z coordinates indicate human activity at an elevation reference from ground level.  Hence, in-building connectivity assessment per level is possible if elevation values (Z values) are collected, processed, and considered as part of sample coordinates X, Y, Z. The confidence level of valid Z values can be impacted by rough terrain, location accuracy and sample size. As indoor and outdoor samples (X,Y,Z) coordinates are accumulated within dense population, the following benefits are identified:

- 3D building dimensional modelling which also serves as an indoor/outdoor validation source.

- Indoor coverage penetration visuality.

- In-building service optimization and quality assessment per level visuality.

## 6.5	Requirements and characteristics of Mapping and Visualization tools

Editor’s note: Contributions are invited

Assessing the level of connectivity of any particular geography can take many forms, ranging from literal mapping using geographic information system tools to data collection and analysis or polling that helps a region better understand its needs and opportunities. In general, visualization tools are needed to communicate information about the status of connectivity for different purposes and from different perspectives.

Visualization is a summary term for presenting data in visual form, such as bar graphs, pie charts, time-axis diagrams etc.

A special case of visualization is termed "mapping" or geo-visualization, meaning the visualization of data onto geographical maps.

In order to support this, data needs to have respective geographical information with it. For instance, demographic information may be related to postcode areas; in that case geo visualization could display this information with respective polygons on a map; in this case data is related to a given area. It is also possible that primary data is linked to a specific point (given by e.g. GPS co-ordinate; for example locations where actual measurements showed a call drop). In that case, visualization can either be direct (drawing respective points on the map) or data can be aggregated. In that case, the unit of aggregation would be a polygon (a tile, a postcode areas, etc.) and the quantity to be visualized would be e.g. a color-coded representation of an indicator, in the example case of a call-drop rate.

    - Mapping

  - Improving knowledge of the different services availability in a defined geography.

  - Providing information to attract investment or generating support initiatives.

  - Exposing common misperceptions.

  - Reducing duplication of efforts by identifying gaps and potential ways to fill them

  - Helping to prioritize and allocate resources

  - Serving as the basis for tracking and demonstrating impact over time

  - Understanding where there are systemic inequities and opportunities.

6.5.2	Visualization

Visuality should consider high sample location accuracy in reference to topography and time dimensions.

The visualization tool should offer flexibility and interactivity for the users. The users should be able to export the visualised data on a common format. It should also be possible to display in different languages.

Editor’s note:  *Types of maps (open signal presentation)

Different types of maps may be needed to represent different kind of information and purpose.

- Point maps: may be used to represent highly localized events and structures

- Choropleth maps offer a clear way of visualizing a metric by regional blocks. They can provide a useful means of highlighting which administrative groupings may require investment.

- Hexagon based maps allow metrics to be visualized at a variety of scales. As the map is zoomed hexagons can be replaced by another layer.

- Heatmaps or smoothly varying maps may work well for variables that can change rapidly on small dimensional scales (such as measures of signal strength).

- Three dimensional maps enable the visualization of measures at different elevations

## 6.6	Strategies for communicating the status of connectivity

Editor’s note: Contributions are invited

6.6.1	   From Regulators’ view

Connectivity assessment Interface for regulators should be able to accommodate performance and coverage KPI derived from different database sources, such as crowdsourcing, drive or walk testing and operator network elements (OSS data) databases, operator’s official declaration.

Such informational data can provide composite indicators such as optimization priority, service availability, ranking and partial geo-marketing guidance. Such details shall provide regulators with deeper knowledge and amplify their regulatory capacity.

6.6.2	   From End Users’ view

It is critical to develop and enhance end user applications as it generates raw data constantly and reflects service quality directly from users. End user application should be informative, simple, relevant and clear. Methodologies such as ranking, scoring and different high-end maps per service can enhance the samples statistics and provide a higher quality of sample representation, it is considered as the main stream for data generation. Hence, features such as the following are recommended to have for end users:

- General Performance KPI such as Throughput and Latency

- Video Streaming Analytics: Load time (ms), Maximum Resolution (p) and Buffering %

- Service coverage with auto scale heat Map per technology

- Service availability

Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

<Appendix Title>

(This appendix does not form an integral part of this Recommendation.)

<Body of appendix I>

Bibliography

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title.

[b-ITU GCR]	International Telecommunication Union (2022), Global Connectivity Report.

____________________