| Radiocommunication Study Groups |  |
| --- | --- |
| Received: Date 20xx Source: Annex 4.4 to Document 5D/1668-E | Document XX/-E |
|  | Date 20xx |
|  | Original: English |
| India (Republic of) |  |
| AMENDMENT ON WORKING DOCUMENT TOWARDS A PRELIMINARY DRAFT NEW REPORT ITU-R M.[IMT.AAS] |  |

Introduction

India (Republic of) submitted document ANNEX 2.10: [SOURCE DOC. 5D/320] and ANNEX 2.11: [SOURCE DOC. 5D/320] at 36th meeting of  WP5D held in Oct 2020 to incorporate graphical representation of mathematical form of Advance Antenna System(AAS) given in Recommendation ITU-R M.2101.

In the document, 2D/3D plots after simulation has shown side/grating lobe as a result of various elevation angle of main beam at various dVλ factor, after certain point beyond 0.5 dVλ factor. Simulation in the document has shown the limitations of technical parameters, which can be consider to use Recommendation ITU-R M.2101within that limits for sharing studies.

Proposal

This proposal intent to bring more clarity in general way by making editorial changes and updating document indicated in turquoise colour.

Annex 4.4 to Working Party 5D Chairman’ Report

…

[Editor’s note: The Annex below has not been fully reviewed nor agreed to be included in the current Working Document.]

[ANNEX 2.10: [Source Doc. 5D/320 (IND)]

Mathematical form of composite antenna pattern given in Recommendation ITU-R M.2101 has been represented in graphical form in this proposal. Recommendation ITU-R M.2101 equations was simplified in [ANNEX 2.11] to study. Simplified equation has been simulated to see the effect of main beam when steered in vertical plane. Main beam was steered from 0° to 60° in vertical plane at dVλ factor varying from 0 to 1. 2D/3D plots after simulation gives an idea on in-band and adjacent band side/grating lobe  at various elevation of main beam by co-relating in-band and adjacent band with dVλ factor.

The mathematical form of radiation pattern from array of elements given in Recommendation ITU-R M.2101 are as per below.

# 1	Element radiation pattern (Table 3 of Rec. ITU-R M.2101-0)

Vertical radiation pattern

AE,Vθ=-min12θ-90θ3dB2, SLA

where:

θ:	elevation angle in degrees;

SLAV:	front-to-back ratio;

θ3dB:	vertical 3 dB beamwidth.

Horizontal radiation pattern

AE,Hφ= -min12φφ3dB2,AmdB

where:

φ:	azimuth angle in degrees;

Am:	front-to-back ratio;

φ3dB:	horizontal 3 dB beamwidth.

AEφ,θ=GE,max-min-AE,Hφ+AE,Vθ,Am

where:

GE,max:	Element gain (dBi).

# 2	Composite radiation pattern (Table 4 of Rec. ITU-R M.2101-0)

AA,Beamiθ,φ=AEθ,φ+10log10m=1NHn=1NVwi, n, m.νn,  m2

where:

νn,  m=ei.2π.n-1. dVλ .cosθ.ei.2π.m-1. dHλ .sin⁡(θ).sin⁡(φ)

wi, n, m=1NH.NVei.2π.n-1. dVλ .sin⁡(θi,etilt).e-i.2π.m-1. dHλ. cos(θi,etilt).sin(φi,escan)

NH:	number of horizontal elements;

NV:	number of vertical elements;

dH:	horizontal element spacing;

dV:	vertical element spacing;

λ:	frequency wavelength;

θi,etilt:	vertical electrical steering;

φi,escan:	horizontal electrical steering.

# 3	Simplification of complex mathematical form of composite radiation pattern

Mathematical form of composite radiation pattern given in Recommendation ITU-R M.2101 is complex. This complex form was simplified to visualize the graphs to study the radiation pattern to study the effect of elevation angle of main beams against dVλ factor The mathematical form after simplification can be written as follow:

AA,Beamiθ,φ=AEθ,φ+10log101NV.NH2.sinΨ1.N2sinΨ122.sinΨ2.M2sinΨ222

where:

Ψ1=2π.dVλ.cosθ+sinθi,etilt

Ψ2=2π.dHλ.sinθ.sinφ-cos(θi,etilt).sin(φi,escan)

φi,escan≠0 (i.e it can be 0.000000001)

NH:	number of horizontal elements;

NV:	number of vertical elements;

dH:	horizontal element spacing;

dV:	vertical element spacing;

λ:	frequency wavelength;

θi,etilt:	vertical electrical steering;

φi,escan:	horizontal electrical steering.

Details of simplification in [Annex 2.11].

# 4	Technical parameter

Frequency 26 GHz has been used to determine element spacing. At 0.5 dVλ factor, elements spacing is 5.8 mm. Fixed parameters taken for plot are given in Table 1.

Table 1

| Technical parameter | Value |
| --- | --- |
| Gain of single element | 5 dBi |
| No. of elements (NHX NV ) | (8 × 8) and (16 × 16) |
| Front-to-back ratio (Am) | 30 dB |
| Front-to-back ratio (SLAV) | 30 dB |
| Vertical 3 dB beamwidth (θ3dB) | 65° |
| Horizontal 3 dB beamwidth (φ3dB) | 65° |
| Element spacing (at 26 GHz and dVλ factor = 0.5) | 5.8 mm |

# 5	3D Graph showing composite radiation pattern from array of elements (8 × 8 and 16 × 16) at 0°, 30° and 60° elevation angle of main beam from horizon

MATLAB (R2008b) and MS Excel has been used for plotting the graphs. 3D Graph has been plotted at an elevation angle of main beam at 0°, 30° and 60°. In the graph Y-axis is dVλ factor, X-axis is elevation angle, and Z-axis is radiation from array of elements. Z-axis is shown perpendicular to screen which can be read as per colour bar. Y-axis (dVλfactor) has scaling from 0 to 1, and X-axis has scaling from 0 to 180 degree. Steering of azimuth angle of main beam (φi,escan) has been kept close to zero.

Figure 1

| Array of elements (8 × 8) | Array of elements (16 × 16) |
| --- | --- |
| Main beam elevation angle, θi,etilt = 0° |  |
| Main beam elevation angle, θi,etilt = 30° |  |
| Main beam elevation angle, θi,etilt = 60° |  |

# 6	2D Graph showing composite radiation pattern from array of elements (8 × 8 and 16 × 16) at 0°, 30° and 60° elevation angle of main beam from horizon at various dVλ factor

26 GHz has been taken as design frequency to study the side lobes/grating lobes on other frequencies on different elevation of main beam below horizon. In the graph Y-axis is the gain of radiation pattern (as per colour bar in Figure 1), and X-axis is elevation angle.

## 6.1	Radiation pattern at 24.25 GHz, 26 GHz and 27.5 GHz (corresponding dVλ factor 0.46, 0.5, and 0.52)

Figure 2

| Array of elements (8 × 8) | Array of elements (16 × 16) |
| --- | --- |

As per Figure 2, radiation at elevation from 0° to 180° on frequency 24.25 GHz, 26 GHz and 27.5 GHz, does not show major side lobes and grating lobes when the main beam was steered up to 60° elevation angle. Main beam is sharper and has more gain in case of 16 × 16 array elements in comparison to 8 × 8.

## 6.2	Radiation pattern at 0.65 and 0.8 dVλ factor

Figure 3

| Array of elements (8 × 8) | Array of elements (16 × 16) |
| --- | --- |

Figure 4

Figure 3 and Figure 4 show grating lobe when the main beam elevation angle is steered below the horizon. At 0.65 dVλ factor and 60° elevation angle of main beam, grating lobe was noted which has more gain than the main beam.

After giving inputs of  dVλ factor and vertical plane elevation ( θi,etilt), some results of main beam gain and grating lobe with its elevation angle are mentioned in below table based on simulation.

| Input |  | Result |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| dVλ | θi,etilt | Main beam Gain (dBi) |  | Grating lobe Gain (dBi)/elevation angle |  |  |  |
|  |  | 8x8 | 16x16 | 8x8 |  | 16x16 |  |
| 0.5 | 0° | 23.0 | 29.0 |  |  |  |  |
| 0.5 | 30° | 20.6 | 26.5 |  |  |  |  |
| 0.5 | 60° | 14.0 | 19.2 |  |  |  |  |
| 0.65 | 0° | 23.0 | 29.0 |  |  |  |  |
| 0.65 | 30° | 20.5 | 26.5 | 5.2 | 18° | 6.1 | 8° |
| 0.65 | 60° | 13.6 | 19.0 | 18.1 | 50° | 24.0 | 48° |
| 0.8 | 0° | 23.0 | 29.0 |  |  |  |  |
| 0.8 | 30° | 20.5 | 26.5 | 16.5 | 44° | 22.4 | 42° |
| 0.8 | 60° | 13.3 | 18.8 | 21.6 | 68° | 27.5 | 68° |

Following are the observations;

- After certain point beyond 0.5  dVλ factor, at certain fixed elevation angle of main beam, grating lobe gain and its elevation angle increases on increasing dVλ factor.

- At higher elevation angle of main beam after certain point beyond 0.5 dVλ factor, grating lobe gain is more prominent than lower elevation angle of main beam.

- Gain of main beam reduces on increasing elevation of main beam.

- Number of side lobes increases with increase in number of elements.

# 7	dVλ factor and corresponding operating frequency range

Spacing between elements at 26 GHz and 0.5 dVλ factor is 5.8 mm. The spacing of element remains fixed irrespective of operating frequency. This give rise to various values of dVλ factor. Table 2 shows operating frequency range and corresponding dVλ factor with reference to 26 GHz as design frequency. The frequency bands shown are below 50 GHz, identified for IMT during WRC-19.

Table 2

Element spacing 5.8 mm at frequency 26 GHz and 0.5 dVλ

| S. No | Operating frequency range (GHz) | Operating dVλ range |
| --- | --- | --- |
| 1 | 24.25 to 27.5 | 0.466 to 0.528 |
| 2 | 37 to 43.5 | 0.711 to 0.836 |
| 3 | 45.5 to 47 | 0.875 to 0.903 |
| 4 | 47.2 to 48.2 | 0.907 to 0.926 |

Figure 5

# 8	Conclusion

The study demonstrated that due to grating lobe the flexibility of steering of main beam in vertical plane reduces as dVλ factor increases, after certain point beyond 0.5 dVλ factor. So, in general it is appropriate to use the Recommendation ITU-R M.2101 for sharing studies in certain limits as resulted in simulation.

[Editor’s note: The Annex below has not been fully reviewed nor agreed to be included in the current Working Document.]

[ANNEX 2.11: [Source Doc. 5D/320 (IND)]

Simplification of antenna factor given in Table 4 of 
Recommendation ITU-R M.2101

# 1	Element pattern for antenna array model (Table 3 of Rec. ITU-R M.2101-0)

AE,Vθ=-min12θ-90θ3dB2, SLA

AE,Hφ= -min12φφ3dB2,AmdB

AEφ,θ=GE,max-min-AE,Hφ+AE,Vθ,Am	(i)

# 2	Composite antenna pattern (Table 4 of Rec. ITU-R M.2101-0)

AA,Beamiθ,φ=AEθ,φ+10log10m=1NHn=1NVwi, n, m.νn,  m2	(ii)

where:

νn,  m=ei.2π.n-1. dVλ .cosθ.ei.2π.m-1. dHλ .sin⁡(θ).sin⁡(φ)

wi, n, m=1NH.NVei.2π.n-1. dVλ .sin⁡(θi,etilt).e-i.2π.m-1. dHλ. cos(θi,etilt).sin(φi,escan)

# 3	Antenna factor simplification

The product of super position vector (νn,  m) and weighting (wi, n, m) is given by

wi, n, m.νn,  m=1NH.NVei.2π.n-1. dVλ .sinθi,etilt.e-i.2π.m-1. dHλ. cos(θi,etilt).sin(φi,escan).ei.2π.n-1. dVλ .cosθ.ei.2π.m-1. dHλ .sin⁡(θ).sin⁡(φ)

wi, n, m.νn,  m=1NH.NVei.2π.n-1. dVλ .cosθ.ei.2π.n-1. dVλ .sinθi,etilt.ei.2π.m-1. dHλ .sin⁡(θ).sin⁡(φ).e-i.2π.m-1. dHλ. cos(θi,etilt).sin(φi,escan)

wi, n, m.νn,  m=1NH.NVei.n-1.2π. dVλ .( cosθ+.sinθi,etilt).ei.m-1.2π. dHλ .((sin⁡(θ).sin⁡(φ) -. cos(θi,etilt).sin(φi,escan))

wi, n, m.νn,  m=1NH.NVei.n-1.Ψ1.ei.m-1.  Ψ2	(iii)

where:

Ψ1=2π.dVλ.cosθ+sinθi,etilt;

Ψ2=2π.dHλ.sinθ.sinφ-cos(θi,etilt).sin(φi,escan)

Summation of elements and putting eq.(iii) in 2nd part of  (ii)

n=1Nm=1Mwi, n, m.νn,  m=n=1Nm=1M1NH.NVei.n-1.Ψ1.ei.m-1.  Ψ2

n=1Nm=1Mwi, n, m.νn,  m=1NV.NHn=1Nei.n-1.Ψ1.m=1Mei.m-1.Ψ2

n=1Nm=1Mwi, n, m.νn,  m=1NV.NHAFN . AFM

where:

AFN=n=1Nei.n-1.Ψ1;  AFM=m=1Mei.m-1.Ψ2

Factor AFN and AFM are simplified:

AFN=n=1Nei.n-1.Ψ1	(iv)

Expansion of equation (iv):

AFN=e0+ei.Ψ1+ei.2Ψ1+    .    .    .    + ei.Ψ1.N-1

AFN=1+ei.Ψ1+ei.2Ψ1+    .    .    .    + ei.Ψ1.N-1

AFN-1=ei.Ψ1+ei.2Ψ1+    .    .    .    + ei.Ψ1.N-1	(v)

Equation (iv) can also be written in this form:

AFN.ei.Ψ1=n=1Nei.n.Ψ1

Expansion of above equation:

AFN.ei.Ψ1=ei.Ψ1+ei.2Ψ1+  .    .     .      + ei.Ψ1.N-1+ei.Ψ1.N	(vi)

Put equation (v) in (vi):

AFN.ei.Ψ1=AFN-1+ei.Ψ1.N

AFN.ei.Ψ1-AFN=-1+ei.Ψ1.N

AFN.ei.Ψ1-1=ei.Ψ1.N-1

AFN=ei.Ψ1.N-1ei.Ψ1-1

AFN=ei.Ψ1.N2.ei.Ψ1.N2-ei.Ψ1.N2.e-i.Ψ1.N2ei.Ψ12.ei.Ψ12-ei.Ψ12.e-i.Ψ12.2i2i

AFN=ei.Ψ1.N2ei.Ψ12ei.Ψ1.N2-e-i.Ψ1.N22iei.Ψ12-e-i.Ψ122i

AFN=ei.Ψ1.(N-1)2.sinΨ1.N2sinΨ12

If the reference point is the physical centre of the array, the factor (AFN)above reduces to:

A EQ FN=ei.Ψ1.(N-1)2.sinΨ1.N2sinΨ12

Similarly,

A EQ FM=ei.Ψ2.(M-1)2.sinΨ2.M2sinΨ22

Now equation (ii) can be written as below:

AA,Beamiθ,φ=AEθ,φ+10log101NV.NH.A EQ FN.A EQ FM2

AA,Beamiθ,φ=AEθ,φ+10log101NV.NH.sinΨ1.N2sinΨ12.sinΨ2.M2sinΨ222

AA,Beamiθ,φ=AEθ,φ+10log101NV.NH2.sinΨ1.N2sinΨ122.sinΨ2.M2sinΨ222

where:

Ψ1=2π.dVλ.cosθ+sinθi,etilt

Ψ2=2π.dHλ.sinθ.sinφ-cos(θi,etilt).sin(φi,escan)

…

____________________

Constantine A. Balanis, Antenna theory analysis and design, Third edition, Chapter 6 (6-10a)