|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  |  | SG11-TD325/GEN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 11 |  |  |
|  |  |  |  | Original: English |  |  |
| Question(s): |  |  | 12/11 |  | Geneva, 19-28 February, 2025 |  |
| TD |  |  |  |  |  |  |
| Source: |  |  | Editors |  |  |  |
| Title: |  |  | Output - draft new recommendation ITU-T Q.MUD_IoT “Framework for testing and monitoring IoT devices & networks using Technical Requirements from Manufacturer Usage Description (MUD)” (Geneva, 19-28 February 2025) |  |  |  |
| Contact: |  |  | Anshul Kumar Gupta TEC, DoT Ministry of Communications, India |  |  | Tel: +919785951159 E-mail: anshulkr.gupta@gov.in |
| Contact: |  |  | Krishna Kumar Lahoti Criterion Network Labs (CNLABS) India |  |  | Tel: +91 7207580125 E-mail: krishna@cnlabs.in |
| Abstract: |  | This document contains output baseline text of new work ITU-T Q.MUD_IoT “Framework for testing and monitoring IoT devices & networks using Technical Requirements from Manufacturer Usage Description (MUD)”. |  |  |  |  |

This output document is based on the input document as follows reviewed at Q12/11meeting (Geneva, 19-28 February 2025).

| Document Number | Source | Title | Meeting results |
| --- | --- | --- | --- |
| SG11-C118 | Ministry of Communications, India | Proposal to advance baseline text of new work item ITU-T Q.MUD_IoT "Framework for testing and monitoring IoT devices & networks using Technical Requirements from Manufacturer Usage Description (MUD)" | Accepted with some modifications. 1. In section 3.1: Terms Defined Elsewhere: Addition of definition of MUD as per RFC 8520 2. In section 9.2: In All References section, only MUD RFC 8520 sections are kept and removed other RFC sections. Replaced MUD with RFC 8520 in all References so that it can be readily referenced by anyone. |

Draft new Recommendation ITU-T Q.MUD_IoT

Framework for testing and monitoring IoT devices & networks using Technical Requirements from Manufacturer Usage Description (MUD)

Summary

IoT technology has permeated diverse sectors, facilitating automation, data-driven insights, and operational efficiencies. However, this widespread adoption has introduced challenges related to security vulnerabilities, interoperability complexities, and network performance optimization. The pervasive integration of IoT (Internet of Things) devices across industries underscores the pressing need for robust network infrastructures that prioritize reliability, interoperability, and security.

Manufacturer Usage Description (MUD), as specified in IETF RFC 8520, emerges as a foundational tool in fortifying IoT ecosystems, offering not only security enhancements but also significant benefits in addressing interoperability challenges and optimizing network performance.

The main objective of the IETF's Manufacturer Usage Description (MUD) Specification is to provide a means for end devices to signal to the network what sort of access and network functionality they require to properly function. MUD can offer a standardized approach for manufacturers to specify the necessary network communications for a IoT device to operate according to their manufacturers' intentions. IETF MUD framework requires standardized approach for generation of operational behavioural profile of IoT devices called MUD profiles and use of these profiles to secure their network and monitor the runtime behaviour of IoT devices. By implementing MUD, the network can automatically allow only the essential traffic required for the device to operate as intended, while blocking any unauthorized communication attempts.

This recommendation aims to establish a standardized testing framework for testing and monitoring IoT devices & networks to ensure conformance with the IETF RFC 8520 (MUD). It consists of testing requirements, testing procedures and expected behaviour to verify an IoT device’s conformance to MUD Specification. Compliance with MUD is crucial and serves as a key requirement for IoT Security, thereby mitigating potential risks associated with unauthorized network access and cyber threats.

Keywords:

Internet of things, Test specification, Manufacturer Usage Description

Draft new Recommendation ITU-T Q.MUD_IoT

Framework for testing and monitoring IoT devices & networks using Technical Requirements from Manufacturer Usage Description (MUD)

# 1	Scope

This draft recommendation proposes a standardized framework for testing and monitoring IoT devices & networks using technical requirements from the Manufacturer Usage Description (MUD). This recommendation will also focus on test scenarios and compliance criteria relevant to the behaviour and communication patterns of IoT devices with mandatory requirements of MUD RFC 8520.

It does not cover test scenarios that verify implementation of other network components such as MUD Controller/manager.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.2060]	Recommendation ITU-T Y.2060, Overview of Internet of Things.

[IETF RFC 8520]	RFC 8520, Manufacturer Usage Description Specification

<TBD>

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

- Manufacturer Usage Description [IETF RFC 8520]: A component-based architecture that is designed to provide a means for end devices to signal to the network what sort of access and network functionality they require to properly function.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

None.

<TBD>

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

MUD		Manufacturer Usage Description

IETF		Internet Engineering Task Force

ACL		Access Control List

ACE		Access Control Entry

DHCP 		Dynamic Host Configuration Protocol

URL		Unique Resource Locator

RFC		Request for Comments

LLDP		Link Layer Discovery Protocol

TLV		Type Length Value

CA		Certificate Authority

URN 		Universal Resource Name

UTF		Unicode Text Format

<TBD>

# 5	Conventions

None.

# 6	Overview

With the growing usage and deployments of IoT devices & networks, there is a need to address potential interoperability issues, network performance optimization challenges and security concerns. MUD plays a pivotal role in addressing some of these issues by requiring IoT devices to specify their network communication requirements, thereby allowing only essential traffic for IoT devices’ intended use.

The proposed standardized test framework for testing and monitoring of IoT devices & networks using technical requirements from MUD encompasses the test requirements, test procedures and expected behaviours for verifying compliance of an IoT device with MUD specification as prescribed by IETF in RFC 8520. Devices will be verified to check if they meet the mandatory clauses from the MUD specification.

# 7	Description of Manufacturer Usage Description and its Test Framework

# As stated in RFC 8520, the goal of Manufacturer Usage Description (MUD) is to provide a means for end devices to signal to the network what sort of access and network functionality they require to properly function. MUD intends to do the following:

- Substantially reduce the threat surface on an IoT device

- Scale network policies for ever-increasing number of types of IoT devices

- Address vulnerabilities in a faster way

- Reduce cost of implementing such a robust security system

- Provide a means of extensibility for manufacturers to express other device capabilities or requirements.

# MUD is not intended for general purpose computers and even for devices that have single or small number of uses but with broad communication patterns. It is intended only for IoT devices that have a specific use-case(s) with limited, well-defined communication patterns.

# MUD meets its intended objectives by asking the IoT devices to specify its network communication requirements in a MUD URL that can be emitted by the IoT device using any one of the three methods:

- Using DHCP Options message sent by the DHCP client to the DHCP server while seeking an IPv4 or IPv6 address. The DHCP server may itself act as a MUD Manager or pass on the URL to the actual MUD manager in the network

- Using X.509 Constraint – security certificate-based approach.

- Using Link Layer Discovery Protocol (LLDP)

# This work item’s objective is to verify an IoT device’s compliance with MUD specifications by verifying whether an IoT device emits the MUD URL properly and that the contents of the MUD URL are in accordance with the requirements of RFC 8520. It does NOT verify the implementation of other network components such as MUD Manager or MUD Controller.

# The following sections describe the test framework’s topology and applicable test scenarios, procedures and expected behaviours to verify compliance with MUD Specification.

<TBD>

# 8. 	Test Framework Architecture

Following diagram depicts the test framework’s topology.

- DUT = Device Under Test i.e., IoT Device

- Analyser server with abilities to capture and analyse packets to and from DUT.

- Network switch/gateway with Wi-Fi and Ethernet interfaces, configured to mirror DUT packets to the Analyser server.

- DUT has the capacity to emit a MUD URL.

<TBD>

9	Test Framework for Verifying Conformance to MUD Specification

The primary objective is to verify if the IoT device can emit a MUD URL in accordance with the requirements prescribed in MUD RFC 8520. For this, the following sections provides the details on test requirements, their mandatory or optional nature, basic test procedure, and expected outcomes.

9.1 Testing Requirements for MUD Conformance

- Seeking MUD URL String – via DHCP requests, LLDP frames, and X.509 Certificates

- Checking Consistency of Emitted MUD URL Strings

- Validating MUD URL String

- Verifying if a MUD file can be retrieved

- Verifying the signature of the MUD file – method of signature and the content

- Verifying the Certificate Validity of MUD file

- Verifying the encoding method of the MUD file

- Verifying the usage of nodes in the MUD file

- Verifying of the declaration of extensions in the MUD file – declared and undeclared extensions, both must be verified

- Verifying extensions are standardised by IETF standards

- Verifying the Direction of Individual Policies in the MUD file – direction and position

- Verifying the pattern cache-validity of MUD Model

- Verifying the node systeminfo in the MUD file

- Verifying the optional fields in the MUD file

- Verifying the format of local addresses in the MUD file and URL of the node “controller”. This requirement is optional.

- Verifying the usage of ACL leaf nodes in the MUD file

- Verifying ACL policies in the MUD file – inbound and outbound

- Verifying the usage of domain names in the ACEs

- Verify dnsname extensions for endpoints in ACEs are configured by domain names

- Verifying the consistency of policies in the ACLs – from-device and to-device

- Verifying the usage of “direction-initiated” for ACEs in MUD file

- Verifying the actions of ACEs in the MUD File

- Verifying the count of ACEs in the MUD file

<TBD>

9.2 Test Procedures and Expected Behaviour for MUD Conformance

## Test MUD.1.1.1: Seeking MUD URL String

Purpose: To verify the ability of DUT to emit a MUD URL string.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 1.5, Section 10, Section 11 and Section 12

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: DHCP requests

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Capture DHCPv4 or DHCPv6 packets sent by DUT and seek a MUD URL for option code 161 in DHCPv4 requests or option code 112 in DHCPv6 requests. | The MUD URL string can be extracted from DHCPv4 or DHCPv6 requests given the corresponding DHCP option code. |

#### Part B: LLDP frames

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 2. | Capture LLDP frames sent by DUT and verify they contain the TLV Type=127, OUI=00 00 5E, subtype=1 and the MUD string as a list of octets | The MUD URL string can be decoded by extracting octet values in the LLDP frames from DUT. The expected format of LLDP packets is shown as follows. |

#### Part C: X.509 Certificates

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 3. | Capture a certificate presented by DUT (e.g., via TLS protocol), look for an X.509 non-critical extension, and verify if it contains a URL. | The MUD URL string can be extracted from the X.509 non-critical extension of the retrieved certificate. |

Possible Problems: A manufacturer may choose other means to emit MUD URL. In this case, the usage of protocols and methodology must be declared.

## Test MUD.1.1.2: Checking Consistency of Emitted MUD URL Strings

Purpose: Verify that the MUD URL strings emitted by DUT are identical.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 10.1

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Compare the extracted MUD URL strings

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Capture the generated DHCP requests (v4 and/or v6), LLDP frames, and X.509 certificates to check the emitted MUD URLs. | The MUD URL strings extracted from various packet types must be identical |

Possible Problems: None.

## Test MUD.1.1.3: Validating MUD URL String

Purpose: Verify whether the emitted MUD URL is in a valid format.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 1.5, Section 6, Section 10, Section 11, and Section 12

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A:  Verify the MUD URL string is valid

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Test whether the MUD URL string starts with “https” instead of “http”. | The MUD URL string must start with “https”. |
| 2. | Check the length of the MUD URL string is less than a threshold | The length of the MUD URL string must not exceed 255 octets for both DHCP (v4 and/or v6) requests and certificates. Additionally, the length of the MUD URL string must not exceed 253 octets for LLDP frames. |
| 3. | Check whether the URL string has the following structure: | The MUD URL string is expected to adhere to the standard structure. |

https://domainName/<string>/IRIs

where the string only contains legal characters and the domainName should be a valid format with the end such as “.com”, the IRIs represents the descriptor of resource, i.e. the name of the MUD file.

Possible Problems: None.

## Test MUD.1.1.4: Verifying if a MUD File Can be Retrieved

Purpose: Verify if a MUD file can be retrieved from the specified HTTPS server using the emitted MUD URL.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 1.6

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Check whether the MUD file can be retrieved

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Using the GET method, verify whether the MUD file can be retrieved from the HTTPS server specified by the MUD URL. | The GET request for the MUD URL must be successful. |

Possible Problems: None.

## Test MUD.1.1.5: Verifying Presence and Structure of Digital Signature of MUD File

Purpose: Verify that the retrieved MUD file contains a digital signature and is correctly signed using Cryptographic Message Syntax (CMS).

Functionality Tag: Optional

Reference:

- [RFC 8520] – Section 13.1 and Section 13.2

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Verify the signature method of the MUD file

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Retrieve the digital signature by obtaining the value of “mud-signature” within the MUD file. | The retrieved MUD file should have the node “mud-signature” and the signature file can be retrieved from its value. |
| 2. | Check whether the MUD file is signed using DER-encoded CMS, a standard way to digitally sign and encrypt data. | The signature file is a binary object with file type as “.p7s” and in DER-encoded format. |

Possible Problems: None.

## Test MUD.1.1.6: Verifying Content Type and Certificates in Digital Signature of MUD File

Purpose: Verify that content type and certificates within the digital signature of the retrieved MUD file are correct and valid.

Functionality Tag: Optional

Reference:

- [RFC 8520] – Section 11

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Verify the certificates within the signature

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Extract the signing certificates in the signature file. | Certificates must exist within the signature file. |
| 2. | Examine the Key Usage Extension pattern within the signing certificate to confirm whether the bit “digitalSignature” is set to 0. | The bit “digitalSignature” must be set to 0 in the Key Usage Extension pattern. |
| 3. | Check whether the “id-pe-mudsigner” extension is present in the X.509 section, and verify if its content matches the information within the subject field of the certificate. | The content of subject in the certificate of the signature must match the content of the “id-pe-mudsigner” extension in the retrieved X.509 certificate. |

#### Part B: Verify the content type of the signature

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 4. | Verify if the content-type “id-ct-mud” is used as the content type for the signed data. | The content-type is expected to have the object identifier “id-ct” equal to 41. |

Possible Problems: None.

## Test MUD.1.1.7: Verifying Integrity of Digital Signature for MUD File

Purpose: Verify the validity of the digital signature associated with the retrieved MUD file to ensure its integrity and authenticity, thereby confirming that the file has not been tampered with.

Functionality Tag: Optional

Reference:

- [RFC 8520] – Section 13.1, Section 13.2 and Section 11

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Verify the integrity of digital signature

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Extract the encrypted digital signature and the signer’s information, including a public key, a hash function, and a message digest (MD). |  |
| 2. | Use the signer’s public key and decrypt the digital signature to retrieve the unencrypted hash value (H1). |  |
| 3. | Compare the unencrypted hash value with the message digest. | The unencrypted hash value (H1) must match the signer’s message digest (MD). |

#### Part B: Verify the integrity of MUD file

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 4. | Calculate the hash value (H0) of the retrieved MUD file using the signer’s hash function. |  |
| 5. | Compare the calculated hash value (H0) with the signer’s message digest (MD). | The hash value of the MUD file (H0) must match the signer’s message digest (MD). |

Possible Problems: None.

## Test MUD.1.1.8: Verifying Certificate Validity of MUD File Signature

Purpose: Verify the validity of the certificate from the retrieved MUD signature.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 1.6, Section 13.1

Test Setup: Common Test Topology is performed.

| X.509 Certificate |  |
| --- | --- |
| Patterns to be verified | Expected Results |
| The digital signature applied by the Certificate Authority (CA) using its own private key on the certificate needs to be verified using the CA's corresponding public key | When the certificate data, excluding the signature, is encrypted using the algorithm specified in the certificate, the result should match the value obtained by decrypting the digital signature with the CA's public key |
| The validity period of the X.509 certificate does not expire | The current system time should not before the start date or after the end date shown on the certificate |
| The revocation information of the certificate needs to be checked to ensure the certificate is not revoked | The certificate is still active and not revoked |
| The validity of each certificate along the chain of trust associated with the Certificate Authority (CA) responsible for issuing the certificate need to be checked | The verification process is successfully completed when it reaches the root of the certificate chain without encountering any self-signed certificates, and all certificates in the chain are confirmed as valid |

### Procedure:

#### Part A: Verify the integrity of digital signature

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Obtain the intermediate certificate from the retrieved signature. | There should include an intermediate X.509 certificate from the retrieved signature. |
| 2. | Verify the obtained certificate is valid using the matching rules listed above. | The obtained X.509 certificate must be valid. |

Possible Problems: None.

## Test MUD.1.2.1: Verifying MUD File Format

Purpose: Verify whether the content of the retrieved MUD file is in JSON format.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 1.6, Section 2 and Section 17

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Checking the file format used by the MUD file

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Examine the syntax and structure of the retrieved MUD file. | The content of the retrieved MUD file is expected to be in JSON format. |

Possible Problems: None.

## Test MUD.1.2.2: Verifying Encoding Method of MUD File

Purpose: Verify that the retrieved MUD file is encoded in UTF-8.

Functionality Tag: Optional

Reference:

- [RFC 8520] – Section 17.5

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Checking the encoding method used by the MUD file

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Examine the encoding method applied to the retrieved MUD file. | The retrieved MUD file must be encoded in UTF-8. |

Possible Problems: None.

## Test MUD.1.2.3: Verifying Usage of Nodes in MUD File

Purpose: Verify that the retrieved MUD file includes only certain specified nodes, except optional nodes defined by extensions.

Functionality Tag: Mandatory

### Reference:

- [RFC 8520] – Section 2, Section 2.1 and Section 3.9

Test Setup: Common Test Topology is performed.

| MUD file structure | Specified Node Names |
| --- | --- |
| “mud” container | mud-version, mud-url, last-update, mud-signature, cache-validity, is-supported, systeminfo, mfg-name, model-name, firmware-rev, software-rev, documentation, extensions, from-device-policy, to-device-policy |
| Augmentation to “acl” | manufacturer, same-manufacturer, model, local-networks, controller, my-controller, direction-initiated |

### Procedure:

#### Part A: Check whether the MUD file only includes certain specified nodes

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Retrieve all node names from the retrieved MUD file. |  |
| 2. | Compare all node names with the table above. | The nodes utilized in the retrieved MUD file must be restricted to those listed in the table above. |

Possible Problems: None.

## Test MUD.1.2.4: Verifying Extension Declaration in MUD File

Purpose: Verify whether extensions used in the retrieved MUD file are explicitly mentioned in the “extension” list. Additionally, ensure that undeclared MUD extensions are not utilized in the retrieved MUD file.

Functionality Tag: Optional

Reference:

- [RFC 8520] – Section 3.9, Section 14, Section 17.8

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Verify that all the extensions used in the MUD file are explicitly named in the “extension” list

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Compare each extension with those listed in the “extension” list. | The extensions used must match those listed in the “extension” list. |

#### Part B: Verify the usage of undeclared MUD extensions in the MUD file

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 2. | Obtain the nodes of the MUD file. |  |
| 3. | Verify the existence of undeclared MUD extensions within the nodes of the retrieved MUD file. | The retrieved MUD file must not use undeclared MUD extensions in its nodes. |

Possible Problems: None.

## Test MUD.1.2.5: Verifying Extensions are Standardized by IETF

Purpose: Verify if all extensions declared and used in the MUD file have undergone the IETF standardization process and are registered with the IANA.

Functionality Tag: Optional

Reference:

- [RFC 8520] – Section 14

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Check whether all the extensions used in the MUD file are standardized by IETF

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Obtain all the extensions declared and used. |  |
| 2. | Verify if the structure of each extension conforms to the format specified as follows: | All extensions used in the retrieved MUD file must be referenced to a published RFC document. |

Extension name: UTF-8-encoded string, not to exceed 40 characters. Each Extension must refer to a certain RFC documentation

Possible Problems: None.

## Test MUD.1.3.1: Verifying Direction of Individual Policies in MUD File

Purpose: Verify that the MUD file explicitly delineates communication directions in each policy, indicating what is to be permitted or denied in either direction of communication, i.e., to-device and from-device. Additionally, verify the accurate specification of policies for their respective communication directions.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 3.3, Section 7

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Validate directional policies

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Verify the retrieved MUD file explicitly declares the “from-device-policy” and “to-device-policy” nodes. | The “from-device-policy” node and “to-device-policy” node must be declared. |

#### Part B: Validate the correct policy positions

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 2. | Examine individual ACLs within the “from-device-policy” and “to-device-policy” to ensure that the direction of each ACL aligns with the corresponding policy. | The ACLs within the “from-device-policy” are expected to specify communications originated from the DUT and the ACLs within the “to-device-policy” are expected to specify communications destined to the DUT. |

Possible Problems: None.

## Test MUD.1.3.2: Verifying Node cache-validity in MUD File

Purpose: Verify that the “cache-validity” value in the retrieved MUD file is within the valid range.

Functionality Tag: Optional

Reference:

- [RFC 8520] – Section 3.5, Section 7

Test Setup: Common Test Topology is performed.

cache-validity Type: uint8 Value: must not exceed 168, recommended to be no less than 24, should be no shorter than any period determined through HTTP caching directives.

| HTTP caching directives |  |
| --- | --- |
| Headers in GET response | How to derive the value of HTTP caching-directives |
| Expires | = the value of “Expires” header - the system time that the response is received |
| Cache-Control | = the value of “maxage” attribute - the value of “age” attribute |
|  | = if the attribute “s-maxage” exists, it takes the highest priority. In this case, the values specified by the max-age or Expires are ignored, and the value of “s-maxage” is used |

### Procedure:

#### Part A: Verify the value of the node “cache-validity” specified in the MUD file

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Extract the value of the “cache-validity “node. |  |
| 2. | Obtain the value the HTTP caching directives using the table above. | The value of the HTTP caching directives can be retrieved. |
| 3. | Compare the value of cache-validity with the valid range [24, 168] against the values obtained from the HTTP caching directives. | The value of the cache-validity must be equal to or smaller than 168 and is recommended to be greater than 24. Additionally, the value of the cache-validity should be greater than the value of the retrieved HTTP caching directives. |

Possible Problems: None.

## Test MUD.1.3.3: Verifying Node systeminfo in MUD File

Purpose: Verify that the values of the “systeminfo” node in the retrieved MUD file, describing the DUT, are appropriately set.

Functionality Tag: Optional

Reference:

- [RFC 8520] – Section 3.7, Section 7

Test Setup: Common Test Topology is performed.

systeminfo Type: UTF-8 Length: should not exceed 60 characters worth of display space.

### Procedure:

#### Part A: Verify the type and length of the “systeminfo” values

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Obtain the values of the “systeminfo” node. |  |
| 2. | Check the Type (encoding method). | The encoding method used is UTF-8. |
| 3. | Check the Length of description. | The length of the “systeminfo” value should not exceed 60 characters, excluding white spaces. |

Possible Problems: None.

## Test MUD.1.3.4: Verifying Optional Fields in MUD File

Purpose: Verify that the “firmware-rev” and “software-rev” fields are not included in the MUD file if the device can be upgraded, but the MUD URL cannot be upgraded.

Functionality Tag: Optional

Reference:

- [RFC 8520] – Section 3.6, Section 3.8

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Check the optional fields of the MUD file

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Obtain the Boolean value of the node “is-supported” in the MUD file. |  |
| 2. | Check whether the optional fields “firmware-rev” and “software-rev” are present in the MUD file. | If the Boolean value of “is-support” is false, the optional fields “firmware-rev” and “software-rev” must not exist in the MUD file. |

Possible Problems: None.

## Test MUD.1.4.1: Verifying Format of Local Addresses in MUD File

Purpose: To ensure that local addresses in the “local-network” attribute in the ACEs from the retrieved MUD file conform to the acceptable formats, including IPv4/v6 addresses with prefixes and masks (e.g., 255.255.255.255/32).

Functionality Tag: Optional

Reference:

- [RFC 8520] – Section 7

Test Setup: Common Test Topology is performed.

| Local Addresses |  |
| --- | --- |
| Valid IPv4 Addresses format | Prefix/mask: xxx.xxx.xxx.xxx/yy where xxx is a decimal number within the range [0,255], separated by the dots, followed by a slash and yy, which can be either the CIDR (Classless Inter-Domain Routing) notation that is a decimal number within the range [0, 32] or the Traditional Subnet Mask Format that is the same format as the prefix pattern. |
| Valid IPv6 Addresses format | Prefix/mask: xxxx:xxxx:xxxx:xxxx:xxxx:xxxx:xxxx:xxxx/yyy where xxxx is hexadecimal number ranging from 0 to FFFF, separated by the colons, followed by a slash the and yyy, which is the CIDR (Classless Inter-Domain Routing) notation that is a decimal number within the range [0, 128]. |

### Procedure:

#### Part A: Verify local address formats

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Retrieve the ACEs from the MUD file that reference “local-network”. |  |
| 2. | Examine the IP addresses within those ACEs to confirm they include valid formats with prefixes and masks. | Local addresses in the “local-network” attribute in the ACEs may be specified with valid prefixes and masks, as indicated in the table above. |

Possible Problems: None.

## Test MUD.1.4.2: Verifying URLs of Controllers in MUD File

Purpose: Examine the controller URLs to ensure they are valid or in the form of URNs.

Functionality Tag: Optional

Reference:

- [RFC 8520] – Section 4.6, Section 17.7

Test Setup: Common Test Topology is performed.

| controller |  |
| --- | --- |
| MUD Well-Known Universal Resource Name (URNs) | “urn:ietf:params:mud:dns”, “urn:ietf:params:mud:ntp” |
| Valid URL form | “http[s]://domainName/<string>”, where the domainName is in a valid format with endings, such as “.com” |

### Procedure:

#### Part A: Check the URL of the node “controller”

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Obtain the URL of the “controller” nodes in the ACLs. |  |
| 2. | Compare these URL values with the MUD well-known URNs. | The controller URL may take the form of MUD well-known URNs. |
| 3. | Verify the validity of values if they are in the form of URLs. | The controller URL may take the form of a valid URL. |

Possible Problems: None.

## Test MUD.1.5.1: Verifying Usage of ACL Leaf Nodes in MUD File

Purpose: Verify that the ACL leaf nodes include the required attributes, specifically: “name” of the ACL, “type”, “name” of the ACEs, and TCP and UDP source and destination port information from the retrieved MUD file.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 2

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Verify inclusion of ACL attributes in the MUD File

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Examine all ACL entries within the retrieved MUD file to ensure they include the following attributes: “name” of the ACL, “type”, “name” of the ACEs, and source and destination port information for ACEs based on TCP or UDP. | Each ACL should contain the specified attributes: “name” of the ACL, “type”, “name” of the ACEs, and source and destination port information for ACEs based on TCP/UDP. |

Possible Problems: None.

## Test MUD.1.5.2: Verifying ACL Policies in MUD File

Purpose: Verify that each ACE policy in the MUD file explicitly defines rules for either inbound or outbound traffic relative to the DUT.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 7

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Verify Inbound and/or Outbound Traffic Policies in ACLs

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Examine each ACE in the retrieved MUD file to verify that there are policies governing either inbound (to-device) or outbound (from-device) traffic in relation to the DUT by inspecting the “name” attribute of each ACL. | Each ACL should clearly specify policies for either inbound or outbound traffic with respect to the DUT in their “name” attribute which contains either “from” or “to”. |

Possible Problems: None.

## Test MUD.1.5.3: Verifying Usage of Domain Names in ACEs

Purpose: Verify that ACEs with “ietf-acldns:src-dnsname” and “ietf-acldns:dst-dnsname” for endpoints are configured by valid domain names.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 2, Section 8

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Verify dnsname extensions for endpoints in ACEs are configured by domain names

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Retrieve those ACEs in the MUD file that specify the endpoints using DNS attributes: “ietf-acldns:src-dnsname” or “ietf-acldns:dst-dnsname” |  |
| 2. | Confirm the values of these attributes are set by valid domain names, not IP addresses. | Each ACE that utilizes the DNS extensions should specify the endpoint address using a domain name, not an IP address. |

Possible Problems: None.

## Test MUD.1.5.4: Verifying Consistency of Policies in ACLs

Purpose: Verify that attributes of the policy specified by each ACE is consistent with their intent in the MUD file.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 8.1 and Section 8.2

Test Setup: Common Test Topology is performed.

| Device policies | Allowed attributes | Disallowed attributes |
| --- | --- | --- |
| from-device | dst-dnsname, destination-ipv4-network, destination-ipv6-network | src-dnsname, source-ipv4-network, source-ipv6-network |
| to-device | src-dnsname, source-ipv4-network, source-ipv6-network | dst-dnsname, destination-ipv4-network, destination-ipv6-network |

### Procedure:

#### Part A: Verify the policy criteria used for “from-device” policies.

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Examine all ACEs within the ACL container in the MUD file where the policy is defined as “from-device” |  |
| 2. | Verify each ACE using “from-device” employs only allowed attributes: “dst-dnsname”, “destination-ipv4-network” or “destination-ipv6-network” to signify the destination endpoint and does not utilize any of the disallowed attributes listed in table above. | All “from-device” ACEs MUST utilize allowed attributes: “dst-dnsname”, “destination-ipv4-network” or “destination-ipv6-network" for endpoints, and there is no instance of disallowed attributes in this context. |

#### Part B: Verify the policy criteria used for “to-device” policies.

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 3. | Examine all ACEs within the ACL container in the MUD file where the policy is defined as “to-device” |  |
| 4. | Verify each ACE using “to-device” employs only allowed attributes: “src-dnsname”, “source-ipv4-network” or “source-ipv6-network” to signify the source endpoint and does not utilize any of the disallowed attributes listed in table above. | All “to-device” ACEs MUST utilize allowed attributes: “src-dnsname”, “source-ipv4-network” or “source-ipv6-network" for endpoints, and there is no instance of disallowed attributes in this context. |

Possible Problems: None.

## Test MUD.1.5.5: Verifying Usage of “direction-initiated” for ACEs in MUD File

Purpose: Verify that the “direction-initiated” attribute is exclusively applied to TCP-based ACEs in the retrieved MUD file.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 4.8, Section 9

Test Setup: Common Test Setup is performed.

### Procedure:

#### Part A: Verify “direction-initiated” is applied TCP connections.

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Ensure that the attribute “direction-initiated” is only associated with TCP-based ACEs in the retrieved MUD file. | The “direction-initiate” must only apply to TCP communications (i.e., ACEs with protocol=6). |

Possible Problems: None.

## Test MUD.1.5.6: Verifying Actions of ACEs in MUD File

Purpose: Verify that each ACE in the retrieved MUD file specifies only “accept” or “drop” as actions without additional information.

Functionality Tag: Mandatory

Reference:

- [RFC 8520] – Section 2

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Verify the “actions” attribute of ACEs.

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Extract all ACEs from the MUD file. |  |
| 2. | Verify that the “actions” attribute of each ACE in the MUD file specifies only “accept” or “drop”. | Each ACE in the retrieved MUD file should only specify “accept” or “drop” for the “actions” attribute. |

Possible Problems: None.

## Test MUD.1.5.7: Verifying Count of ACEs in MUD File

Purpose: Verify that the number of ACEs in the retrieved MUD file is “relatively” small.

Functionality Tag: Optional

Reference:

- [RFC 8520] – Section 16

Test Setup: Common Test Topology is performed.

### Procedure:

#### Part A: Check the number of ACEs in the MUD file

| Step | Action | Expected Behavior |
| --- | --- | --- |
| 1. | Obtain all the ACEs in the retrieved MUD file. |  |
| 2. | Count the number of ACEs in the retrieved MUD file. | The number of ACEs in the retrieved MUD file should be relatively small (e.g., less than 50). |

Possible Problems: None.

Appendix I

(This part  does not form integral part of this recommendation)

Relevance of the Recommendation and Gap-analysis

Most IoT devices are designed to be low-cost, are resource-constrained, and have limited functionality. Thus, it is not realistic to address IoT device vulnerabilities through the usual state-of-the-art security mechanisms like traditional VAPT, fuzzing tools, and Common Criteria testing as it is costly and complicated for such tiny devices. These mechanisms may be well suited to full-featured devices like laptops, phones or even Routers, Switches, etc., but not for IoT devices like security cameras, baby monitors, connected thermostats, lighting control systems and other smart home sensors/devices. Users often lack awareness of the IoT devices present on their networks and lack effective means for managing their access throughout their life cycles. If the security of such IoT devices is not addressed, they can be prone to network-based attacks such as DDoS (Ex: Mirai), as attackers can gain access to and attack an IoT device with known vulnerabilities using techniques such as botnets. This could have a catastrophic impact on organizations, communities and individuals.

The Manufacturer’s Usage Description (MUD) Specification defined in IETF RFC 8520 is a practical and effective solution that addresses the challenges described above for ensuring IoT security. MUD has the capability to empower networks to automatically authorize each IoT device to transmit and receive only the necessary traffic for its designated purpose, while blocking any other communication with the device. By preventing unauthorized traffic to and from a device, the approach detailed in this guide effectively minimizes the potential for an IoT device to be compromised by network-based attacks, as well as limits the capacity of compromised devices to engage in network-based attacks like DDoS campaigns.

One of the main goals of Q12/11 is to develop the tests and the methodology and/or mechanism for testing the IoT-based technologies and protocol. It includes the developing of the test suites to be used for testing IoT network elements, developing the methodology for security testing and test specification related to security testing of IoT, developing the test suites for testing IoT identification/authentication procedures, developing test suites for testing IoT technical solutions to be used for combating counterfeiting and etc. The importance of such testing is constantly growing due to the practical implementation of IoT in the “smart cities” and in the “smart manufacturing”.

As the MUD specification prescribed in IETF RFC 8520 is written more from a product implementation perspective, this work item, in accordance with the goals of Q12/11, aims to establish standardized testing methodologies and framework for testing IoT devices to verify their compliance with MUD specification. This work item will eventually serve as a MUD Conformance Test Specification, and could be used by the industry, academia, regulators and other bodies to verify if an IoT device implements MUD as prescribed in the IETF RFC 8520.

As MUD is a relatively new mechanism to address IoT security concerns, there are no existing work items in ITU-T or any other standard bodies related to Conformance Testing for MUD. NIST has developed a practical guide to showcase the viability and effectiveness of leveraging MUD to enhance security of IoT devices within home and small-business networks. The guide illustrates how entities can employ MUD to diminish the susceptibility of IoT devices to network-based threats like distributed denial of service attacks (DDoS) and alleviate the risks associated with the exploitation of IoT devices. However, this guide is only from a MUD implementation, deployment and its benefits perspective. It does not describe any testing mechanisms to ensure IoT device compliance with MUD.

The following table provides a gap analysis between this work item (MUD Conformance Test Specification) and other testing specifications from ITU-T SG-11.

| S. No. | SDO | Title | Status | Analysis |
| --- | --- | --- | --- | --- |
| 1 | ITU-T SG-11 | Q.4062 Framework for IoT testing | In Force | Q.4062 describes the types of the tests for the domains with multiple access technologies and specifies the test procedures and the considerations corresponding to the testing types. It does not specifically cover conformance testing requirements or specify any requirements on IoT devices to communicate their network-access requirement. Whereas, this presented draft recommendation aims to create a conformance test specification for verifying compliance with MUD specification. |
| 2 | ITU-T SG-11 | Q.4063: Framework for testing identification systems used in Internet of things | In force | Q.4063 describes a framework for testing the identity of IoT devices and the classification of IoT in terms of testing the identification systems and the relevant testing approaches. It does not cover conformance testing or any requirements on IoT devices to communicate their network-access requirement. Whereas, this presented draft recommendation aims to create a conformance test specification for verifying compliance with MUD specification. |
| 3 | ITU-T SG-11 | Q.4060: The structure of the testing of heterogeneous Internet of things gateways in a laboratory environment | In Force | Q.4060 Recommendation describes the testing methodology of the heterogeneous network gateway which is to be used for communication among IoT devices. It covers tests such as stress load, types and sizes of frames, operation settings etc for the gateway. It does not cover conformance testing or any requirements on IoT devices to communicate their network-access requirement. Whereas, this presented draft recommendation aims to create a conformance test specification for verifying compliance with MUD specification. |
| 4 | ITU-T SG-11 | Q.4069: Testing requirements and procedures for Internet of things based green data centres | In force | Recommendation ITU-T Q.4069 specifies testing requirements and procedures for green data centres (GDCs) based on the Internet of things (IoT). The testing requirements include those for: interoperability between platform, systems and IoT devices; functions (e.g., analysis of IoT device status); and self-optimization (e.g., data quality audit). The testing procedures include those for: interoperability; functions; and self-optimization. It does not cover conformance testing or any requirements on IoT devices to communicate their network-access requirement. Whereas, this presented draft recommendation aims to create a conformance test specification for verifying compliance with MUD specification. |
| 5 | ITU-T SG-11 | Q.4071: The testing of 3D ultra-high density Internet of things networks | In force | Recommendation ITU-T Q.4071 describes testing methods/procedures for 3-dimensional (3D) ultra-high density Internet of things (IoT) networks. New models and test methods for three-dimensional ultra-high density IoT networks will be developed and the structure of a model network for testing are presented in the proposed Recommendation. It does not cover conformance testing or any requirements on IoT devices to communicate their network-access requirement. Whereas, this presented draft recommendation aims to create a conformance test specification for verifying compliance with MUD specification. |

Appendix II

(This part does not form integral part of this recommendation)

Use Cases

MUD’s primary use-case lies in strengthen security for IoT devices on home and small-business networks. However, it could also be used for securing large IoT networks where hundreds or thousands of devices are deployed across large regions with technologies such as NB-IoT or EnOcean. Irrespective of the type of IoT technology, MUD can be leveraged to provide the fundamental security required to prevent network-based attacks on an IoT device or network in an automated manner.

This work item aims to establish standardized testing methodology and requirements for ensuring an IoT device’s compliance with MUD specification as prescribed in RFC 8520. The work item will eventually be developed into a MUD Conformance Test Specification and can be used by the product vendors, large organizations, small businesses, individuals for their home networks, academia and government regulators to verify if an IoT device(s) is in compliance with MUD requirements specified in RFC 8520.

The conformance test specification can then be used to develop an automated test suite or a test tool that can be used before deploying any IoT device in a network, or before deploying an IoT networks as a whole, and also for continuous monitoring of an IoT networks to ensure all devices in that network are conforming to MUD specification.

_____________________