|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  |  | SG11-TD263/GEN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 11 |  |  |
|  |  |  |  | Original: English |  |  |
| Question(s): |  |  | 15/11 |  | Geneva, 19-28 February 2025 |  |
| TD |  |  |  |  |  |  |
| Source: |  |  | Editors |  |  |  |
| Title: |  |  | Output – draft Technical Report ITU-T TR-CF-QoS “Impact of Counterfeit Mobile devices on Quality of Service” (Geneva, 19-28 February 2025) |  |  |  |
| Contact: |  |  | Rafael Araujo Brazil |  |  | Tel: +55-61-2312-2000 E-mail: rafael.araujo@anatel.gov.br |
| Contact: |  |  | Kofi Ntim Yeboah-Kordieh National Communications Authority Ghana |  |  | Tel: +233244384288 E-mail: kofi.yeboah-kordieh@nca.org.gh |
| Contact: |  |  | Biren Karmakar C-DOT, India |  |  | Tel: +91-11-26598474 Email: biren@cdot.in |
| Contact: |  |  | Rajendra Kumar C-DOT, Ministry of Communication India |  |  | Tel: +91-11-26598466 Email: rajk@cdot.in |
| Abstract: |  | This document contains the updated text of the draft Technical Report ITU-T TR-CF-QoS “Impact of Counterfeit Mobile devices on Quality of Service” based on the discussion result during the Q15/11 sessions at the SG11 meeting held in Geneva, 19-28 February 2025. |  |  |  |  |

Introduction:

This document contains the updated text of draft Technical Report ITU-T TR-CF-QoS : “Impact of Counterfeit Mobile devices on Quality of Service.”, based on the discussion result during the Q15/11 sessions at the SG11 meeting held  in Geneva, 19-28 February 2025.

It reflects meeting agreements including the received contributions, C70 (India).

The following table shows input documents and attached is the revised text.

| Doc. Number. | Source | Titles |  |
| --- | --- | --- | --- |
|  |  | Proposals | Discussion and results |
| SG11-C70 | India | The proposed changes in section 6.7 (Network Integrity Risks) |  |

Draft new Technical Report ITU-T TR-CF-QoS

Impact of Counterfeit Mobile devices on Quality of Service

Summary

According to ITU-T Rec E. 800 Quality of Service is defined as the totality of characteristics of a telecommunications service that bear on its ability to satisfy stated and implied needs of the user of the service.

World Trade Organization - WTO also defines counterfeit as the unauthorized representation of a registered trademark carried on goods identical or similar to goods for which the trademark is registered, with a view to deceiving the purchaser into believing that he/she is buying the original goods.

Counterfeit mobile devices pose QoS delivery issues. Studies conducted have shown that such mobile devices suffered from low call completion rates, high call drop rates and handover failures. The counterfeit mobile devices can impact the network performance due to their low call completion rates and frequent call drops which in turn can lead to network congestion and reduced speed for the legitimate users. A survey conducted in the Africa region revealed that all categories of mobile devices were perceived to be the most counterfeited ICT devices in the region.

This technical report will outline the challenges and impact of counterfeit mobile devices on Quality of Service for the users and networks.

Keywords

Counterfeiting, Impact, Quality of Experience, Quality of Service.

CONTENTS

Page

1 Scope 4 2 References 4 3 Terms and definitions 4 3.1 Terms defined elsewhere 4 3.2 Terms defined here 4 4 Abbreviations and Acronyms 5 5 Introduction 5 6 Impact of Counterfeit related to QoS/QoE 5 6.1 Call Drop Rate 5 6.2 Call Setup 6 6.3 Hand Over Rate 6 6.4 Throughput 6 6.5 Network Coverage 7 6.6 Security Vulnerability 7 7 Conclusions 7 Bibliography 9

Draft new Technical Report ITU-T TR-CF-QoS

Impact of Counterfeit Mobile devices on Quality of Service

# Scope

This technical report contains a study on negative effects and impact of counterfeit mobile devices on network’s Quality of Service along with the negative effects and service degradation experienced by the mobile subscribers.

# References

[ITU-T Q.5050]	Recommendation ITU-T Q.5050 (2019), Framework for solution to combat counterfeit ICT Devices.

[ITU-T Q.5051]	Recommendation ITU-T Q.5051 (2020), Framework for combating the use of stolen mobile devices.

[ITU-T Q.5052]	Recommendation ITU-T Q.5052 (2020), Addressing mobile devices with duplicate unique identifier.

[ITU-T E.807]	Definitions, associated measurement methods and guidance targets of user-centric parameters for call handling in cellular mobile voice service

[ITU-T K.114]	Electromagnetic compatibility requirements and measurement methods for digital cellular mobile communication base station equipment

[ITU-T SR-AFR]	Survey report on counterfeit ICT devices in Africa region

# Terms and definitions

## Terms defined elsewhere

This Technical Report uses the following terms defined elsewhere:

3.1.1	drop rate or voice service cut-off ratio: [ITU-T E.807]: The probability of a call terminating without the user's action.

3.1.2	Throughput [ITU-T K.114]: The number of payload bits successfully received per second for a reference measurement channel in a specified reference condition.

3.1.3 Quality of Experience (QoE) [ITU-T P.915]: The degree of satisfaction of the user of an application or service. It results from the fulfilment of his or her expectations with respect to the utility or enjoyment of the application or service in the light of the user's personality and current state.

3.1.4 Quality of Service (QoS) [ITU-T E.800]: Totality of characteristics of a telecommunications service that bear on its ability to satisfy stated and implied needs of the user of the service.

3.1.5 Counterfeit ICT Devices [ITU-T Q.5050]: A counterfeit ICT device is a product that explicitly infringes the trademark, copies hardware or software designs, or infringes brand or packaging rights of an original or authentic product and, in general, infringes applicable national and/or international technical standards, regulatory requirements or conformity processes, manufacturing licensing agreements, or other applicable legal requirements.

## Terms defined here

This Technical Report defines the following terms:

None.

# Abbreviations and Acronyms

This Technical Report uses the following abbreviations and acronyms:

MWF 	Mobile Wireless Forum

QoS		Quality of Service

QoE		Quality of Experience

WTO 	World Trade Organization

# Introduction

The rise of counterfeit products can potentially cause businesses to experience an increase in costs and loss of productivity. Most importantly, counterfeiting can negatively impact a company's reputation and may cost millions in revenue losses. Counterfeiting has a damaging effect on business, the economy and the general population.

World Trade Organization - WTO defines counterfeit as the unauthorized representation of a registered trademark carried on goods identical or similar to goods for which the trademark is registered, with a view to deceiving the purchaser into believing that he/she is buying the original goods. Counterfeit mobile devices may impact on Quality of Service.

The ITU-T SR-AFR “Survey report on counterfeit ICT devices in Africa region” revealed that all categories of mobile devices were perceived to be the most counterfeited ICT devices in the region. Additionally, the same survey pointed that counterfeit ICT devices suffered from low call completion rates, high call drop rates and handover failures.

This technical report will outline the challenges and impact of counterfeit mobile devices on Quality of Service for the users and networks.

# Impact of Counterfeit related to QoS/QoE

Counterfeit devices on the network may affect the following Key Performance Indicators of the Networks:

## Call Drop Rate

A call may drop for different reasons – however for counterfeit devices using retainability tests, it has been found that the failure due to counterfeit devices are higher when compared to those of genuine mobile devices. Explanations for this behaviour are traceable to elements that impact retainability such as:

  - MS transmit power;

  - Phase frequency error;

  - UL power control time response;

  - Delayed handover and;

  - Device sensitivity.

Note that generally, networks are planned with standard device sensitivity, counterfeit devices especially as we approach the coverage edge are disadvantaged. Given also that they are low sensitivity mobile devices, their overall interference become worst due to the increase of average transmission power to maintain calls – as such they are more susceptible to drop the radio link or experience call blocking. This increased interference also impact the overall noise floor and this can bear negative impact on the overall QoS for especially networks where power is the ultimate resource.

## Call Setup

Generally, accessibility tests show that in areas of poor radio frequency conditions, that counterfeit devices showed no service whereas this was not the case with original devices. This reflected that the sensitivity on such counterfeit devices was worse than those of genuine mobile devices. This is one of the reasons why counterfeit devices are responsible for call setup issues. For some of the trace monitored failures, the call setup failures are captured to most times be due to bearer capacity. It is either the device does not have the reported capability, or it does not have the authority to use such capability due to its cloned nature.

Failure code most often associated with this failure is a “Bearer capacity not authorized” which represents a failure which happens when the mobile station requested a bearer capability that it is not authorized to use.

Using trace records from network A shows a typical case of initial context setup failure for a LTE network. In each case the culprit device had an IMEI that was not found in the type allocation code (TAC) database.

The coverage in a mobile network is defined by the maximum allowed path loss (MAPL) in both the uplink and downlink. Counterfeit devices with their poor receiver sensitivity reduce the DL MAPL. This reduction which is also seen in the uplink MAPL as a result of the phone not being able to set the maximum transmitter output power defined in the 3GPP standards requires the operator to deploy additional network investment to cover for the holes due to these mobile devices.

## Hand Over Rate

Handover failure rate with counterfeit devices are higher and this can be traced to the longer execution time it takes for the typical counterfeit phone to complete its handover [b-MWF-Publication]. Flowing from the receiver sensitivity of counterfeit mobile devices, it follows that as their maximum allowable path loss reduces due to their low sensitivity, their ability to retain radio links coupled with their extended handover time, is a recipe for increased handover failure and ultimately drop calls.

Against results from RF functional test which evaluates the quality of the engine machine inside phone terminals which puts counterfeit mobile devices as worse off – it follows that service reduction in terms of handover will be perceived by customers with such devices.

## Throughput

With the advent of smartphones, increased adoption and improvement in network capabilities, the preponderance of traffic on mobile networks has transformed from voice to data. This growth has also exposed a fundamental challenge with the sort of scheduling deployed by networks to ensure fairness among users. In the downlink of networks, the proportional fairness (PF) scheduling mechanism is used to ensure this fairness. Proportional fairness schedulers depend on honest and a priori trust on feedback provided by the mobile station (MS) or User Equipment (UE). These feedback forms the basis for scheduling the mobile device or providing network resources for its utilization. The network performs such scheduling decisions dependent on feedback reports from the channel quality (CQI) and the automatic repeat request (ARQs). Counterfeit / malicious mobile devices explore this trust in a manner that ultimately impacts on the network quality of service. Observations for mobiles in this category, is such that they are either providing:

- erroneous channel quality feedback or

- misreporting of the packet transmission success.

The vulnerability associated with the network scheduler blindly trusting the feedback from the UEs with regards the downlink quality and transmissions is a security backdoor that allows producers of these counterfeit mobile devices to fake CQI or ARQ reports that in many instances come in different forms

- correct CQI - fake ARQ – the most severe case;

- fake CQI and fake ARQ – also very severe impact;

- fake CQI - correct ARQ – mild impact.

The critical impact of these false reports is that, the scheduler trusting on these reports / feedback provides the affected UEs with increased service time / an unfair share of time slots. This starves well behaved UEs of this resource; whereas the counterfeit devices in receipt of either higher transmission block size or very large fraction of time slots than it can actually sustain in the downlink, leads to packet error and waste of scarce resource. Thus the culprit device does not enjoy the expected higher throughput while at the same time, it denies other users of access to same, as such impacting the QoS of the network. These wrong device settings are counterproductive on the network.

## Network Coverage

The range of the network coverage is restricted by minimum received power and maximum transmitted power of mobile device:

- Minimum Received Power: The mobile devices need a strong enough signal from the tower for a reliable connection.

- Maximum Transmission Power: When mobile devices transmit excessive power, there is a risk that they can create interference with other devices. Conversely, if they transmit insufficient power, their own service quality may be compromised.

Note 1: Studies in [b-MWF-Publication] and [b-IEEE-Report] have revealed that received signal power and transmit power of the counterfeit mobile devices have been reduced significantly when compared to genuine mobile devices.

Note 2: This reduction in received and transmitted powers leads to reduction in the maximum achievable distance between the antenna tower and the counterfeit mobile devices to sustain a connection, particularly for voice calls.

## Security Vulnerability

Counterfeiting mobile devices may not have proper security features and software updates, making them more susceptible to viruses, malwares, and other security threats. These devices can significantly impact the networks performance due to various attacks and can disrupt the service availability to the genuine mobile devices.

## Network Integrity Risks:

The use of counterfeit devices can compromise the integrity of the network. They may not adhere to the necessary security protocols, allowing unauthorized access and manipulation of network resources, which can facilitate spoofing and other malicious activities.

# Conclusion

There’s evidence that the use counterfeit devices lower the overall quality of service (QoS) and causes a negative impact on mobile networks. In the report [b-IEEE-Report], it’s shown results of tests, using 3GPP specification [TS51.010-1], of counterfeit mobile devices along with original devices. The results of the tests provided findings that show that counterfeit devices presented a lower sensitivity, lower transmit power, higher absolute frequency error and longer handover time when compared to original devices. Those poor performance characteristics have several impacts on the mobile network as described in section 6.

# Bibliography

[b-MWF-Publication]	 Counterfeit/substandard mobile phones - a resource guide for governments Framework for solutions to combat counterfeit. (https://spotafakephone.com/docs/eng/MMF_CounterfeitPhones_EN.pdf)

[b-IEEE-Report] 		Substandard cell phones: impact on network quality and a new method to identify an unlicensed IMEI in the network (https://ieeexplore.ieee.org/document/6766091)

[b-IEEE-Report2]	Policy Framework and Recommendations to Minimize the Usage of Stolen and Counterfeit or Substandard Mobile Communication Devices

[b-GSMarena]	 https://www.gsmarena.com/glossary.php3?term=network-coverage#:~:text=The%20geographical%20area%20covered%20by,Network%20capacity

[b-TelecomReview]	 https://www.telecomreview.com/articles/reports-and-coverage/4865-telecommunication-security-ddos-attacks

[b-twobirds.com] 	https://www.twobirds.com/insights/2019/uae/counterfeit-mobile-devices-more-than-just-a-security-risk

_______________________