|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  | SG11-TD266/GEN |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 11 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 15/11 |  | Geneva, 19-28 February 2025 |  |
| TD |  |  |  |  |  |
| Source: |  | Editor |  |  |  |
| Title: |  | Output – baseline text of draft new Recommendation ITU-T Q.GIR “Technical requirement and implementation strategy for Global International Mobile Equipment Identity Registry” (Geneva, 19-28 February 2025) |  |  |  |
| Contact: |  | Pushpendra Kumar Singh Ministry of Communications India |  |  | Email: pksingh.its@gmail.com |
| Contact: |  | Biren Karmakar C-DOT, Ministry of Communications India |  |  | Tel: +91-11-26598474 Email: biren@cdot.in |
| Contact: |  | Dr. Arun Kumar Bediya C-DOT, Ministry of Communications India |  |  | Tel: +91-11-26598839 Email: arunb@cdot.in |

| Abstract: | This document contains the draft new Recommendation ITU-T Q.GIR “Technical requirement and implementation strategy for Global International Mobile Equipment Identity Registry” (Geneva, 19-28 February 2025). |
| --- | --- |

Introduction:

This document contains the draft new Recommendation ITU-T Q.GIR “Technical requirement and implementation strategy for Global International Mobile Equipment Identity Registry” based on the base text contained SG11-TD1002/GEN, contributions SG11-C69 and the discussions held at SG11 meeting (Geneva, 19-28 February 2025).

Draft Recommendation ITU-T Q.GIR

Technical requirement and implementation strategy for Global International Mobile Equipment Identity Registry

[Editor´s Note: Further contributions invited for more detailed requirements]

Summary

With the growing usage of information and communications technology (ICT) equipment in people's daily lives in recent years, unwelcome side effects related to the increase in the sale, circulation and use of counterfeit ICT devices in the market has also been increased.

As described in the Recommendation ITU-T Q.5050 (Framework for solutions to combat counterfeit ICT devices), this has resulted in adverse consequences for multiple stakeholders such as users, network operators, genuine device manufacturers, traders and governments, including decreased security protection and quality of service for users and revenue losses to a range of stakeholders.

Section 10.1 of the above recommendation pointed out to “Prohibit the use of invalid and non-genuine device identifiers”.

As the issue of IMEI re-programming still a major concern in combating counterfeiting ICT devices, section 9.3 of ITU-T Q.5050 mentioned about Reliable unique identifier - genuine ICT devices are required to have unique and persistent identifiers that are secure, in the sense that they cannot be changed by unauthorized entities, are unique to each equipment and have been assigned by the authorized assigner. Which implies that in order to reduce IMEI re-programming, it is required to identify that the IMEI is re-programmed.

When IMEI is re-programmed, newly implanted IMEI will be either random i.e. invalid, non-genuine IMEI, duplicate/already used IMEI or IMEI which is valid but not manufactured by the OEM. Invalid or non-genuine IMEI could be detected by MNO EIR by checking the IMEI format and matching with TAC information allocated by GSMA. Even already used IMEI can also be checked through CEIR. But to find out the IMEI which has not been manufactured by OEM/Manufacturers can be done only through the IMEI registry. The IMEI registry will contain the list of IMEIs which have been manufactured by the mobile manufacturers and issued for sale in the market/export. Some of the nations may already have implemented an IMEI registry for their country, but the issue remains the same when mobile devices are imported from other countries. This identifies the requirement of the Global IMEI registry. Participating nations will share IMEI produced in their country – which will be used by all other countries to find counterfeit mobile devices. ITU-T Q.5050 also mentioned about the Certification of the ICT device in section 10.2 to assist all stakeholders and guarantee the authenticity of the products, one possible solution should be to deploy a device lifecycle management system, based on these unique identifiers, that are able to track the ICT devices from the beginning of the manufacture process (including component origin, transportation and retail store where it will be sold) until it is delivered to the end-user - which could easily be implemented through Global IMEI Registry.

ITU-T Q.5051 recommends having the support of applicable national legal and regulatory framework before implementing any restrictive actions against stolen devices with tampered and duplicated unique identifiers.

ITU-T Q.5052 recommendation urges the requirement of Duplicate IMEI detection across countries in section 7.5. However, this is related to the IMEI which is already in use by the subscribers. It does not include the IMEI which has been manufactured but not used in the network.

Resolution 96 resolved that "ITU-T should consider solutions to be used to differentiate between authentic/genuine and counterfeit or tampered telecommunication/ICT devices, e.g. establishing a centralized national reference database of authorized equipment;

Resolution 97 resolved that "ITU-T should, in collaboration with the relevant standards organizations, develop solutions to address the problem of duplication of unique identifiers". Global IMEI Registry will assist to resolve the problems such as IMEI cloning/duplication due to re-programming of IMEI.

History

| Edition | Recommendation | Approval | Study Group | Unique ID* |
| --- | --- | --- | --- | --- |
| 1.0 | ITU-T Q.GIR | --- | 11 | -- |

Keywords

GIR, Global IMEI Registry, Combat counterfeit ICT devices, unique identifiers, replicated devices, unique IMEI

Table of Content

1	Scope	5

2	References	5

3	Definitions	5

3.1	Terms defined elsewhere	5

3.2	Terms defined in this Recommendation	5

4	Abbreviations and acronyms	5

5	Conventions	6

6	General aspects	6

7	Description of the System	7

8. 	Architectural Framework for IMEI Registry	7

8.1 Functional Architecture for NIR	8

8.2 Functional Architecture for RIR	8

8.3 Functional Architecture for GIR	9

Figure 3: Multi-level GIR Architecture	9

9	External Interfaces	9

10	System features	9

11	Stakeholders	10

12	Non-functional Requirements	10

12.1 Performance Requirements	10

12.2 Reliability Requirements	10

12.3 Availability Requirements	10

12.4 Security Requirements	10

13. Security Recommendation	10

Draft Recommendation ITU-T Q.GIR

Technical requirement and implementation strategy for Global International Mobile Equipment Identity Registry

# 1	Scope

This Recommendation proposes the requirement for implementation of Global IMEI Registry and suggests strategy through which it could be implemented.

# 2	References

This ITU-T Recommendation and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

ITU-T Q.5050 - “Framework for solutions to combat counterfeit ICT devices”.

ITU-T Q.5051 - "Framework for Combating the use of Stolen Mobile Devices”.

ITU-T Q.5052 - “Addressing mobile devices with a duplicate unique identifier”.

ITU-T Q.5054- “Consumer centric framework for combating counterfeit and stolen ICT mobile devices”.

ITU-T Q.5055 - “Technical requirement, interfaces and generic functions of CEIR”.

ITU-T QTR-RLB-IMEI - “Reliability of International Mobile station Equipment Identity (IMEI)”.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

counterfeit ICT device [ITU-T Q.5050]: An information and communication technology (ICT) device that explicitly infringes the trademark, copies hardware or software designs, or infringes brand or packaging rights of an original or authentic product and, in general, infringes applicable national and/or international technical standards, regulatory requirements or conformity processes, manufacturing licensing agreements, or other applicable legal requirements.

unique identifier [ITU-T Q.5050]: An identifier associated with a single device that aims to uniquely identify it.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

None.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

API		Application Programming Interface

ABAC		Attribute-Based Access Control

CEIR		Central Equipment Identity Register

EIR		Equipment Identity Register

GIR		Global IMEI Registry

GSMA		Global System for Mobile Communications (GSM) Association

ICT		Information and Communications Technology

IMEI		International Mobile Equipment Identity

ITU		International Telecommunication Union

LEA		Law Enforcement Agency

MFA 		Multi-Factor Authentication

MNO		Mobile Network Operator

MSISDN	Mobile Station Integrated Services Digital Network

NIR		National IMEI Registry

ODM		Original Design Manufacturer

OEM		Original Equipment Manufacturers

PII 		Personally Identifiable Information

RBAC		Role-Based Access Control

REST		Representational State Transfer

RIR		Regional IMEI Registry

TAC		Type Allocation Code

# 5	Conventions

This Recommendation applies the following verbal forms for the expression of provisions:

a)	The keyword "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Recommendation is to be claimed.

b)	The keyword "should" indicate a requirement which is recommended but which is not absolutely required. Thus, this requirement need not be present to claim conformance.

c)	The keyword "may" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor's implementation must provide the option, and the feature can be optionally enabled by the network operator/service provider. Rather, it means the vendor may optionally provide the feature and still claim conformance with this Recommendation.

# 6	General aspects

When an IMEI is reprogrammed, the new IMEI could be arbitrary, incorrect, non-genuine, duplicate, recently used, or even legitimated but not manufactured by the original equipment manufacturer (OEM). Mobile network operator equipment identity register (MNO EIR) and central equipment identity register (CEIR) can detect fake or invalid IMEIs by comparing their format with the TAC issued by GSMA. However, only the IMEI registry can identify IMEIs that were not manufactured by OEMs or manufacturers.

The IMEI registry contains a list of IMEIs that are authorized for retail sale or export and manufactured by mobile device manufacturers. However, the problem still exists when mobile devices are imported from other countries. To solve this problem, a Global IMEI Registry is needed, where participating countries can exchange their IMEIs, which can be used by all countries to identify counterfeited mobile devices. One potential solution could be a device lifecycle management system that uses these unique identifiers to track ICT devices from the beginning of the manufacturing process (including component origin, shipping, and retail outlet) to when they are delivered to the end consumer. The Global IMEI Registry is needed to address this issue and it has the potential to prevent the problems due to IMEI cloning and duplication that arise from re-programming of IMEIs.

# 7	Description of the System

Global IMEI Registry is a system to store and manage all the legitimate IMEIs manufactured by the mobile device manufacturers/OEMs/ODMs. Mobile devices available in GIR may be in-use by subscribers or kept in the stores for sale. It will verify as there are different mobile manufacturers in different countries, manufactured IMEIs to be collected from the local manufacturers of the country to prepare National IMEI Registry (NIR) – which needs to be shared with GIR to maintain IMEI data globally. GIR data could be used by any nation to check/validate the IMEI. As the mobile manufactured in one country could be exported to another country for sale and even mobile sold in one country could be used in another country. GIR data will be very useful for IMEI verification in those cases. GIR data can be utilized by the Government agencies, national telecom authority/ regulator, and law enforcement agencies. GIR data could be useful for the general public that can be accessible through mobile apps, web portals etc with required limited access.

When a manufacturer submits an IMEI number to the GIR for registration, the IMEI undergoes thorough validation procedures to ensure its uniqueness and integrity within the database. These validation procedures involve cross-referencing the submitted IMEI against existing entries in the registry to detect any potential duplicates or inconsistencies.

If the submitted IMEI number is found to be identical to an existing entry in the registry, indicating a duplicate or conflicting record, the validation system will promptly identify it as such. In such cases, the duplicate IMEI will be flagged for rejection by the system to prevent its inclusion in the database. This rejection mechanism serves to maintain the accuracy and reliability of the registry by ensuring that each IMEI entry is unique and corresponds to a distinct mobile device.

# 8. 	Architectural Framework for IMEI Registry

An IMEI registry serves as a crucial tool in the telecommunications industry, playing a vital role in combating unauthorized mobile device usage and deter criminals from engaging in phone-related crimes. Additionally, IMEI registries enhance consumer protection by enabling individuals to verify the authenticity of mobile devices before purchase, thus reducing the risk of unwittingly acquiring stolen or counterfeit phones.

The concept of IMEI registries can be divided into National IMEI Registries (NIRs), Regional IMEI Registry (RIRs) and a Global IMEI Registry (GIR) to highlight their respective roles at the national, regional and international levels.

## 8.1 Functional Architecture for NIR

The NIR plays a vital role in safeguarding mobile phone users against counterfeited devices, reducing related crimes, and promoting a secure telecommunications environment within the nation/country boundary. However, its success depends on robust regulatory frameworks, industry cooperation, and ongoing efforts to adapt to emerging threats and technologies. Figure 1 shows the abstract National IMEI Registry architecture.

Figure 1: NIR Architecture

## 8.2 Functional Architecture for RIR

The establishment of multiple National IMEI Registries (NIRs) within the same geographical region lays the groundwork for the formation of a Regional IMEI Registry (RIR). Connecting NIRs to create a RIR enhances the effectiveness of combating mobile device unauthorized usage, and illegal trafficking on a larger scale. Figure 2 depicts the abstract architecture of RIR.

Figure 2: RIR Architecture

## 8.3 Functional Architecture for GIR

The establishment of multiple RIRs lays the groundwork for the formation of a Global IMEI Registry (GIR). Connecting RIRs and NIRs to create a GIR enhances the effectiveness of combating mobile device unauthorized usage, and illegal trafficking on a global scale. Figure 3 highlights the abstract architecture of GIR.

## Figure 3: Multi-level GIR Architecture

# 9	External Interfaces

GIR requires interface only with NIR/RIR for IMEI data synchronization. It may interface with CEIR system. National level CEIR may interface with NIR, Regional CEIR may interface with RIR and Global level CEIR may interface with GIR.

All other national level stakeholders will be interfacing with NIR for IMEI verification. If required data is not available within the NIR, it will query GIR directly or through RIR to get the status of the requested IMEI preferably through Representational State Transfer (REST)  based web-service, accordingly the result will be shown to the user.

# 10	System features

GIR will collect legitimate IMEI details from the manufacturers and provide details of the legitimate IMEIs including brand name, model, marketing name, manufacturing date. Option will also be there to delete IMEIs of the mobile devices which are scrapped by competent authority. IMEI insertion and deletion statistics will be available to the privileged users. There can be functionality to create different types of users with different privileges to access the system. Some other features such as report generation, activities performed by different users, list of imported IMEI from one nation to another, nation wise statistics, region wise statistics and global statistics etc. can be implemented based on requirement.

An Interface to communicate to NIR, RIR and GIR with CEIR can also be implemented.

# 11	Stakeholders

- OEM/ODM/Brand Owners: Design, manufacture, distribute, market and sell them under their brand name.

- Importers: Procure mobile devices from OEMs/ODMs and bring them into the market for further distribution and sale.

- Customs Department: Enforce import regulations by inspecting shipments of mobile devices entering the country, ensuring compliance with national trade laws and taxes.

- Mobile Network Operators (MNOs): Provide network infrastructure and services for mobile communication, including voice, data, and messaging. They activate and manage subscriber accounts and provide technical support. Validate and keep records of the mobile device used by their subscribers.

- Law Enforcement Agencies: Enforce laws related to mobile device use, such as investigating illegal activities involving mobile devices and ensuring compliance with regulations regarding device registration and use.

- Regulatory Organization: Develop and enforce regulations governing the mobile telecommunications industry, including standards for device certification, spectrum allocation, and consumer protection. They oversee compliance with regulatory requirements and may impose penalties for non-compliance.

# 12	Non-functional Requirements

## 12.1 Performance Requirements

All other performance related to storage, memory, and processing should follow
industry recommended practices to ensure resource requirements are minimised.

## 12.2 Reliability Requirements

In accordance with industry recommended practices, the system should undergo
feature testing, load testing, and regression testing prior to release and/or deployment.

## 12.3 Availability Requirements

Reasonable efforts should be made to ensure the software system is available with an
preferable uptime of 99.99%. The uptime is calculated as the percentage of time during the year in which the software system was available to the users.

## 12.4 Security Requirements

The system defined in this document must follow industry recommended practices for
secure software development. At a minimum, the software development must practice the
principle of least privilege for defining access-level requirements of the software system and its associated services. The production-release version of the software system must pass an automated dynamic application security testing/audit tool.

# 13. Security Recommendation

Implementing a Global IMEI Registry (GIR) involves rigorous adherence to technical security measures to fortify the integrity and confidentiality of the system. Key technical considerations include robust data encryption mechanisms, utilizing industry-standard cryptographic algorithms to encrypt IMEI data both during transmission and storage.

Access control mechanisms are instituted through the implementation of role-based access control (RBAC) and attribute-based access control (ABAC), enforced through secure authentication protocols such as OAuth or OpenID Connect. Multi-factor authentication (MFA) is employed to bolster user authentication, typically requiring a combination of factors such as passwords, biometrics, or token-based authentication.

Secure APIs can be developed utilizing standards like HTTPS with Transport Layer Security (TLS) encryption to ensure secure communication among the GIR, RIRs and NIRs, with stringent authorization checks in place. Audit trails are meticulously maintained, recording all user activities and system events for audit and forensic analysis, monitoring of the registry’s health and performance to detect failures, system abuse, and other irregularities. Data anonymization techniques such as tokenization or pseudonymization can be applied to protect sensitive information  stored within the registry.

_______________________